| 3GPP TS 28.104 V0.3.0 (2021-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Management Data Analytics (MDA) (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Concepts and overview	9

4.1	Overview	9

5	 MDA functionality and service framework	10

5.1	General framework	10

5.2	Interaction with CN and RAN domains	11

5.3	Deployment of multiple MDAs	12

6	MDA in management loop	14

6.1	MDA role in the management loop	14

6.2	MDA role in the management loop for service assurance	15

6.3	MDA role in cross-domain service assurance	15

7	MDA use cases and requirements	17

7.1	General	17

7.2	MDA capabilities	17

7.2.1	Coverage related analytics	17

7.2.1.1	Coverage problem analysis	17

7.2.1.1.1	Description	17

7.2.1.1.2	Use case	17

7.2.1.1.3	Requirements	18

7.2.1.2	Slice coverage analysis (NS-COV_MDA)	18

7.2.1.2.1	Description	18

7.2.1.2.2	Use case	18

7.2.1.2.3	Requirements	18

7.2.1.3	Paging optimization analysis	19

7.2.1.3.1	Description	19

7.2.1.3.2	Use Case	19

7.2.1.3.3	Requirements	19

7.2.2	SLS analysis	20

7.2.2.1	Service experience analysis (UC-Ser_Exp_MDA)	20

7.2.2.1.1	Description	20

7.2.2.1.2	Use case	20

7.2.2.1.3	Requirements	20

7.2.2.2	Network slice throughput analysis (UC-THR_MDA)	20

7.2.2.2.1	Description	20

7.2.2.2.2	Use case	20

7.2.2.2.3	Requirements	21

7.2.2.3	Network slice traffic prediction	21

7.2.2.3.1	Description	21

7.2.2.3.2	Use case	21

7.2.2.3.3	Requirements	21

7.2.2.4	E2E latency analysis (UC- E2E_LAT_MDA)	22

7.2.2.4.1	Description	22

7.2.2.4.2	Use case	22

7.2.2.4.3	Requirements	22

7.2.2.5	Network slice load analysis (UC- NS_LOAD_MDA)	23

7.2.2.5.1	Description	23

7.2.2.5.2	Use cases	23

7.2.2.5.3	Requirements	23

7.2.3	MDA assisted fault management	24

7.2.3.1	Fault prediction	24

7.2.3.1.1	Description	24

7.2.3.1.2	Use case	24

7.2.3.1.3	Requirements	24

7.2.4	MDA assisted Energy Saving (UC-ES_MDA)	24

7.2.4.1	Description	24

7.2.4.2	Use cases	24

7.2.4.3	Requirements	25

7.2.5	MDA assisted mobility management	25

7.2.5.1	Mobility performance analysis (UC-MRO_MDA)	25

7.2.5.1.1	Description	25

7.2.5.1.2	Use case	26

7.2.5.1.3	Requirements	26

7.2.5.2	Handover optimization analysis	26

7.2.5.2.1	 Description	26

7.2.5.2.2	 Use cases	26

7.2.5.2.2.1	Handover optimization	26

7.2.5.2.2.2	Handover optimization based on UE Load	27

7.2.5.2.3	Requirements	27

7.2.5.3	Inter-gNB beam selection optimization	27

7.2.5.3.1	Description	27

7.2.5.3.2	Use case	27

7.2.5.3.3	 Requirements	28

7.2.6	MDA assisted critical maintenance management	28

7.2.6.1	RAN Node Software Upgrade	28

7.2.6.2.1	Description	28

7.2.6.2.2	Use case	28

7.2.6.2.3	Requirements	29

7.3	MDA MnS	29

7.3.1	MDA request and control	29

7.3.1.1	Description	29

7.3.1.2	Use case	29

7.3.1.3	Requirements	30

7.3.2	Obtaining MDA Output	30

7.3.2.1	Description	30

7.3.2.2	Use case	30

7.3.2.3	Requirements	31

7.4	Supporting aspects for MDA	31

7.4.1	Machine Learning (ML) for MDA	31

7.4.1.1	Description	31

7.4.1.1.1	MDA with ML support	31

7.4.1.1.2	ML model training	32

7.4.1.2	Use cases	32

7.4.1.2.1	ML model training requested by consumer	32

7.4.1.2.2	ML model training initiated by producer	33

7.4.1.3	Requirements for ML model training for MDA	33

8	MDA capability data definitions	33

8.1	Introduction	33

8.2	About analytics	33

8.2.1	About enabling data	33

8.2.2	About analytics outputs	33

8.3	Common information elements of analytics outputs	34

8.4	Data definitions per MDA capability	34

8.4.1	Coverage related analytics	34

8.4.1.1	 Coverage problem analysis	34

8.4.1.1.1	MDA type	34

8.4.1.1.2	Enabling data	34

8.4.1.1.3	Analytics output	35

9	Information model definitions for MDA	36

9.1	Imported and associated information entities	36

9.1.1	Imported information entities and local labels	36

9.1.2	Associated information entities and local labels	36

9.2	Class diagram	36

9.2.1	Relationships	36

9.2.2	Inheritance	37

9.3	Class definitions	37

9.3.1	MDARequest	37

9.3.1.1	Definition	37

9.3.1.2	Attributes	37

9.3.1.3	Attribute constraints	37

9.3.1.4	Notifications	38

9.4	Attribute definitions	38

9.4.1	Attribute properties	38

9.4.2	Constraints	38

9.5	Common notifications	38

9.5.1	Alarm notifications	38

9.5.2	Configuration notifications	38

9.5.3	Threshold Crossing notifications	38

10	MDA related service components	39

10.1	MDA MnS Service components	39

10.1.1	General	39

10.1.2	MDA report request and control	39

10.1.2.1	Service components	39

10.1.3	MDA reporting	39

10.1.3.1	Service components	39

11	Workflows for MDA management	39

Annex X (informative): Change history	40

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

The present document specifies the MDA capabilities with corresponding analytics inputs and analytics outputs (reports), as well as processes and requirements for MDAS (Management Data Analytics Service), historical data handling for MDA, and ML support for MDA.

This document also describes the MDA functionality and service framework, and MDA role in the management loop.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.500: "Telecommunication management; Self-Organizing Networks (SON); Concepts and requirements".

[3]	3GPP TS28.535: “Management services for communication service assurance; Requirements”.

[4]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[5]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[6]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[7]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[8]	3GPP TS 28.405: "Telecommunication management, Quality of Experience (QoE) measurement collection; Control and configuration".

[9]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[10]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[11]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[12]	3GPP TS 32.425: "Telecommunication management; Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[13]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[14]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[15]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[16]	3GPP TS 28.658: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[17]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM); Information Service (IS)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

<defined term>: <definition>.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AI				Artificial Intelligence

MDA			Management Data Analytics

MDAS		Management Data Analytics Service

MDA MnS	MDA Management service

ML			Machine Learning

# 4	Concepts and overview

Editor’s note: this clause is to accommodate the concepts and overview of MDA/MDAS.

## 4.1	Overview

Management Data Analytics (MDA), as a key enabler of automation and intelligence, is considered a foundational capability for mobile networks and services management and orchestration.

The MDA provides a capability of processing and analysing data related to network and service events and status including e.g., performance measurements, Trace/MDT/RLF/RCEF reports, QoE reports, alarms, configuration data, network analytics data, and service experience data from AFs, etc. to provide analytics reports, that describe the analysed issues and may also include recommended actions to enable necessary actions for network and service operations. The MDA reports are provided by the MDAS (Management Data analytics Service) producer to the consumer.

The MDA can identify ongoing issues impacting the performance of the network and service, and help to identify in advance potential issues that may cause potential failure and/or performance degradation. The MDA can also assist to predict the network and service demand to enable the timely resource provisioning and deployments which would allow fast time-to-market network and service deployments.

Management Data Analytics Service (MDAS), the services exposed by the MDA, can be consumed by various consumers, for instance MnFs (i.e., MnS service producers/consumers for network and service management), NFs (e.g., NWDAF), SON functions, network and service optimization tools/functions, SLS assurance functions, human operators, and AFs, etc.

NOTE: 	Throughout this specification the terms, MDAS and MDA MnS are equivalent and may be used interchangeably.

# 5		MDA functionality and service framework

## 5.1	General framework

MDA MnS (also referred to as MDAS) in the context of SBMA enables any authorized consumer to request and receive analytics as illustrated in Figure 5.1-1.

Figure 5.1-1: MDA functional overview and service framework

A management function may play the roles of MDA MnS producer, MDA MnS consumer, other MnS consumer, NWDAF consumer and also interact with other non-3GPP management systems.

The internal business logic related to MDA leverages the current and historical data related to:

-	Performance Measurements (PM) as per TS 28.552 [4] and Key Performance Indicators (KPIs) as per TS 28.554 [5].

-	Trace data, including MDT/RLF/RCEF, as per TS 32.422 [6] and TS 32.423 [7].

-	QoE and service experience data as per TS 28.405 [8] and TS 28.406 [9].

-	Analytics data offered by NWDAF as per 23.288 [10] including 5GC data and external web/app-based information (e.g., web crawler that provides online news) from AF.

-	Alarm information and notifications as per TS 28.532 [11].

-	CM information and notifications.

-	MDA reports from other MDA MnS producers.

-	Management data from non-3GPP systems.

Analytics output from the MDA internal business logic are made available as MDA reports by the management functions playing the role of MDA MnS producers to the authorized consumers, (including but not limited to other management functions, network functions/entities, NWDAF, SON functions, optimization tools and human operators).

## 5.2	Interaction with CN and RAN domains

The MDA MnS producer provides analytics data for management purposes based on input data related to different types of NFs or entities in the network, e.g., data reported from gNB and/or specific core network function(s). Depending on the use case and when needed, the MDA MnS producer may use the analytics results produced by NWDAF as input.

MDA Management Function may act as 3GPP domain-specific (e.g., RAN or CN) or as 3GPP cross-domain. Figure 5.2-1 illustrates the example of coordinations between NWDAF, gNB and MDA MnS producer(s) for data analytics purpose.

RAN domain MDA MnS producer

MDA MnS

3GPP cross-domain MDA MnS consumer

3GPP cross-domain MDA MnS producer (domain MDA MnS consumer)

CN domain MDA MnS producer

gNB

MDA MnS

MDA MnS

NWDAF

Other 5GC NF

Nnf

Nnwdaf

MDA MnS

MDA MnS

MnS

MnS

Nwdaf

RAN domain

CN domain

MnS

MDA MnS

Figure 5.2-1	Example of coordination between NWDAF, gNB and MDAS (MDA MnS) producer

Any authorized MnS consumers get access to MDA reports by interacting with MDA MnS producers. These scenarios include but are not limited to the following:

- 	The NWDAF, leveraging MDA reports (e.g., for control purposes and other 5GC NFs), interacts with MDA MnS producers.

-	The gNB may consume the MDA MnS for RAN control purpose.

-	The 3GPP cross domain MDA MnS Producer may consume (acting as Domain MDA MnS consumer) MDA MnS provided by domain-specific (RAN and/or CN) MDA MnS producer(s), and produce MDA MnS that may be consumed by 3GPP cross-domain MDA MnS consumer(s).

The management function playing the role of domain MDA MnS producer may interact with 5GC and RAN MnSs and NFs to receive analytics inputs per each MDA use case, including:

-	The CN Domain MDA MnS producer may consume the service provided by NWDAF and other 5GC NFs for MDA purpose.

-	The RAN Domain MDA MnS producer may consume the MnS service provided by/for gNB for MDA purpose.

The management function playing the role of 3GPP cross domain MDA MnS producer interacts with 5GC, RAN domain MDA, 5GC and RAN MnS to receive analytics inputs per each MDA use case including:

-	The cross domain MDA MnS producer may consume the domain MDA MnS.

-	The cross domain MDA MnS producer may consume MnS provided by RAN and/or CN domains, and produce MDA MnS that may be consumed by 3GPP cross-domain MDA MnS consumer(s).

## 5.3	Deployment of multiple MDAs

Multiple MDA instances may be deployed according to deployment need.

The 3GPP cross domain management may consume MDA MnS provided by core network management as shown in Figure 5.3-1.

MDA MnS

MDA MnS

Core Domain

Core Network

Other 5GC NF

NWDAF

Nnf

Nnwdaf

MnS

Nnwdaf

Cross-domain management

Cross-domain MDA

3GPP cross-domain MDA MnS consumer

MDA MnS

Core network management

CN domain MDA

Figure 5.3-1: 	Example of coordination cross domain MDA and CN domain MDA

The management function playing the role of 3GPP cross domain MDA MnS producer interacts with CN domain MDA per each MDA use case as follows:

-	The cross domain MDA MnS producer may consume the CN domain MDA MnS.

-	The cross domain MDA MnS producer may consume MnS provided by CN domains, and produce MDA MnS that may be consumed by 3GPP cross-domain MDA MnS consumer(s).

The management function playing the role of CN domain MDA MnS producer interacts with MnS producers per each use case as follows:

- The CN domain MDA MnS producer may consume analytics results produced by NWDAF, MnS provided by CN domain management, other MDA MnS producers, management data derived by subnetwork management function, and management data derived by element management function.

The 3GPP cross domain management may consume MDA MnS provided by RAN management as shown in Figure 5.3-2.

MDA MnS

MDA MnS

MnS

RAN domain

Radio access network

gNB

Cross-domain management

MDA MnS

3GPP cross-domain MDA MnS consumer

Cross-domain MDA

RAN network management

RAN domain MDA

…

…

Radio access network

gNB

Radio access network

gNB

Figure 5.3-2: 	Example of coordination cross domain MDA and RAN domain MDA

The management function playing the role of 3GPP cross domain MDA MnS producer interacts with RAN domain MDA per each MDA use case as follows:

-	The cross domain MDA MnS producer may consume the RAN domain MDA MnS.

-	The cross domain MDA MnS producer may consume MnS provided by RAN domains, and produce MDA MnS that may be consumed by 3GPP cross-domain MDA MnS consumer(s).

The management function playing the role of RAN domain MDA MnS producer interacts with MnS producers per each use case as follows:

-	The RAN domain MDA MnS producer may consume MnS provided by RAN domain management, other MDA MnS producers, management data derived by subnetwork management function, and management data derived by element management function.

# 6	MDA in management loop

Editor’s note: this clause is to describe 1) MDA role (related to Analytics stage) in the management loop (including open control loop and closed control loop supporting Closed Loop SLS Assurance (COSLA) and eCOSLA), and 2) how the MDA output is aligned to Analytics Insights defined in COSLA and eCOSLA work.

## 6.1	MDA role in the management loop

Intelligence in Analytics, played by MDA, in the management loop which can be open loop (operator controlled) or closed loop (autonomous) [1] as shown in Figure 6.1-1, generates value by processing and analysis of management and network data, where AI and ML techniques may be utilized.

Figure 6.1-1: Analytics in management loop

The management loop constitutes number of elements including the analytics, and these are briefly described below:

Observation: The observation of the managed networks and services. It involves monitoring and collection of events, status and performance of the managed networks and services, and providing the observed/collected data.

Analytics: The data analytics for the managed networks and services. MDA plays the role of Analytics in the management loop. It prepares, processes and analyses the observed/collected data or time series of the observed/collected data related to the managed networks and services. MDA reports may contain root cause analysis of ongoing issues, predictions of potential issues and corresponding relevant causes and recommended actions for preventions, and/or prediction of network and/or service demands.

Decision: The decision making for the management actions for the managed networks and services. The management actions are decided based on the analytics reports (provided by MDA) and other management data (e.g., historical decisions made previously) if necessary. The decision may be made by the consumer of MDAS (in the closed management control loop), or by a human operator (in the case of open management loop). The decision may include e.g. what actions to take, and when to take the actions.

Execution: The execution of the management actions according to the decisions. During the execution step, the actions are carried out to the managed networks and services, and the reports (e.g., notifications, logs) of the executed actions are provided.

## 6.2	MDA role in the management loop for service assurance

MDA represents Analytics roles in the management control loop for communications service assurance [3]. The management and control of resources used by a communication service and the assurance of this communication service  level agreements (e.g., per SLS) is provided by the management control loop involving different management services produced by the management system, which includes management data analytics service (MDAS, or MDA MnS).  The MDAS (MDA MnS) may be produced based on a combination of information including e.g., the user quality of service experience, network performance and network resource utilisation analysis and the SLS.

The MDAS complements other services in the management loop in order to perform SLS communication service assurance. Prior to operation phase, the MDA role in the management control loop is to prepare, process and analyse the data related to the managed communication service, in order to provide the analytics output (analytics report) which may include prediction and feasibility checks of network resource requirements to meet the SLS.

During the operation phase, the MDA can identify ongoing issues impacting the performance of the communication service per the SLS, and identify in advance potential risks that would cause potential failure and/or performance degradation. The MDA can also predict the network and service demand to maintain delivery of communication service per the contracted SLS.

## 6.3	MDA role in cross-domain service assurance

Cross-domain MDA may base its analysis on the outputs from one or multiple single-domain MDA. Figure 6.3-1 shows the simplest case, where a cross-domain analysis service incorporates the results of single-domain analysis service(s).

Cross domain analysis service

Analytics

Single domain analysis service

Single domain analysis service

Single domain analysis service

Analytics

Domain-level analysis

Figure 6.3-1 Cross-domain analysis based on single-domain analysis

Figure 6.3-2 shows the case where a cross-domain analysis service incorporates the results of single-domain analysis service(s) which are embedded within single-domain control service(s).

Cross domain analysis service

Analytics

Single domain control service

Single domain control service

Single domain control service

Analytics

Execution

Observation

Decision

Domain-level analysis

Figure 6.3-2 Cross-domain analysis based on single-domain control loop

Figure 6.3-3 shows the case where a cross-domain analysis service is part of a cross-domain control service. Also in this case, cross-domain analysis service incorporates the results of single-domain analysis service(s).

Cross domain control service

Analytics

Execution

Observation

Decision

Single domain analysis service

Single domain analysis service

Single domain analysis service

Analytics

Domain-level analysis

Figure 6.3-3 Cross-domain control loop based on single-domain analysis

Figure 6.3-4 shows another case where a cross-domain analysis service is part of a cross-domain control service. In this case, cross-domain analysis service incorporates the results of single-domain analysis service(s) which are embedded within single-domain control service(s).

Cross domain control service

Analytics

Execution

Observation

Decision

Single domain control service

Single domain control service

Single domain control service

Analytics

Execution

Observation

Decision

Domain-level analysis

Figure 6.3-4 Cross-domain control loop based on single-domain control loop

# 7	MDA use cases and requirements

## 7.1	General

## 7.2	MDA capabilities

Editor’s note: this clause is about what types of analytics the MDA is capable to do and provide the corresponding analytics outputs.

### 7.2.1	Coverage related analytics

#### 7.2.1.1	Coverage problem analysis

##### 7.2.1.1.1	Description

This MDA capability is about analytics of coverage related problem.

##### 7.2.1.1.2	Use case

The RAN coverage problem may cause UEs out of service or result in downgrade of network performance offered to the UEs, such as failure of random access, paging, RRC connection establishment or handover, low data throughput, abnormal releases of RRC connection or UE context, and dissatisfied QoE.

There are various types of coverage problems, e.g., weak coverage, a coverage hole, a pilot pollution, an overshoot coverage, or a DL and UL channel coverage mismatch, etc, caused by different sorts of reason, such as insufficient weak transmission power, blocked by constructions and/or restricted by terrain.

The 5G related coverage problem may exist in NR, in E-UTRA or both.

To unravel a coverage problem, it is necessary for MDAS consumer to determine the details about when and where the problem occurred, and the type and cause(s) of the problem. Therefore, it is desirable for MDA to correlate and analyze multifold data (such as performance measurements, MDT reports, RLF reports, RCEF reports, UE location reports, together with the geographical, terrain and configuration data of the RAN) to detect and describe the problem with detailed information.

To help MDAS consumer to solve the coverage problem as quickly as possible, MDA may also provide the recommended remedy actions (e.g., reconfigure or add some cells, beams, antennas, etc.) along with the description of the problem.

##### 7.2.1.1.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-COV_MDA-01 | 3GPP management system shall have the capability to provide the coverage problem analysis related to e.g., weak coverage, coverage hole, pilot pollution, overshoot coverage, or DL and UL channel coverage mismatch. | Coverage problem analysis |
| REQ-COV_MDA-02 | 3GPP management system shall have the capability to provide area specific coverage problem analysis. | Coverage problem analysis |

#### 7.2.1.2	Slice coverage analysis (NS-COV_MDA)

##### 7.2.1.2.1	Description

The 3GPP management system shall have the capability to provide the slice coverage analysis

##### 7.2.1.2.2	Use case

The slice coverage is one of the indicators when a 3rd party (i.e., slice tenant) issues a slice request and is mapped into the desired geographical coverage area with the available radio coverage which depends on the base station planning and deployment. In order to map the desired slice coverage perfectly, MDA can be used to optimize the slice coverage on the slice instantiation and runtime considering (i) slice-aware statistics, e.g., slice-UE distributions and mobility patterns, (ii) slice SLA and (iii) access node capabilities.

In 5G the notion of coverage is represented by a set of one or more Tracking Areas (TAs), which are contained in a Registration Area (RA), which is assigned to a UE once it registers to the network. Depending on the MDA MnS producer output, TA and RA planning, i.e., grouping cells to form a TA and then TAs to an RA, can be optimized and the RAN parameters can be adjusted to shape the cell edges and load distribution. The main objective is to fulfill a given slice SLA involving as few cells as possible by leveraging the benefits of adjusting cell configurations for satisfying the desired coverage.

##### 7.2.1.2.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-NS_COV_MDA-01 | 3GPP management system shall be able to provide the analytics output describing the slice coverage, slice availability and slice prediction information. | (NS-COV_MDA) Slice coverage analysis |
| REQ-NS_COV_MDA-02 | 3GPP management system shall be able to provide the mapping between slice coverage and actual radio deployment. | (NS-COV_MDA) Slice coverage analysis |
| REQ-NS_COV_MDA-03 | 3GPP management system shall be able to provide recommended actions that involve options to reconfigure TA and/or RAN attributes including HO parameters, cell reselection parameters, beam configuration, computing resource and slice support in a cell. | (NS-COV_MDA) Slice coverage analysis |

#### 7.2.1.3	Paging optimization analysis

##### 7.2.1.3.1	Description

This capability deal with enabling various functionalities related with paging optimization.

##### 7.2.1.3.2	Use Case

As per the current procedures, if the UE goes out-of-coverage (OOC) the paging which was initiated by the network Access and Mobility Management Function (AMF) fails. The re-attempts continue to fail until UE comes in the coverage and reacts to the paging attempts. This repetitive paging attempts result in the wastage of network resources. As an example, the use case includes a user or a group of users getting into an area, with no cellular coverage on a regular basis for a considerably long duration, for e.g., the user gets into a shielded room for some testing purpose every day for a defined period. The Network initiated paging for such users will fail until they are back in the area with cellular coverage. This would result in in-efficient network resource usage.

It is desirable to use MDAS (Management data analytic service) to optimize the current paging procedures in 5G networks. MDAS producer provides an analytics output containing the user(s) paging analytics indicating the time window at which the user is OOC on a regular basis at the particular location and hence will not be able to respond on a network-initiated paging. Based on the output MDAS consumer (e.g., AMF, gNB) decides on whether, when and where to initiate or not to initiate the paging procedures, thereby ensuring the efficient paging procedures and optimal network resource utilization, as paging can be initiated only when there are more chances for it to be successful.

##### 7.2.1.3.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-PAG_MDA-01 | The MDAS producer should have a capability allowing the authorized consumer to get the paging analytics output describing paging result patterns for a particular user or a group of users. | Paging optimization analysis |
| REQ-PAG_MDA-02 | The MDAS producer should have a capability to provide the paging analytics output describing the paging result patterns based on successful and un-successful paging attempts at a particular time and duration. | Paging optimization analysis |
| REQ-PAG_MDA-03 | The paging analytics output describing the paging result patters should contain the following information: - User Identification: Identification of the user or a group of users. - Daily-OOC-Duration: Predicting the time window during which UE is out-of-coverage every day. - Daily-OOC-Location: Predicting the last known location before UE going out-of-coverage every day. - Recommended Action: The recommendation may suggest stopping paging the UE for Daily-OOC-Duration at Daily-OOC-Location. | Paging optimization analysis |

### 7.2.2	SLS analysis

#### 7.2.2.1	Service experience analysis (UC-Ser_Exp_MDA)

##### 7.2.2.1.1	Description

The 3GPP management system shall have the capability to provide the service experience analysis.

##### 7.2.2.1.2	Use case

Service experience of end user is key indicator directly reflects the user satisfaction degree. In 5G system, the diversity of network service are explored and the requirements of different service especially form vertical users are standardized. Considering these diverse requirements (e.g., priorities of SLA related attributes such as latency, throughput, maximum user number or different required values of these attributes), the service experience as a comprehensive indicator is analysed.

##### 7.2.2.1.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-SER_EXP_MDA -01 | 3GPP management system should have the capability to identify the type of the service experience issue, e.g., RAN issue, CN issue, TN issue, UE issue, service provider issue. | (UC-Ser_Exp_MDA) Service experience analysis |
| REQ-SER_EXP_MDA -02 | 3GPP management system should have the capability to provide the analytics output with following information describing the current service experience aspects and potentially future prediction: - The predicted service experience or observed service experience statistics, may split into subcounters in different levels, e.g., per S-NSSAI, per 5QI, per UE, etc. - Service experience root cause analysis. | (UC-Ser_Exp_MDA) Service experience analysis |
| REQ-SER_EXP_MDA -03 | 3GPP management system should have the capability to provide the level of service experience | (UC-Ser_Exp_MDA) Service experience analysis |
| REQ-SER_EXP_MDA-04 | 3GPP management system should have the capability to provide the recommendation for improving service experience. | (UC-Ser_Exp_MDA) Service experience analysis |

#### 7.2.2.2	Network slice throughput analysis (UC-THR_MDA)

##### 7.2.2.2.1	Description

The 3GPP management system shall have the capability to provide the network slice throughput analysis.

##### 7.2.2.2.2	Use case

Throughput is of great importance which represents the end users' experiences and also reflects the network problems, e.g., low UE throughput may be caused by the resource shortage. In order to satisfy the requirements of dL/ulThptPerSlice in the ServiceProfile, MDAS may be utilized for throughput related analysis/predictions for network slice instance.

MDAS producer should have the capability to receive the request from the consumer to analyse the network slice throughput related issues and identify the root cause to assist throughput assurance. Network slice throughput analysis can be for a specific domain or for cross-domain. The two levels of MDAS producers worked in a coordinated way to assure the throughput performance. The producer of MDAS is able to provide the MDA report including the network slice throughput analytics output.

##### 7.2.2.2.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-THR_MDA-1 | 3GPP management system should have the capability to identify the network slice throughput issue, e.g., RAN issue, CN issue | (UC-THR_MDA) Network slice throughput analysis |
| REQ-THR_MDA -2 | 3GPP management system should have the capability to provide the root cause analysis of the network slice throughput issue | (UC-THR_MDA) Network slice throughput analysis |
| REQ-THR_MDA -3 | 3GPP management system should have the capability to provide the analytics output of the network slice throughput should which contain the following information: - Network slice throughput statistics; - Network slice throughput predictions. | (UC-THR_MDA) Network slice throughput analysis |
| REQ-THR_MDA-04 | 3GPP management system shall have the capability to provide the prompt when the network slice throughput exceeds or falls below a certain threshold. | (UC-THR_MDA) Network slice throughput analysis |

#### 7.2.2.3	Network slice traffic prediction

##### 7.2.2.3.1	Description

This MDA capability is to predict network slice traffic patterns.

##### 7.2.2.3.2	Use case

It is desirable to use MDAS to get the network slice traffic predictions including individual traffic predictions on each of the constituent network functions instances present in the network slice. The individual traffic predictions can be used for better resource management of the network slice. For example, resources can be pre-configured considering the predicted traffic on the network slice.

##### 7.2.2.3.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-TRA_MDA--01 | 3GPP management system should have a capability allowing the authorized consumer to request the network slice traffic analytics output describing traffic prediction of the network slice including its constituent network functions. | Network slice traffic prediction |
| REQ-TRA_MDA-02 | 3GPP management system should have a capability to provide the network slice traffic analytics output describing the traffic predictions for each constituent network function instance in the network slice. | Network slice traffic prediction |
| REQ-TRA_MDA-03 | The network slice traffic analytics output providing traffic prediction for the network slice may include the following information: - Predicted uplink and downlink throughput on each User Plane Function instance (UPF) present in the network slice. - Predicted number of Packet Data Unit (PDU) session for each Session Management Function (SMF) instance present in the network slice. - Predicted number of UE or Registered subscriptions for each AMF instance present in the network slice. - Predicted maximum packet size for each UPF instance present in the network slice. - Predicted UE uplink and downlink throughput on each gNodeB (gNB) instance present in the network slice. - Predicted number of UE for each gNB/NR cell instance present in the network slice. | Network slice traffic prediction |

#### 7.2.2.4	E2E latency analysis (UC- E2E_LAT_MDA)

##### 7.2.2.4.1	Description

The 3GPP management system shall have the capability to provide E2E latency related issue analysis.

##### 7.2.2.4.2	Use case

E2E latency is an important parameter for URLLC services. User data packets should be successfully delivered within certain time constraints to satisfy the end users requirements. Latency could be impacted by the network capability and network configurations. These factors may be the root cause if the latency requirements cannot be achieved. Packet transmission latency may dynamically change if these factors change. The latency requirement should be assured even if some of the network conditions may degrade. It is important for the MDAS producer to analyze the latency related issues to support SLS assurance.

##### 7.2.2.4.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-LAT_MDA-01 | 3GPP management system should have the capability to identify the type of the E2E latency issue, e.g., RAN latency issue, CN latency issue, TN latency issue, UE latency issue，service provider latency issue. | (UC- E2E_LAT_MDA) E2E latency analytics |
| REQ-LAT_MDA-02 | 3GPP management system should have the capability to provide the root cause analysis of the E2E latency issue. | (UC- E2E_LAT_MDA) E2E latency analytics |
| REQ-LAT_MDA-03 | 3GPP management system should have the capability to provide the recommended actions to solve the E2E latency issue. | (UC- E2E_LAT_MDA) E2E latency analytics |

#### 7.2.2.5	Network slice load analysis (UC- NS_LOAD_MDA)

##### 7.2.2.5.1	Description

The 3GPP management system shall have the capability to provide network slice load analysis.

##### 7.2.2.5.2	Use cases

Network slice load may vary during different time periods. Therefore, network resources allocated initially could not always satisfy the traffic requirements, for example, the network slice may be overloaded or underutilized. Overload of signalling in control plane and/or user data congestion in user plane will lead to underperforming network. Besides, allocating excessive resources for network slice with light load will decrease resource efficiency.

The analysis of network slice load should consider the load of services with different characteristics (e.g., QoS information, service priority), load distribution to derive the corresponding resource requirements. Load distribution analytic result may be provided, e.g., load distribution for network slices, different locations and/or time periods etc.

Traffics and resources related performance measurements and UE measurements can be utilized by MDAS producer to identify degradation of the performance measurements and KPI documented in an SLS due to load issues, e.g., radio resource utilization. MDAS producer may further provide recommendations to the network slice load issue. This analytics results can be considered as an input to support SLA assurance to perform further evaluation.

##### 7.2.2.5.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-NS_LOAD_MDA-01 | 3GPP management system should have the capability to identify the domain of the network slice load issue, e.g., RAN issue, CN issue. | (UC- NS_LOAD_MDA) network slice load analytics |
| REQ-NS_LOAD_MDA-02 | 3GPP management system should have the capability to identify the phase of the network slice load issue, e.g., historic/ongoing/potential network slice load issue | (UC- NS_LOAD_MDA) network slice load analytics |
| REQ-NS_LOAD_MDA-03 | 3GPP management system should have the capability to identify the state of the network slice load issue, e.g., overload/underutilized network slice load issue. | (UC- NS_LOAD_MDA) network slice load analytics |
| REQ-NS_LOAD_MDA-04 | 3GPP management system should have the capability to identify the list of the network entities which are involved in the network slice load issue. | (UC- NS_LOAD_MDA) network slice load analytics |
| REQ-NS_LOAD_MDA-05 | 3GPP management system should have the capability to provide analytics related to network slice load within specified time schedules and geographic locations or target objects. | (UC- NS_LOAD_MDA) network slice load analytics |
| REQ-NS_LOAD_MDA-06 | 3GPP management system should have the capability to provide the root cause and recommended actions to the network slice load issue. | (UC- NS_LOAD_MDA) network slice load analytics |

### 7.2.3	MDA assisted fault management

#### 7.2.3.1	Fault prediction

##### 7.2.3.1.1	Description

This MDA capability is to provide fault prediction analysis.

##### 7.2.3.1.2	Use case

There are multiple types of faults in the 5G system and it needs extensive troubleshooting. In order to reduce network and service failure time and performance degradation by faults, it is necessary to supervise the status of various network functions and resources, and predict the running trend of network and potential faults to intervene in advance.

Due to the fact that fault prediction could depend on the existing alarm incidents and relevant historical and real-time data (performance measurement information, configuration data, network topology information, etc.), there is a possibility for MDA to be used in conjunction with AI/ML technologies for model training and potential faults prediction.

In order to avoid the occurrence of faults and abnormal network states, it is necessary for users to obtain the required details of potential fault and the corresponding degradation trend (abnormal KPI, performance measurement information, possible alarm type, fault root cause, etc,). Therefore, MDA, may in conjunction with AI/ML technology, be required to obtain basic health maintenance knowledge (e.g., the relationship between the faults or potential faults and the related maintenance actions) through predefined expertise or model training, so as to effectively predict potential faults. The basic health maintenance knowledge could be updated with feedback.

If necessary, MDA could provide corresponding recommended actions for fault prevention.

##### 7.2.3.1.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-FAULT_PRED_MDA-01 | 3GPP management system shall be able to collect, correlate, filter and analyse the required data (e.g., alarm information, historical and real-time data, etc.) as inputs for analytics and provide the analytics output. | Fault prediction |
| REQ-FAULT_PRED_MDA-02 | 3GPP management system shall be able to obtain basic health maintenance knowledges (e.g., the relationship between the faults or potential faults and the related maintenance actions) through predefined expertise or model training. | Fault prediction |
| REQ-FAULT_PRED_MDA-03 | 3GPP management system shall be able to provide the analytics output including predictions of potential faults, as well as the possible recommendation options. | Fault Prediction |

### 7.2.4	MDA assisted Energy Saving (UC-ES_MDA)

#### 7.2.4.1	Description

This MDA capability is to provide the energy saving analysis.

#### 7.2.4.2	Use cases

Operators are aiming at decreasing power consumption in 5G networks to lower their operational expense with energy saving management solutions. Energy saving is achieved by activating the energy saving mode of the NR capacity booster cell or 5GC NFs (e.g., UPF etc). The energy saving activation decision making is typically based on the load information of the related cells/UPFs, the energy saving policies set by operators and the energy saving recommendations provided by MDAS producer. Under the energy saving state, the required network performance and network experience should also be guaranteed. Therefore, it is important to formulate appropriate energy saving policies (start time, dynamic threshold setting, base station parameter configuration, etc.).

To achieve an optimized balance between the energy consumed and the network performance, MDA can be used to assist the MDAS consumer to make energy saving decisions. To make the energy saving decision, it is necessary for MDAS consumer to determine where the energy efficiency issues (e.g., high energy consumption, low energy efficiency) exist, and the cause of the energy efficiency issues. Therefore, it is desirable for MDA to correlate and analyze the energy saving related performance measurements (e.g., PDCP data volume of cells, power consumption, etc.) and the network analysis data (e.g., observed service experience related network data analytics) to provide the analytics results of current network energy efficiency.

To make the energy saving decision, it is necessary for MDAS consumer to determine which EE KPI related factor(s) (e.g., traffic load, end-to-end latency, active UE numbers, etc) are affected or potentially affected. The MDAS producer can utilize historical data to predict the efficiency KPI related factors (e.g., load variation of cells at some future time, etc). The prediction result of these information can then be used by operators to make energy-saving decision to guarantee the service experience.

The MDAS producer may also provide energy saving related recommendation to the MDAS consumer, The MDAS consumer may take the recommendations into account for making energy saving decisions. After the recommendations have been executed, the MDA producer may start evaluating and further analyzing network management data to optimize the recommendations.

Editor’s Note: The energy saving related recommendation and the current energy saving state of cell are both necessary for consumer. The current energy saving state of cell can be discussed in the future.

#### 7.2.4.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ES_MDA-01 | 3GPP management system shall have the capability to identify the energy efficiency issue (including high energy consumption, low energy efficiency), and identify the cell/NFs or location area of where the indicated energy efficiency issue exists. | (UC-ES_MDA) MDA assisted Energy Saving |
| REQ-ES_MDA-02 | 3GPP management system shall have the capability to identify the root cause of the energy efficiency issue when necessary. | (UC-ES_MDA) MDA assisted Energy Saving |
| REQ-ES_MDA-03 | 3GPP management system shall have the capability to utilize the network status analysis and predictions information of the energy efficiency KPI factors (e.g., traffic load trends) to assist achieving energy saving. | (UC-ES_MDA) MDA assisted Energy Saving |
| REQ-ES_MDA-04 | 3GPP management system shall be able to provide the energy saving recommendation, such as policies and simple configuration actions to guarantee the network performance and network experience. | (UC-ES_MDA) MDA assisted Energy Saving |

### 7.2.5	MDA assisted mobility management

#### 7.2.5.1	Mobility performance analysis (UC-MRO_MDA)

##### 7.2.5.1.1	Description

3GPP management system shall be able to provide the mobility performance analysis.

##### 7.2.5.1.2	Use case

The mobility performance related problems may result from too-early/too-late/ping-pong handovers due to inappropriate handover parameters. MDAS can be used to analyse service experience and network performance during handover period in different mobility scenarios. It may also be able to provide the recommendations of optimal handover parameters to MDAS consumer.

In different NSA and SA deployment architecture scenarios, handover mechanisms (e.g., DAPS, CHO or RACH-less handover) will have different impacts on the mobility performance. The analytics report to identify the most optimal handover mechanism may be provided by MDAS producer.

##### 7.2.5.1.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-MRO_MDA-01 | 3GPP management system shall be able to provide the mobility performance in NSA and SA deployment architectures. | (UC-MRO_MDA) mobility performance issue analysis |
| REQ-MRO_MDA-02 | 3GPP management system shall be able to provide the mobility issue analysis including too-early handovers, too-late handovers and ping-pong handovers. | (UC-MRO_MDA) mobility performance issue analysis |
| REQ-MRO_MDA-03 | 3GPP management system shall be able to identify the most optimal handover mechanism including DAPS, CHO or RACH-less handover. | (UC-MRO_MDA) mobility performance issue analysis |
| REQ-MRO_MDA-04 | 3GPP management system shall be able to provide the area specific mobility performance analysis. | (UC-MRO_MDA) mobility performance issue analysis |

#### 7.2.5.2	Handover optimization analysis

##### 7.2.5.2.1		Description

3GPP management system shall be able to provide the handover optimization analysis.

##### 7.2.5.2.2		Use cases

###### 7.2.5.2.2.1	Handover optimization

Current handover procedures are mainly based on radio conditions for selecting the target gNB upon a handover. The target gNB accepts or rejects the handover (HO) request depending on various conditions. In virtualized environment, the HO may be rejected due to inadequate available resources within the target gNB. The notion of resources may include virtual resources (e.g., compute, memory) and/or radio resources (e.g., PRB, RRC connected users). If the HO request is rejected, a UE will try to connect to a different gNB until the request is successfully accepted. Several target gNBs can be tried until the request is successfully accepted. This process can result in wastage of UE and network resources, while it may also introduce service disruption due to increased latency and radio link failures (RLFs). It also introduces inefficiency in the HO or other network procedures.

To address this handover optimization issue, it is desirable to use MDA (Management data analytics) to provision and/or select a particular target gNB for handover in order to reduce or even avoid HO rejections. The MDAS producer provides a HO optimization analytics output containing the current and future/predicted resource consumption, resources capabilities and other KPIs' status for the available target gNB(s). The analytics output also provides recommended actions to optimize the target gNB for handover. This may include resource re-configuration or the updated selection criteria for target gNB. Based on the output, the MDAS consumer adjusts (e.g., scale-out/up the virtual resource, re-schedule/optimize radio resource) the resources before continuing with the handover and/or adjusts the selection criteria of the target gNB by also considering the overlapping coverages of inter-frequency and inter-RAT deployments.

###### 7.2.5.2.2.2	Handover optimization based on UE Load

The target node, eNB, may not have adequate resources to accept certain handover requests. In the context of network virtualization, these resources may include not only legacy radio resources, but also virtual resources such as processor and memory. Handover optimization can benefit from knowledge about the projected UE load on the target cell including additional radio and virtual resources.

##### 7.2.5.2.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-MOB_MDA-01 | The MDAS producer should have a capability to provide the analytics output related to current statistics and future predictions of virtual resource consumption of gNB. | Handover optimization |
| REQ-MOB_MDA-02 | The MDAS producer should have a capability to provide the analytics output related to current statistics and future predictions of radio resource consumption of gNB. | Handover optimization |
| REQ-MOB_MDA-03 | The MDAS producer should have a capability to provide an analytics output indicating a selection priority for the target cell, among a set of candidate inter-frequency cells. | Handover optimization |
| REQ-MOB_MDA-04 | The MDAS producer should have a capability to provide an analytics output indicating a list of target cells to spare, i.e., avoid, a handover for an indicated time period. | Handover optimization |
| REQ-MOB_MDA-05 | The analytics output describing inter-frequency target cell selection for handover may provide information for provisioning or selecting a target gNB with respect to a specific service or slice, if the same Network Slice Instance (NSI) is available in both the current and target gNB. | Handover optimization |
| REQ-MOB_MDA-06 | The analytics output describing inter-frequency target cell selection for handover should provide indication of current and expected QoE (for the UE) at the current and target gNB. | Handover optimization |
| REQ-MOB_MDA-07 | The analytics output should contain the following information that can be used to optimize handover decisions: - Indication on whether the target gNB is optimal for handover. - Recommended action to optimize the target gNB and/or the selection of the target gNB for handover. | Handover optimization |
| REQ-MOB_MDA-08 | The MDAS producer should have a capability to provide an analytics output indicating the projected UE load with respect to virtual resource and radio resource on the target cell. | Handover optimization based on UE Load |

#### 7.2.5.3	Inter-gNB beam selection optimization

##### 7.2.5.3.1	Description

The MDA capability is about analytics of inter-gNB beam selection optimization.

##### 7.2.5.3.2	Use case

With the deployment of 5G networks, Massive MIMO has been used on a large scale. Beamforming, as a key technology to reduce user interference, which can suppress interference signals in non-target directions and enhance sound signals in target directions, is always combined with Massive MIMO to further decrease interference. A cell can make use of multiple beams for serving residing users (SSB or CSI-RS) with each user served by a single beam at a time. The cell level quality can be represented as an aggregated metric over one or more beams. So, although handover is performed between two 5G cells, the granularity of handover can be further broken down to beam level.

The handover of beams could be performed if the network resource or the user's state have changed to obtain better network performance. Beam optimization includes the handover between different beams and configuration of beam parameters.

In order to avoid selecting the wrong beam to perform RACH on the target cell and causing RLF of the UE, MDA can be used to recommend a means to prioritize and/or select the beam in case of handover for a specific target cell. MDA can provide a beam level HO optimization analysis considering information on the handover performance of different beam combinations between the source and target cell pairs. Beams of the target cell with a successful handover are preferred in the selection.

MDA could also provide recommended actions and priority options for beam selection. Based on the recommended actions, the MDA MnS consumer adjusts the priorities for the beam selection at HO, i.e., the beam combinations that are likely to succeed are prioritized, less optimal beam combinations are down prioritized. The target cell may also obtain analytics to allocate RACH resources in a way that ensures HO success.

In order to optimize antenna and beam configuration, so as to reduce energy loss and enhance network performance, MDA can be used to analyze the current network status.

##### 7.2.5.3.3		Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-HO_BEAM_OPT-01 | MDA for beam selection optimization shall be able to provide the analytics of the handover performance of beam pair combinations between cell pairs. | Inter-gNB beam selection optimization |
| REQ-HO_BEAM_OPT-02 | MDA for beam selection optimization shall be able to provide an indication if a beam pair is to be prioritized or down prioritized. | Inter-gNB beam selection optimization |
| REQ-HO_BEAM_OPT-03 | MDA for beam selection optimization shall be able to provide feasible antenna and beam configuration analysis. | Inter-gNB beam selection optimization |

### 7.2.6	MDA assisted critical maintenance management

#### 7.2.6.1	RAN Node Software Upgrade

##### 7.2.6.2.1	Description

This MDA capability is related to network critical maintenance during RAN node software upgrade process.

##### 7.2.6.2.2	Use case

As per the current mechanism of software upgrade at RAN node results in service disruption or huge operational cost. Consider a scenario, when a RAN Node is required to shut down manually to undergo critical maintenance for a very short duration of time. Software upgrade can be one such critical maintenance scenario. In such cases, all the resources (bearer, security functions, mobility management) that are managed by this RAN Node need to be purged and reconfigured at another RAN Node (standby RAN Node) or if another RAN Node is not available then resources will be reconfigured again when former RAN Node comes up after software upgrade. Both the situations lead to additional operational expenses and data loss. Operational expense in terms of all the resources to be released/attached again and data loss for all GBR sessions/bearer.

It is expected to use MDAS to optimize the procedure of software upgrade at RAN Node by providing the right time to execute the required upgrade. The software upgrade should be automatically initiated by the OAM system, once configured, during the time frame when the expected impacts are minimum i.e., at the optimal time when there would be minimum expected operational cost and data loss. The Optimal Time (current or futuristic) can be derived by collecting and analysing the data related to DRBs including GBR/non-GBR, state, modification count, ongoing handover etc. MDAS can utilize historical data and AI/ML (e.g., time series based) algorithm to derive the future optimal time frame for software upgrade.

Note: 	RAN Node above refers to CU-CP in case of gNB split case.

##### 7.2.6.2.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-SWA_MDA-01 | The MDAS producer should have a capability allowing the authorized consumer to get the DRB info analytics output describing the DRBs info at a particular RAN Node(s). | RAN Node Software Upgrade |
| REQ-SWA_MDA-02 | The MDAS producer should have a capability to provide the DRB info analytics output describing the DRB info based on the following DRB characteristics; type (GBR/non-GBR), state (idle/active), modification count (indicating number of times, this bearer has gone for modification since its creation), handover in-progress (indicates whether the bearer is undergoing handover or not). | RAN Node Software Upgrade |
| REQ-SWA_MDA-03 | The DRB info analytics output describing the DRB info should contain the following information: - Timeframe: Time frame/duration at which the output is generated - CurrentUpgradeOptimal: Whether RAN Node is optimal for upgrade at present - FutureUpgradeOptimal: Whether RAN Node will be optimal for upgrade during a future time frame. This will also provide a future frame. - DRB count: Total number of GBR and non-GBR DRBs at future point of time frame. This will also provide a future frame. | RAN Node Software Upgrade |

## 7.3	MDA MnS

Editor’s note: 1) this clause is about the necessary interactions between MDA MnS consumer and producer in order to get the MDA reports, 2) the titles of clause 7.3 and its subclauses are to be revisited.

### 7.3.1	MDA request and control

#### 7.3.1.1	Description

The MDA request and control allow any authorized MDA MnS consumer to request management data analytics.

#### 7.3.1.2	Use case

The MDA MnS consumer can request the MDA MnS producer to provide MDA output for a specified type of analytics, i.e., MDA type, which corresponds to a MDA use case. The MDA MnS consumer may introduce control attributes related to the MDA output with respect to the geographic location (i.e., area scope) and/or the target objects, e.g., managed elements, time schedule for obtaining a MDA output, time conditions related to the preparation of MDA output, and potential filter conditions to be met before a MDA output is made available, e.g., load or delay threshold crossing related to a target object. The geographical location indicates an area of interest for obtaining MDA output and target objects include affected objects or objects of interest for obtaining MDA output.

The MDA MnS consumer may control the MDA output attributes related to, e.g., time schedule, geographical location, target objects, etc., and has the capability to modify them at any point in time. The MDA MnS consumer can request the MDA MnS producer to generate an MDA output that contains numeric output results, e.g., average, normal distribution, etc., recommendation options, e.g., potential handover target cells, or root case analysis, e.g., alarm prediction.

The MDA MnS consumer can be informed with an acknowledgment if the request was successful. If the request was not successful, the consumer is informed about potential errors indicating the reasons. The MDA MnS consumer can also deactivate the MDA reporting control request once it is no longer needed.

#### 7.3.1.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-MDA-CONT-01 | The MDA MnS producer shall have capability to allow any authorized MDA MnS consumer to request MDA output, while indicating its selection on the MDA type. | All use cases |
| REQ-MDA-CONT-02 | The MDA MnS producer shall have the capability to allow any authorized MDA MnS consumer to request MDA output, while indicating its selection on the reporting time schedule. | All use cases |
| REQ-MDA-CONT-03 | The MDA MnS producer shall have the capability to allow any authorized MDA MnS consumer to request MDA output, while indicating its selection on geographic location and/or the target objects if applicable. | All use cases |
| REQ-MDA-CONT-04 | The MDA MnS producer shall have the capability to allow any authorized MDA MnS consumer to request MDA output, while indicating its selection on the time schedule related to MDA results. | All use cases |
| REQ-MDA-CONT-05 | The MDA MnS producer shall have the capability to allow any authorized MDA MnS consumer to modify the attributes related to the requested MDA output. | All use cases |
| REQ-MDA-CONT-6 | The MDA MnS producer shall have the capability to allow any authorized MDA MnS consumer to specify filter conditions on target objects based on threshold crossing for MDA output when this is applicable. | All use cases |

Editor’s Note: If applicable is for FFS.

### 7.3.2	Obtaining MDA Output

#### 7.3.2.1	Description

Following a successful MDA request any authorized MDA MnS consumer can obtain management data analytics from the corresponding MDA MnS producer. The MDA MnS consumer can control the MDA output by modifying the attributes related to the MDA request at any point in time.

#### 7.3.2.2	Use case

The MDA MnS consumer can obtain MDA output when the conditions indicated in the MDA request are met. A MDA output can contain one or more MDA results, which may be: (i) numeric, e.g., average,  etc., (ii) recommendation options, e.g., potential handover target cells, or (iii) root case analysis, e.g., alarm prediction. These results may be related to one or more MDA types, which corresponds to MDA use cases, and can also contain information regarding the time schedule or the validity time of the provided MDA output.

MDA MnS consumers can request and obtain different MDA output results, which shall contain the MDA context, i.e., the conditions under which an MDA result was prepared, e.g., network conditions.  The MDA MnS consumer may also obtain information regarding the geographic location and/or the target objects, e.g., managed elements, related to the provided MDA result – from the corresponding element.

The MDA MnS consumer may obtain MDA output results either by pulling or pushing mechanisms. Any MDA output may be obtained once it is prepared or when the specified MDA request and control  conditions are met.

#### 7.3.2.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-MDA_REP-01 | The MDA MnS producer shall have a capability allowing MDA MnS consumers to obtain analytics output from the MnS producer. | All use cases |
| REQ-MDA_REP-02 | The MDA MnS producer shall have a capability allowing MDA MnS consumers to indicate if produced analytics output shall be pushed to the MDA MnS consumer or whether the MDA MnS consumer pulls the data. | All use cases |
| REQ-MDA_REP-03 | The MDA MnS producer shall allow any authorized MDA MnS consumer to obtain the geographic location and/or the target objects related to the MDA output if applicable. | All use cases |
| REQ-MDA_REP-04 | The MDA MnS producer shall allow any authorized MDA MnS consumer to obtain time schedule information related to the MDA output. | All use cases |

## 7.4	Supporting aspects for MDA

Editor’s note: this clause is to accommodate the supporting aspects for MDA, such as ML for MDA, historical data handling for MDA where necessary.

### 7.4.1	Machine Learning (ML) for MDA

Editor’s note: it is TBD that whether the MLMT MnS is to be defined in this TS or in a generic TS.

#### 7.4.1.1	Description

##### 7.4.1.1.1	MDA with ML support

MDA may be supported by ML capabilities.

When MDA is supported by ML capabilities, one example of MDA MnS deployment scenario is  illustrated in Figure 7.X.1.1-1.

Figure 7.X.1.1-1: Deployment example of MDA with ML support

1. 	The Management Function (MDA MnS producer) collects the analytics inputs for MDA.

2.	The MDA MnS consumer requests the MDA reports and controls the MDA reporting from the MDA MnS producer.

3.	The Management Function (MDA MnS producer) processes the analytics inputs for MDA based on the inference of the (trained) ML model, and generates analytics/inference output.

NOTE: How MDA uses the ML model for inference is out of scope of the present specification.

4.	The Management Function (MDA MnS producer) generates the MDA report(s) based on the analytics/inference output, and provides the MDA report(s) to the MDA MnS consumer.

##### 7.4.1.1.2	ML model training

In operational environment before the ML model is deployed to conduct the inference, it needs to be trained (e.g., by an external entity).

The ML model is trained by the ML Model Training (MLMT) MnS producer, and the training can be triggered by the request(s) from one or more MLMT MnS consumer(s), or initiated by the MLMT MnS producer (e.g., as result of model evaluation).

#### 7.4.1.2	Use cases

##### 7.4.1.2.1	ML model training requested by consumer

The ML model training capabilities are provided by an MLMT MnS producer to one or more consumer(s).

Figure 7.X.1-1: ML model training requested by MLMT MnS consumer

The ML model training may be triggered by the request(s) from one or more MLMT MnS consumer(s).To trigger an ML model training, the MLMT MnS consumer requests the MLMT MnS producer to train the ML model. The consumer may provide the data source(s) that contain(s) the training data which are considered as inputs candidates for training.

The MLMT MnS producer provides a response to the consumer indicating whether the request was accepted.

If the request is accepted, the MLMT MnS producer decides when to start the ML model training with consideration of the request(s) from the consumer(s). Once the training is decided, the producer

-	selects the training data, with consideration of the consumer provided candidate training data. Since the training data directly influences the algorithm and performance of the trained model, the MLMT MnS producer may examine the consumer provided training data and select none, some or all of them. In addition, the MLMT MnS producer may select some other training data that are available.

-	trains the ML model using the selected training data , and

-	provides the training result (including the location of the trained model, etc) to the MLMT MnS consumer (s).

##### 7.4.1.2.2	ML model training initiated by producer

The ML model training may be initiated by the MLMT MnS producer, for instance as result of evaluation of performance of the ML model, based on the feedback received from the consumer, or when new training data describing the new network status/events are available.

When the MLMT MnS producer decides to start the ML model training, the producer

-	selects the training data,

-	trains the ML model using the selected training data, and

-	provides the training result (including the location of the trained model, etc) to the MLMT MnS consumer(s) who have subscribed to receive the ML model training results.

#### 7.4.1.3	Requirements for ML model training for MDA

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-MDA_ML-FUN-1 | The MLMT MnS producer shall have a capability allowing the consumer to request ML model training. | ML model training requested by consumer (clause 7.4.1.2.1) |
| REQ-MDA_ML-FUN-2 | The MLMT MnS producer shall have a capability allowing the consumer to specify the data sources containing the candidate training data for ML model training. | ML model training requested by consumer (clause 7.4.1.2.1) |
| REQ-MDA_ML-FUN-3 | The MLMT MnS producer shall have a capability to provide the training result (including the location of the trained model) to the consumer. | ML model training requested by consumer (clause 7.4.1.2.1), and ML model training initiated by producer (clause 7.4.1.2.2) |

# 8	MDA capability data definitions

## 8.1	Introduction

## 8.2	About analytics

### 8.2.1	About enabling data

Analytics are capability-specific, and the present document provides the enabling data for each MDA capability in the respective tables. It is not restrictive or mandatory to use the analytics inputs exactly the same as the provided enabling data for implementation, and other (additional or different) data are also allowed in order to facilitate the production of analytics outputs.

### 8.2.2	About analytics outputs

For analytics outputs, there are 1) common information elements that can be generated by MDA and be applicable for all MDA capabilities, and 2) capability-specific information elements, and 3) vendor specific extensions. The common information elements are provided in clause 8.3, and the capability-specific information elements are provided per MDA capability in clause 8.4 of the present document.

## 8.3	Common information elements of analytics outputs

## 8.4	Data definitions per MDA capability

### 8.4.1	Coverage related analytics

#### 8.4.1.1		Coverage problem analysis

##### 8.4.1.1.1	MDA type

The MDA type for coverage problem analysis is: CoverageAnalytics.CoverageProblemAnalysis.

##### 8.4.1.1.2	Enabling data

The enabling data for coverage problem analysis are provided in table 8.4.1.1.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.1.1.2-1: Enabling data for coverage problem analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | SS-RSRP distribution per SSB (beam) of serving NR cell | SS-RSRP distribution per SSB (clause 5.1.1.22.1 of TS 28.552 [4]). |
|  | SS-RSRP distribution per SSB (beam) of neighbor NR cell | Editor’s note: to be defined in TS 28.552 |
|  | RSRP distribution of neighbor E-UTRA cell for an NR cell | Editor’s note: to be defined in TS 28.552 |
|  | Power headroom distribution for NR cell | Type 1 power headroom distribution (clause 5.1.1.26.1 of TS 28.552 [4]). |
|  | Wideband CQI distribution for NR cell | Wideband CQI distribution (clause 5.1.1.11.1 of TS 28.552 [4]). |
|  | Timing Advance distribution for NR cell | Editor’s note: to be defined in TS 28.552 |
|  | Number of UE Context Release Request (gNB-DU initiated) | Number of UE Context Release Request (gNB-DU initiated) (clause 5.1.3.5.1 of TS 28.552 [4]). |
|  | Number of UE Context Release Request per SSB (gNB-DU initiated) | Editor’s note: to be defined in TS 28.552 |
|  | Number of UE Context Release Requests (gNB-CU initiated) | Number of UE Context Release Request (gNB-CU initiated) (clause 5.1.3.5.2 of TS 28.552 [4]). |
|  | Number of UE Context Release Requests per SSB (gNB-CU initiated) | Editor’s note: to be defined in TS 28.552 |
|  | RSRP related measurements for ng-eNB | RSRP related measurements (clause 6.1 of TS 32.425 [12]). |
|  | UE power headroom related measurements for ng-eNB | UE power headroom related measurements (clause 6.3 of TS 32.425 [12]). |
|  | Wideband CQI distribution for ng-eNB | Wideband CQI distribution (clause 4.10.1.1 of TS 32.425 [12]). |
|  | Average sub-band CQI for ng-eNB | Average sub-band CQI (clause 4.10.1.2 of TS 32.425 [12]). |
|  | UE Rx – Tx time difference related measurements for ng-eNB | UE Rx - Tx time difference related measurements (clause 6.4 of TS 32.425 [12]). |
|  | AOA related measurements for ng-eNB | AOA related measurements (clause 6.5 of TS 32.425 [12]). |
|  | Timing Advance distribution for ng-eNB | Timing Advance Distribution (clause 4.10.2 of TS 32.425 [12]). |
|  | Number of UE CONTEXT Release Request initiated by ng-eNodeB | Number of UE CONTEXT Release Request initiated by eNodeB/RN (clause 4.1.5.1 of TS 32.425 [12]). |
| MDT reports | MDT reports containing RSRPs of the serving cell and neighbour cells, and UE location. | RSRPs and UE location of M1 measurements for NR in TS 32.422 [6] and TS 32.423 [7]. |
| RLF reports | RLF reports containing RSRPs of the last serving cell and neighbour cells, and UE location. | RLF data collection and RLF reporting in TS 32.422 [6], and rlf-Report-r16 in TS 38.331 [13]. |
| RCEF reports | RCEF reports containing RSRPs of NR cell where the RRC connection establishment failed and neighbour cells, and UE location. | RCEF data collection and RCEF reporting in TS 32.422 [6], and ConnEstFailReport-r16 in TS 38.331 [13]. |
| UE location reports | UE location information provided by the LCS which can be used to correlate with the MDT reports. | The UE location information provided by LCS via service-based interface (see TS 23.273 [14]). |
| Geographical data | The geographical information (longitude, latitude, altitude) of the deployed RAN (NG-RAN and E-UTRAN). | Editor’s note: to be defined in TS TS 28.622/623 or 28.541. |
| Configuration data | The NRMs containing the attributes affecting the coverage for (NG-RAN and E-UTRAN). | NRCellDU IOC, NRSectorCarrier IOC, BWP IOC, CommonBeamformingFunction IOC, and Beam IOC in TS 28.541 [15]; EUtranGenericCell IOC in TS 28.658 [16]; SectorEquipmentFunction IOC, AntennaFunction IOC, and TMAFunction IOC in TS 28.662 [17]. |

##### 8.4.1.1.3	Analytics output

The specific information elements of the analytics output for coverage problem analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.1.1.3-1.

Table 8.4.1.1.3-1: Analytics output for coverage problem analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| CoverageProblemId | The identifier of the coverage problem. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CoverageProblemType | Indication of type of the coverage Problem. The allowed value is one of the enumerated values: WeakCoverage, CoverageHole, PilotPollution, Overshoot coverage, DlUlChannelCoverageMismatch, Other, Unknown. | M | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CoverageProblemAreas | Geographical location areas where the coverage problem occurred. | M | type: FFS multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ProblematicRAT | Indication of the RAT(s) where the coverage problem occurred. The allowed value is one of the enumerated values: NR, E-UTRA, Both. | M | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

# 9	Information model definitions for MDA

## 9.1	Imported and associated information entities

### 9.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |

### 9.1.2	Associated information entities and local labels

## 9.2	Class diagram

### 9.2.1	Relationships

This clause provides the relationships of relevant classes in UML.

Figure 9.2.1-1: NRM fragment for MDA Request

Editor’s note: The allowedValue of Proxy Entity is FFS.

### 9.2.2	Inheritance

Figure 9.2.2-1: Inheritance Hierarchy

## 9.3	Class definitions

### 9.3.1	MDARequest

#### 9.3.1.1	Definition

The IOC MDARequest represents the MDA output request and configuration parameters of MDA analysis service capabilities.

To request a desired MDA output, a MDAS consumer has to create MDARequest object instance on the MDAS producer.

#### 9.3.1.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mDAType | M | T | T | F | T |
| analyticsOutputFilters | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |

Editor's Note: whether the reportingMethod can be contained in MDARequest IOC is FFS.

#### 9.3.1.3	Attribute constraints

| Name | Definition |
| --- | --- |

#### 9.3.1.4	Notifications

The common notifications defined in clause 4.5 as defined in TS 28.541 [15] are valid for this IOC, without exceptions or additions.

## 9.4	Attribute definitions

### 9.4.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mDAType | It indicates the type of MDA capability for analysis. AllowedValues: Editor's Note: the allowed value is FFS which depends on the detailed analytic use case. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| analyticsOutputFilters | It indicates the analytics output filters of the dedicated MDA Type to be analyzed. Editor's Note: the data type of AnalyticsOutputFilters is FFS which depends on the detailed MDA Type. | type: AnalyticsOutputFilters multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

### 9.4.2	Constraints

| Name | Affected attribute(s) | Definition |
| --- | --- | --- |

## 9.5	Common notifications

This clause presents notifications that may be referred to by any class defined in the specification. This information is provided in tables.

### 9.5.1	Alarm notifications

| Name | Qualifier | Notes |
| --- | --- | --- |

### 9.5.2	Configuration notifications

| Name | Qualifier | Notes |
| --- | --- | --- |

### 9.5.3	Threshold Crossing notifications

| Name | Qualifier | Notes |
| --- | --- | --- |

# 10	MDA related service components

Editor’s note: Whether putting section 8 (MDA capability data definitions) and section 9 (Information model definitions for MDA) inside section 10 (MDA related service components) is to be evaluated for better readability when we have related content in this specification.

## 10.1	MDA MnS Service components

### 10.1.1	General

TBD

### 10.1.2	MDA report request and control

#### 10.1.2.1	Service components

### 10.1.3	MDA reporting

#### 10.1.3.1	Service components

# 11	Workflows for MDA management

Annex X (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-04 | SA5#137e | n/a | - | - | - | Initial skeleton | 0.0.0 |
| 2021-09 | SA5#138e | S5-214614 | - | - | - | Add abbreviations and an overview subclause | 0.1.0 |
| 2021-09 | SA5#138e | S5-214615 | - | - | - | Add text for the scope | 0.1.0 |
| 2021-09 | SA5#138e | S5-214616 | - | - | - | Add MDA role to the MDA in management loop | 0.1.0 |
| 2021-09 | SA5#138e | S5-214617 | - | - | - | Add text to MDA functionality and service framework clause | 0.1.0 |
| 2021-10 | SA5#139e | S5-215657 | - | - | - | Add structure for TS 28.104 | 0.2.0 |
| 2021-10 | SA5#139e | S5-215637 | - | - | - | Add description of fault prediction analysis | 0.2.0 |
| 2021-10 | SA5#139e | S5-215653 | - | - | - | Add service experience analysis | 0.2.0 |
| 2021-10 | SA5#139e | S5-215659 | - | - | - | Add network slice throughput analysis | 0.2.0 |
| 2021-10 | SA5#139e | S5-215561 | - | - | - | Add traffic Projection use case and requirements | 0.2.0 |
| 2021-10 | SA5#139e | S5-215206 | - | - | - | Add mobility performance analysis use case requirements | 0.2.0 |
| 2021-10 | SA5#139e | S5-215265 | - | - | - | Add MDA role in cross-domain service assurance | 0.2.0 |
| 2021-10 | SA5#139e | S5-215638 | - | - | - | Add example of MDA producers and consumers | 0.2.0 |
| 2021-10 | SA5#139e | S5-215658 | - | - | - | Add ML support for MDA | 0.2.0 |
| 2021-11 | SA5#140e | S5-216471 | - | - | - | Replace alarm incident with alarm information | 0.3.0 |
| 2021-11 | SA5#140e | S5-216472 | - | - | - | Add software management use case and requirements | 0.3.0 |
| 2021-11 | SA5#140e | S5-216473 | - | - | - | Add paging optimization use case and requirements | 0.3.0 |
| 2021-11 | SA5#140e | S5-216474 | - | - | - | Add HO optimization usecase and requirements | 0.3.0 |
| 2021-11 | SA5#140e | S5-216475 | - | - | - | Alignment of terminology | 0.3.0 |
| 2021-11 | SA5#140e | S5-216476 | - | - | - | Add coverage analysis requirement | 0.3.0 |
| 2021-11 | SA5#140e | S5-216350 | - | - | - | Add MDA capability for coverage problem analysis | 0.3.0 |
| 2021-11 | SA5#140e | S5-216556 | - | - | - | Add stage 2 structure for TS 28.104 | 0.3.0 |
| 2021-11 | SA5#140e | S5-216477 | - | - | - | Add MDA assisted Energy Saving use case and requirements | 0.3.0 |
| 2021-11 | SA5#140e | S5-216479 | - | - | - | Add MDA Request and Control | 0.3.0 |
| 2021-11 | SA5#140e | S5-216478 | - | - | - | Add obtaining MDA output | 0.3.0 |
| 2021-11 | SA5#140e | S5-216480 | - | - | - | Add E2E latency analysis use case and requirements | 0.3.0 |
| 2021-11 | SA5#140e | S5-216481 | - | - | - | Add MDA related data, use case and requirements | 0.3.0 |
| 2021-11 | SA5#140e | S5-216552 | - | - | - | Add network slice load analysis use case and requirements | 0.3.0 |
| 2021-11 | SA5#140e | S5-216470 | - | - | - | Add inter-gNB beam selection optimization | 0.3.0 |
| 2021-11 | SA5#140e | S5-216553 | - | - | - | Add slice coverage analysis | 0.3.0 |
| 2021-11 | SA5#140e | S5-216554 | - | - | - | Add requirements for ML model training | 0.3.0 |
| 2021-11 | SA5#140e | S5-216555 | - | - | - | Add MnS producer initiated ML model training | 0.3.0 |
| 2021-11 | SA5#140e | S5-216622 | - | - | - | Add NRM for MDA request | 0.3.0 |