3GPP TSG SA WG5 Meeting 141-e		S5-221140

- Electronic meeting, online, 17 - 21 Jan. 2022

- Source:	Huawei

- Title:	Add E2E latency analysis solution

- Document for:	Approval

Agenda Item:	6.4.15

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-020 “Management and orchestration; Management Data Analytics”

# 3	Rationale

E2E latency is an important parameter for 5G services such as URLLC. User data packets should be successfullydelivered within certain time constraints to satisfy the end users requirements. Latency could be impacted by the network capability and network configurations, e.g. configuration of service priority, RAN capacity, network load, number of re-transmissions, Wireless channel environment and the processing time of the network functions, etc. And the affected objects, the root cause of the too high latency and solutions to solve the latency issue need to be discussed.

This contribution is proposed to provide the E2E latency analysis capability of MDA in [2].

# 4	Detailed proposal

It is proposed to apply following changes in [2].

First of the change

- 8.4.x	SLS analysis

- 8.4.x.4	  E2E latency analysis

- 8.4.x.4.1	  MDA type

The MDA type for Capability-E2E latency analysis is: SLSAnalysis. E2ElatencyAnalysis.

- 8.4.x.4.2	   Enabling data

The enabling data for E2E latency analysis are provided in table 8.4.x.4.2-1.

Table 8.4.x.4.2-1: Enabling data for E2E latency analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | Average e2e uplink/downlink delay for a network slice | Average e2e uplink delay for a network slice as defined in 6.3.1.8.1 in TS 28.554 [5]; |
|  | Average e2e downlink delay for a network slice | Average e2e downlink delay for a network slice as defined in 6.3.1.8.2 in TS 28.554 [5]. |

- 8.4.x.4.3	Analytics output

The specific information elements of the analytics output for E2E latency analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.x.x.2-1.

Table 8.4.x.4.3-1:  E2E latency analytics output

| Information | Description | Support qualifier | Properties |
| --- | --- | --- | --- |
| E2ELatencyIssueId | The identifier indicates the output is for E2E latency issue analysis | M | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| E2ELatencyIssueType | Indication the type of the E2E latency issue. The allowed value is one of the enumerated values: RAN latency issue, CN latency issue, TN latency issue, UE latency issue, Service provider latency issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AffectedObjects | The managed object instances of subnetwork, managed elements or network slices where the latency issue happens | O | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of the change