- 3GPP TSG-SA5 Meeting #141e	S5-221357rev3

- 17 - 26 January 2022, E-meeting

- Source:	Intel

- Title:	pCR 32.257 Add charging information definition for edge enabling services charging

- Document for:	Approval

- Agenda Item:	7.4.2

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TS 32.257-020 “Edge Computing domain charging; stage 2”

# 3	Rationale

This pCR is to add charging information definition for edge enabling services charging to TS 32.257.

# 4	Detailed proposal

Start of modification

## 6.4	Definition of charging information for edge enabling services charging

### 6.4.1	Data description for edge enabling services charging

#### 6.4.1.1	Message contents

##### 6.4.1.1.1	General

The Charging Data Request and Charging Data Response are specified in TS 32.290 [6] and include charging information. The Charging Data Request can be of type [Event].

Table 6.4.1.1.1-1 describes the use of these messages for converged charging.

Table 6.4.1.1.1-1: Converged charging messages reference table

| Message | Source | Destination |
| --- | --- | --- |
| Charging Data Request | EES | CHF |
| Charging Data Response | CHF | EES |

The following clauses describe the different fields used in the Charging Data messages and the category in the tables is used according to the charging data configuration defined in clause 5.4 of TS 32.240 [2].

##### 6.4.1.1.2	Charging Data Request message

Table 6.4.1.1.2-1 illustrates the basic structure of a Charging Data Request message from the EES as used for edge enabling services converged charging.

Table 6.4.1.1.2-1: Charging Data Request message contents

| Information Element | Converged Charging Category | Description |
| --- | --- | --- |
| Session Identifier | - | Described in TS 32.290 [6]. |
| Subscriber Identifier | - | Described in TS 32.290 [6]. |
| NF Consumer Identification | M | Described in TS 32.290 [6]. |
| NF Functionality | M | Described in TS 32.290 [6]. |
| NF Name | OC | Described in TS 32.290 [6]. |
| NF Address | OC | Described in TS 32.290 [6]. |
| NF PLMN ID | OC | Described in TS 32.290 [6]. |
| Charging Identifier | OM | Described in TS 32.290 [6]. |
| Invocation Timestamp | M | Described in TS 32.290 [6]. |
| Invocation Sequence Number | - | Described in TS 32.290 [6]. |
| Retransmission Indicator | OC | Described in TS 32.290 [6]. |
| One-time Event | OC | Described in TS 32.290 [6]. |
| One-time Event Type | OC | Described in TS 32.290 [6]. |
| Notify URI | - | Described in TS 32.290 [6]. |
| Supported Features | OC | Described in TS 32.290 [6]. |
| Service Specification Information | OC | Described in TS 32.290 [6]. |
| Triggers | OC | Described in TS 32.290 [6]. |
| Multiple Unit Usage | OM | Described in TS 32.290 [6]. |
| Rating Group | OM | Described in TS 32.290 [6]. |
| Requested Unit | - | Described in TS 32.290 [6]. |
| Time | - | Described in TS 32.290 [6]. |
| Total Volume | - | Described in TS 32.290 [6]. |
| Uplink Volume | - | Described in TS 32.290 [6]. |
| Downlink Volume | - | Described in TS 32.290 [6]. |
| Service Specific Units | - | Described in TS 32.290 [6]. |
| Used Unit Container | - | Described in TS 32.290 [6]. |
| Service Identifier | - | Described in TS 32.290 [6]. |
| Quota management Indicator | - | Described in TS 32.290 [6]. |
| Triggers | - | Described in TS 32.290 [6]. |
| Trigger Timestamp | - | Described in TS 32.290 [6]. |
| Time | - | Described in TS 32.290 [6]. |
| Total Volume | - | Described in TS 32.290 [6]. |
| Uplink Volume | - | Described in TS 32.290 [6]. |
| Downlink Volume | - | Described in TS 32.290 [6]. |
| Service Specific Unit | - | Described in TS 32.290 [6]. |
| Event Time Stamps | - | Described in TS 32.290 [6]. |
| Local Sequence Number | - | Described in TS 32.290 [6]. |
| EAS ID | OM | This field holds the EAS ID, see TS 23.558 [9]. |
| EAS Provider Identifier | OM | The identifier of the ASP that provides the EAS, see TS 23.558 [9]. |
| Direct Edge Enabling Service Charging Information | OM | This field holds the charging information described in clause 6.3.2.1.2 specific for charging of the edge enabling services directly provided by EES |
| Indirect 5G NF Service Charging Information | OM | This field holds the charging information described in clause 6.3.2.1.3 specific for charging of the 5G NF services indirectly exposed by EES (the detaild charging information is FFS) |

##### 6.4.1.1.3	Charging data response message

Table 6.4.1.1.3-1 illustrates the basic structure of a Charging Data Response message from the CHF as used for edge enabling services converged charging.

Table 66.4.1.1.3-1: Charging Data Response message contents

| Information Element | Converged Charging Category | Description |
| --- | --- | --- |
| Session Identifier | M | Described in TS 32.290 [6]. |
| Invocation Timestamp | M | Described in TS 32.290 [6]. |
| Invocation Result | OC | Described in TS 32.290 [6]. |
| Invocation Result Code | OC | Described in TS 32.290 [6]. |
| Failed parameter | OC | Described in TS 32.290 [6]. |
| Failure Handling | OC | Described in TS 32.290 [6]. |
| Invocation Sequence Number | - | Described in TS 32.290 [6]. |
| Session Failover | - | Described in TS 32.290 [6]. |
| Supported Features | OC | Described in TS 32.290 [6]. |
| Triggers | - | Described in TS 32.290 [6]. |
| Multiple Unit Information | - | Described in TS 32.290 [6]. |
| Result Code | - | Described in TS 32.290 [6]. |
| Rating Group | - | Described in TS 32.290 [6]. |
| Granted Unit | - | Described in TS 32.290 [6]. |
| Tariff Time Change | - | Described in TS 32.290 [6]. |
| Time | - | Described in TS 32.290 [6]. |
| Total Volume | - | Described in TS 32.290 [6]. |
| Uplink Volume | - | Described in TS 32.290 [6]. |
| Downlink Volume | - | Described in TS 32.290 [6]. |
| Service Specific Units | - | Described in TS 32.290 [6]. |
| Validity Time | - | Described in TS 32.290 [6]. |
| Final Unit Indication | - | Described in TS 32.290 [6]. |
| Time Quota Threshold | - | Described in TS 32.290 [6]. |
| Volume Quota Threshold | - | Described in TS 32.290 [6]. |
| Unit Quota Threshold | - | Described in TS 32.290 [6]. |
| Quota Holding Time | - | Described in TS 32.290 [6]. |
| Triggers | - | Described in TS 32.290 [6]. |

#### 6.4.1.2	Ga message contents

See clause 5.2.4.4.

#### 6.4.1.3	CDR description on the Bee interface

##### 6.4.1.3.1	General

This clause describes the CDR content and format generated for edge enabling services charging.

The following tables provide a brief description of each CDR parameter. The category in the tables is used according to the charging data configuration defined in clause 5.4 of TS 32.240 [2]. Full definitions of the CDR parameters, sorted by the name in alphabetical order, are provided in TS 32.298 [3].

##### 6.4.1.3.2	Edge enabling services charging CHF CDR data

If enabled, CHF CDRs for edge enabling services charging shall be produced for each triggering event.

The fields of edge enabling services charging CHF CDR are specified in table 6.4.1.3.2-1.

Table 6.4.1.3.2-1: Edge enabling services charging CHF record data

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | Described in TS 32.298 [3] |
| Recording Network Function ID | OM | Described in TS 32.298 [3] |
| Charging Session Identifier | - | Described in TS 32.298 [3] |
| Subscriber Identifier | - | Described in TS 32.298 [3] |
| Tenant Identifier | OM | Described in TS 32.298 [3] |
| MnS Consumer Identifier | OC | Described in TS 32.298 [3] |
| NF Consumer Information | M | This field holds the information of the CEF that used the charging service. |
| NF Functionality | M | This field contains the function of the node (i.e. CEF) |
| NF Name | OC | This field holds the name of the CEF used. |
| NF Address | OC | This fields holds the IP Address of the CEF used. |
| NF PLMN ID | OC | This field holds the PLMN identifier (MCC MNC) of the CEF. |
| Charging Identifier | OM | Described in TS 32.298 [3] |
| Triggers | OM | Described in TS 32.298 [3] |
| SMF Triggers | - | Described in TS 32.298 [3] |
| List of Multiple Unit Usage | OM | Described in TS 32.298 [3] |
| Rating Group | OM | Described in TS 32.298 [3] |
| Used Unit Container | - | Described in TS 32.298 [3] |
| Service Identifier | - | Described in TS 32.298 [3] |
| Quota management Indicator | - | Described in TS 32.298 [3] |
| Local Sequence Number | - | Described in TS 32.298 [3] |
| Time | - | Described in TS 32.298 [3] |
| Uplink Volume | - | Described in TS 32.298 [3] |
| Downlink Volume | - | Described in TS 32.298 [3] |
| Total Volume | - | Described in TS 32.298 [3] |
| Service Specific Units | - | Described in TS 32.298 [3] |
| Event Time Stamp | - | Described in TS 32.298 [3] |
| Rating Indicator | - | Described in TS 32.298 [3] |
| Triggers | - | Described in TS 32.298 [3] |
| SMF Triggers | - | Described in TS 32.298 [3] |
| Trigger Time Stamp | - | Described in TS 32.298 [3] |
| PDU Container Information | - | Described in TS 32.298 [3] |
| NSPA Container Information | - | Described in TS 32.298 [3] |
| UPF ID | - | Described in TS 32.298 [3] |
| Record Opening Time | - | Described in TS 32.298 [3] |
| Duration | M | Described in TS 32.298 [3] |
| Record Sequence Number | C | Described in TS 32.298 [3] |
| Cause for Record Closing | M | Described in TS 32.298 [3] |
| Local Record Sequence Number | OM | Described in TS 32.298 [3] |
| Record Extensions | OC | Described in TS 32.298 [3] |
| Service Specification Information | OC | Described in TS 32.298 [3] |
| EAS ID | OM | This field holds the EAS ID, see TS 23.558 [9]. |
| EAS Provider Identifier | OM | The identifier of the ASP that provides the EAS, see TS 23.558 [9]. |
| Direct Edge Enabling Service Charging Information | OM | This field holds the charging information described in clause 6.3.2.1.2 specific for charging of the edge enabling services directly provided by EES |
| Indirect 5G NF Service Charging Information | OM | This field holds the charging information described in clause 6.3.2.1.3 specific for charging of the 5G NF services indirectly exposed by EES (the detaild charging information is FFS) |

6.4.2	Edge enabling services charging specific parameters

#### 6.4.2.1	Definition of edge enabling services charging information

##### 6.4.2.1.1	General

The Charging Information parameter used for edge enabling services charging is provided in the following clauses.

##### 6.4.2.1.2	Definition of direct edge enabling services charging specific charging information

Specific charging information used for edge enabling services charging is provided within the Edge Enabling Service Charging Information.

The detailed structure of the Edge Enabling Service Charging Information can be found in table 6.4.2.1.2-1.

Table 6.4.2.1.2-1: Structure of Direct Edge Enabling Service Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| EAS Type | O | The category or type of EAS (e.g. V2X), see TS 23.558 [9]. |
| EAS Geographical Service Area | O | EAS service area for identifying UEs with matching expected geographical location(s) (e.g. route), see TS 23.558 [9]. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| EAS Topological Service Area | O | The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area, see TS 23.558 [9]. ACs in UEs that are located outside this area shall not be served. |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows), see TS 23.558 [9]. |
| EAS Service KPIs | O | Service characteristics provided by EAS, see TS 23.558 [9]. |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS, see TS 23.558 [9]. |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS, see TS 23.558 [9]. |
| EAS Service continuity Support | O | Indicates if the EAS supports service continuity or not, and which ACR scenarios are supported, see TS 23.558 [9]. |
| EEC Service Continuity Support | O | Indicates if the EEC supports service continuity or not, see TS 23.558 [9]. The IE also indicates which ACR scenarios are supported by the EEC or, if this message is sent by the EEC to discover a T-EAS, which ACR scenario(s) are intended to be used for the ACR. |
| EES Service Continuity Support | O | The IE indicates if the S-EES supports service continuity or not, see TS 23.558 [9]. The IE also indicates which ACR scenarios are supported by the S-EES or, if the EAS discovery is used for an S-EES executed ACR, which ACR scenario is to be used for the ACR. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token), see TS 23.558 [9]. |
| UE Location | O | The location information of the UE, see TS 23.558 [9]. |
| ACR Action | OM | Indicates the ACR action (ACR initiation or ACR determination), see TS 23.558 [9]. |
| ACR Initiation data | O | ACR initiation IEs to be included in an ACR request message when ACR action indicates it is ACR initiation request, see TS 23.558 [9]. |
| DNAI of the T-EAS | O | DNAI information associated with the T-EAS or potential target EAS, see TS 23.558 [9]. |
| EEC Context ID | O | Identifier of the EEC Context, see TS 23.558 [9]. |
| S-EES ID | O | Identifier of the EES that provided EEC context ID, see TS 23.558 [9]. |
| T-EES ID | O | Identifier of the T-EES, see TS 23.558 [9]. |
| ACT Result | O | Indicates whether the ACT was successful or failed, see TS 23.558 [9]. |

##### 6.4.2.1.3	Definition of Indirect 5G NF Service specific charging information

Editor’s note: to be defined.

|  |  | - |
| --- | --- | --- |
|  |  | - |

#### 6.4.2.2	Formal edge enabling services charging parameter description

##### 6.4.2.2.1	Edge enabling services CHF CDR parameters

Editor’s note: The detailed definitions, abstract syntax and encoding of edge enabling services CHF CDRs parameters are to be specified in TS 32.298 [3].

##### 6.4.2.2.2	Edge enabling services resources attributes

Editor’s note: The detailed definitions of resources attributes used for edge enabling services charging are to be specified in TS 32.291 [7].

#### 6.4.2.3	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by EES for edge enabling services converged charging.

The Operation Types are listed in the following order: I (Initial)/U (Update)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as IUTE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IUT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. IU-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.4.2.3-1 defines the basic structure of the supported fields in the Charging Data Request message for edge enabling services converged charging.

Table 6.4.3.1: Supported fields in Charging Data Request message

| Information Element | Functionality of EES | Edge enabling services charging |
| --- | --- | --- |
|  | Supported Operation Types | I/E |
| Session Identifier |  | - |
| Subscriber Identifier |  | - |
| NF Consumer Identification |  | I/E |
| NF Functionality |  | I/E |
| NF Name |  | I/E |
| NF Address |  | I/E |
| NF PLMN ID |  | I/E |
| Charging Identifier |  | I/E |
| Invocation Timestamp |  | I/E |
| Invocation Sequence Number |  | - |
| Retransmission Indicator |  | I/E |
| One-time Event |  | I/E |
| One-time Event Type |  | I/E |
| Notify URI |  | - |
| Supported Features |  | I/E |
| Service Specification Information |  | I/E |
| Triggers |  | I/E |
| Multiple Unit Usage |  | - |
| Rating Group |  | - |
| Requested Unit |  | - |
| Time |  | - |
| Total Volume |  | - |
| Uplink Volume |  | - |
| Downlink Volume |  | - |
| Service Specific Units |  | - |
| Used Unit Container |  | - |
| Service Identifier |  | - |
| Quota management Indicator |  | - |
| Triggers |  | - |
| Trigger Timestamp |  | - |
| Time |  | - |
| Total Volume |  | - |
| Uplink Volume |  | - |
| Downlink Volume |  | - |
| Service Specific Unit |  | - |
| Event Time Stamps |  | - |
| Local Sequence Number |  | - |
| EAS ID |  | I/E |
| EAS Provider Identifier |  | I/E |
| Direct Edge Enabling Service Charging Information |  | I/E |
| Indirect 5G NF Service Charging Information |  | FFS |

Table 6.4.2.3-2 defines the basic structure of the supported fields in the Charging Data Response message for edge enabling services converged charging.

Table 6.4.2.3-2: Supported fields in Charging Data Response message

| Information Element | Functionality of EES | Edge enabling services charging |
| --- | --- | --- |
|  | Supported Operation Types | I/E |
| Session Identifier |  | I/E |
| Invocation Timestamp |  | I/E |
| Invocation Result |  | I/E |
| Invocation Result Code |  | I/E |
| Failed parameter |  | I/E |
| Failure Handling |  | I/E |
| Invocation Sequence Number |  | - |
| Session Failover |  | - |
| Supported Features |  | I/E |
| Triggers |  | - |
| Multiple Unit Information |  | - |
| Result Code |  | - |
| Rating Group |  | - |
| Granted Unit |  | - |
| Tariff Time Change |  | - |
| Time |  | - |
| Total Volume |  | - |
| Uplink Volume |  | - |
| Downlink Volume |  | - |
| Service Specific Units |  | - |
| Validity Time |  | - |
| Final Unit Indication |  | - |
| Time Quota Threshold |  | - |
| Volume Quota Threshold |  | - |
| Unit Quota Threshold |  | - |
| Quota Holding Time |  | - |
| Triggers |  | - |

6.4.3	Bindings for edge enabling services converged charging

Editor’s note: This mapping between the Information Elements, resource attributes and CHF CDR parameters for edge enabling services converged charging is to be described in TS 32.291 [7].

End of modifications