3GPP TSG SA WG5 Meeting 141-e		S5-221610

- Electronic meeting, online, 17 - 26 Nov. 2022

- Source:	Huawei

- Title:	Add network slice throughput analysis solution

- Document for:	Approval

Agenda Item:	6.4.15

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-020 “Management and orchestration; Management Data Analytics”

# 3	Rationale

Multiple SLS related attributes are defined in ServiceProfile, e.g., maxNumberofUEs, latency, throughput or etc, which are of great importance and should be satisfied. SLS analysis issues are studied in [1], including not only latency analysis, load analysis, jitter analysis.For different network services or vertical users, the requirements may varies a lot. This contribution is proposed to add the network slice throughput analysis capability of MDA in [2].

# 4	Detailed proposal

It is proposed to apply following changes in [2].

First of changes

- 8.4.x	SLS analysis

- 8.4.x.2	  Network slice throughput analysis

- 8.4.x.2.1	  MDA type

The MDA type for Capability- Network slice throughput analysis is: SLSAnalysis.NetworkSliceThroughputAnalysis

- 8.4.x.2.2	   Enabling data

The enabling data for network slice throughput analysis are provided in table 8.4.x.2.2-1.

Table 8.4.x.2.2-1: Enabling data for network slice throughput analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | UL/DL throughput for network and Network Slice Instance | Upstream throughput for network and Network Slice Instance as defined in 6.3.3 in TS28.554 [5]; Downstream throughput for Single Network Slice Instance as defined in 6.3.4 in TS28.554 [5] |
|  | RAN UE Throughput | RAN UE Throughput as defined in 6.3.6 in TS28.554 [5] |
|  | Throughput at N3 interface | Upstream Throughput at N3 interface as defined in 6.3.4 in TS28.554 [5]; Downstream Throughput at N3 interface as defined in 6.3.5 in TS28.554 [5]; |

- 8.4.x.2.3	Analysis output

The specific information elements of the analytics output for network slice throughput analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.x.2.3-1.

Table 8.4.x.2.3X.x.x.x.2-1:  Network slice throughput analysis output

| Information | Description | Support qualifier | Properties |
| --- | --- | --- | --- |
| NetworkSliceThroughputIssueId | Network slice throughput issue identifier | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceThroughputIssueType | Indication of the network slice throughput issue type The allowed value is one of the enumerated values: RAN issue, CN issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceThroughputUserStatistics | The statistics of the UL and/or DL network slice throughput in a certain time period. The value indicates the average percentage of users, for which the required SLS throughput satisfies | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceThroughputTimeStatistics | The statistics of the UL and/or DL network slice throughput in a certain time period. The value indicates the average percentage of time, during which the required SLS throughput satisfies | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceThroughputUserPredictions | The predictions of the UL and/or DL network slice throughput in a certain time period. The value indicates the average percentage of users, for which the required SLS throughput could be met | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceThroughputTimePredictions | The predictions of the UL and/or DL network slice throughput in a certain time period. The value indicates the average percentage of time, during which the required SLS throughput could be met. | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of changes