3GPP TSG SA WG5 Meeting 141-e		S5-221712

- Electronic meeting, online, 17 - 26 Nov. 2022

- Source:	Huawei

- Title:	Add service experience analysis solution

- Document for:	Approval

Agenda Item:	6.4.15

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[1]	TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-020 “Management and orchestration; Management Data Analytics”

# 3	Rationale

Service experience of end user is key indicator which directly reflects the user satisfaction degree. In 5G system, the diversity of network service is explored and the reuqirements of different service especially form vertical users are standardized. For example, the users who require URLLC service have high requirements on the network latency and reliability, while the users who require NB-IoT service considers the maximum access UE numbers as the first priority for network requirement.

This contribution is proposed to provide the service experience analysis capability of MDA in [2].

# 4	Detailed proposal

It is proposed to apply following changes in [2].

First of changes

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.500: "Telecommunication management; Self-Organizing Networks (SON); Concepts and requirements".

[3]	3GPP TS28.535: “Management services for communication service assurance; Requirements”.

[4]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[5]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[6]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[7]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[8]	3GPP TS 28.405: "Telecommunication management, Quality of Experience (QoE) measurement collection; Control and configuration".

[9]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[10]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[11]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[12]	3GPP TS 32.425: "Telecommunication management; Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[13]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[14]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[15]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[16]	3GPP TS 28.658: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[17]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM); Information Service (IS)".

[x]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[y]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

Second of changes

- 8.4.x	SLS analysis

- 8.4.x.1	  Service experience analysis

- 8.4.x.1.1	   MDA type

The MDA type for Capability-Service experience analysis is: SLSAnalysis. ServiceExperienceAnalysis.

- 8.4.x.1.2	   Enabling data

The enabling data for service experience analysis are provided in table 8.4.x.1.2-1.

Table 8.4.x.1.2-1: Enabling data for service experience analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | Average e2e uplink/downlink delay for a network slice | Average e2e uplink/downlink delay for a network slice (in 6.3.1.8 in TS 28.554 [5]); |
|  | Integrated uplink/downlink delay in RAN | Integrated downlink delay in RAN (6.3.1.2 in TS 28.554 [5]); Integrated uplink delay in RAN (6.3.1.7 in TS 28.554 [5]); |
|  | Round-trip packet delay | Round-trip packet delay between PSA UPF and NG-RAN (5.4.8 in TS 28.552 [4]) |
|  | UL/DL throughput for network and Network Slice Instance | Upstream throughput for network and Network Slice Instance (6.3.3 in TS28.554 [5]); Downstream throughput for Single Network Slice Instance (6.3.4 in TS28.554 [5]) |
|  | RAN UE Throughput | RAN UE Throughput (6.3.6 in TS28.554 [5]) |
|  | Throughput at N3 interface | Upstream Throughput at N3 interface (6.3.4 in TS28.554 [5]); Downstream Throughput at N3 interface (6.3.5 in TS28.554 [5]); |
| QoE Data | The QoE data of the different services | QoE data (TS 26.247 [x] and TS 26.114 [y] can be acquired through the procedures defined in TS 28.405 [8]). |

- 8.4.x.1.3	Service experience analysis output

The specific information elements of the analytics output for service experience analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.x.x.2 -1.

Table 8.4.x.1.3x.x.x.x.2-1:  Service experience analysis output

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| ServiceExperienceId | The identifier indicates the analytics report is related with service experience analysis. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ServiceExperienceIssueType | Indication of the service experience issue type. The allowed value is one of the enumerated values: RAN issue, CN issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AffectedObjects | The managed object instances where the service experience is applicable, e.g., SubNetwork Instance, NetworkSlice Instance, S-NSSAI. | O | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ServiceExperienceStatistics | The statistics of the level of service experience for a service in a certain time period, e.g. there are five levels which are represented by 1, 2, 3, 4, 5 where level 1 represents the users are enduring bad experience while level 5 represents the users’ requirements are perfectly satisfied. | O | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ServiceExperiencePredictions | The predictions of the level of service experience for a service in a certain time period. | O | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of changes