3GPP TSG-SA5 Meeting #141-e 	S5-221205

# e-meeting, 17 -26 January 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 0661 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add containment name of IMS SBA nodes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | adNRM |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | S5-216407 comments should have stage 3 for 3 IOCs CSCFFunction, HSSFunction, PCSCFFunction as the containment hierarchy needed in 28.541. This document is to have stage 2 for those IOCs in 28.541. As an alternative, the stage 2 may be also included in 28.706. This alternaive would be investiaged in other tdoc. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This tdoc is to add stage 2 for 3 IOCs of IMS SBA nodes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The YAML and YANG for IOCs of CSCFFunction, HSSFunction, PCSCFFunction are not covered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

5A	Information model definitions for SBA support of IMS

5A.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [30], IOC, SubNetwork | SubNetwork |
| TS 28.622 [30], IOC, ManagedElement | ManagedElement |
| TS 28.622 [30], IOC, ManagedFunction | ManagedFunction |
| TS 28.622 [30], IOC, EP_RP | EP_RP |
| TS 28.705 [xx], IOC, CSCFFunction | CSCFFunction |
| TS 28.705 [xx], IOC, HSSFunctionHSS | HSSFunction |
| TS 28.705 [xx], IOC, PCSCFFunction | PCSCFFunction |

5A.2	Class diagram

5A.2.1	Class diagram of IMS SBA nodes

5A.2.1.1		Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for NRM of IMS SBA nodes definitions. This clause provides the overview of the relationships of relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes. The information relevant for IMS network resource information is described in TS 28.705 [72].

Figure 5A.2.1.1-1 shows the IMS SBA nodes (SCSCFFuncrtion, PCSCFFunction) NRM containment/naming relationshop.

Figure 5A.2.1.1-1: the IMS SBA nodes NRM containment/naming relationshop.

The set of classes IOC EP_RP for SBA support of SCSCFFunction, HSSFunction, PCSCFFunction and PCFFunction are described in this clause.

The Figure 5A.2.1.1-2 shows the transport view of SCSCFFunction NRM for SBA interfaces.

Figure 5A.2.1.1-2: Transport view of SCSCFFunction for SBA interfaces

The Figure 5A.2.1.1-3 shows the transport view of HSSFunction NRM for SBA interfaces.

Figure 5A.2.1.1-3: Transport view of HSSFunction for SBA interfaces

The Figure 5A.2.1.1-4 shows the transport view of PCFFunction NRM for SBA interfaces.

Figure 5A.2.1.1-4: Transport view of PCFFunction for SBA interfaces

The Figure 5A.2.1.1-5 shows the transport view of PCSCFFunction NRM for SBA interfaces.

Figure 5A.2.1.1-5: Transport view of PCSCFFunction for SBA interfaces

5A.2.1.2	Inheritance

Figure 5A.2.1.2-1 shows the inheritance hierarchy from IOC ManagedFunction related to the IMS SBA nodes.

Figure 5A.2.1.2-2: Inheritance hierarchy from IOC ManagedFunction related to the IMS SBA nodes.

Figure 5A.2.1.2-2 shows the inheritance hierarchy from IOC EP_RP related to SBA interfaces of IMS nodes.

Figure 5A.2.1.2-2: Inheritance hierarchy from IOC EP_RP related to SBA interfaces of IMS

5A.3	Class definitions

5A.3.1	EP_N5

5A.3.1.1	Definition

This IOC represents the N5 interface between P-CSCF and PCF, which is defined in 3GPP TS 23.501 [2].

5A.3.1.2	Attributes

The EP_N5 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

5A.3.2	EP_N70

5A.3.2.1	Definition

This IOC represents the N70 interface between S/I-CSCF and HSS, which is defined in 3GPP TS 23.501 [2].

5A.3.2.2	Attributes

The EP_N70 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

5A.3.3	EP_N71

5A.3.3.1	Definition

This IOC represents the N71 interface between AF and HSS, which is defined in 3GPP TS 23.501 [2].

5A.3.3.2	Attributes

The EP_N71 IOC includes attributes inherited from EP_RP IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| localAddress | O | T | T | F | T |
| remoteAddress | O | T | T | F | T |

5A.3.4	SCSCFFunction

5A.3.4.1	Definition

This IOC represents the SCSCF function. For more information about the SCSCF, see 3GPP TS 23.002 [x].

5A.3.4.2	Attributes

The SCSCFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| commModelList | M | T | T | F | T |

5A.3.4.3	Attribute constraints

None.

5A.3.4.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

5A.3.5	PCSCFFunction

5A.3.5.1	Definition

This IOC represents the PCSCF function. For more information about the PCSCF, see 3GPP TS 23.002 [x].

5A.3.5.2	Attributes

The PCSCFFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| commModelList | M | T | T | F | T |

5A.3.5.3	Attribute constraints

None.

5A.3.5.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

5A.3.6	HSSFunction

5A.3.6.1	Definition

This IOC represents the HSS function. For more information about the HSS, see 3GPP TS 23.002 [x].

5A.3.6.2	Attributes

The HSSFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNIdList | M | T | T | F | T |
| commModelList | M | T | T | F | T |

5A.3.6.3	Attribute constraints

None.

5A.3.6.4	Notifications

The common notifications defined in subclause 5.5 are valid for this IOC, without exceptions or additions.

End of changes