# 3GPP TSG SA WG5 Meeting #141e	S5-221384

# Online, , 15 Nov 2021- 24 Nov 2021

# Source:	Samsung

# Title:	pCR 28.312 ServiceDeploymentExpectation definition

# Document for:	Approval

Agenda Item:	6.4.10

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

None

# 3	Rationale

This contribution provides the concrete ServiceDeploymentExpectation definition.

# 4	Detailed proposal

1st modified section

#### 6.2.1.2.x	ServiceDeployment Expectation

#### 6.2.1.2.x.1 	Definition

This class defines the expectation for a service to be deployed. This can be used by the consumer to request for a service deployment. The MOI of this class is to be created to request for a service deployment using intent.

The attribute serviceTargets will provide the requirements for the service. Service requirements will be translate into resource requirements for underlying Network Slice Instance/Network Slice Subnet Instances (NSI/NSSI) to satisfy the service requirements.

The attribute serviceAvaliability and edgeIdentification contributes to the selection of a particular edge data network where the service is to be deployed.

Editors Note: The definition of Edge in terms of 3GPP NRM is FFS. The reference to relevant definition should be made when available.

#### 6.2.1.2.x.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceTargets | M | T | T | T | T |
| serviceAvaliability | O | T | T | T | T |
| deploymentLocation | CM | T | T | T | T |

#### 6.2.1.2.x.3	Attribute constraints

| Name | Definition |
| --- | --- |
| edgeIdenfitication Support Qualifier | Condition: It shall be present when the expectation is to a deploy the service at the edge of the network. |

#### 6.2.1.2.x.4	Notifications

TBD

#### 6.2.1.3.x	ServiceAva <<dataType>>

#### 6.2.1.3.x.1 	Definition

This data type defines the time deration when the service will be available.

#### 6.2.1.3.x.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| startTime | M | T | F | T | T |
| endTime | M | T | F | T | T |

#### 6.2.1.3.x.3	Attribute constraints

None

#### 6.2.1.3.x.4	Notifications

TBD

#### 6.2.1.3.x	EdgeID <<dataType>>

#### 6.2.1.3.x.1 	Definition

This data type describes the identification of the edge network.

#### 6.2.1.3.x.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| edgeID | CM | T | F | T | T |
| geoLocation | CM | T | F | T | T |

#### 6.2.1.3.x.3	Attribute constraints

| Name | Definition |
| --- | --- |
| geoLocation Support Qualifier | Condition: If the service is to be deployed on a specific edge. |
| edgeID Support Qualifier | Condition: If the service is to be deployed on a specific edge. |

#### 6.2.1.3.x.4	Notifications

TBD

#### 6.2.1.4	Attribute definition

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| userLabel | A user-friendly (and user assignable) name of the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentExpectation | It indicates the expectations including requirements, goals and constraints given to a 3GPP system. The intentExpectation includes the intentAreas which describes a list of coverage areas for the intent to be applied. Editor’s Note: the detailed intentExpectation is for further discussion | type: FFS multiplicity: * isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentFulfilStatus | It describes the status of the intent fulfilment result, which is configured by MnS producer and can be read by MnS consumer. allowedValues: "FULFILLED", “NOT_FULFILLED Editor’s Note: whether other allowed values should be supported is FFS, and the name for the attribute intentFulfilStatus is FFS. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceTargets | This provides various service requirements in form of ServiceProfile[x]. Service requirements will be translate into resource requirements for underlying Network Slice Instance/Network Slice Subnet Instances (NSI/NSSI) to satisfy the service requirements. | type: ServiceProfile multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| serviceAvaliability | This describes the time window during which the service shall be available. This contributes to the selection of the appropate edge data network to be used for service deployment. | type: ServiceAva multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| deploymentLocation | This identifies the edge network where the service need to be deployed. | type: EdgeID multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| startTime | This describes the beginning time of the service availability time window. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| endTime | This describes the ending time of the service availability time window. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| edgeID | This identifies the edge network. This will be a DNN, see TS 23.501. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| geoLocation | This describes the geographical area at which the service should be deployed and made available. This will take a form of either single latitude & longitude or a TAI | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of 1st modified section