3GPP TSG-SA5 Meeting #142-e		S5-222459rev1

- e-meeting, 4 - 12 April 2022

- Source:	Huawei

- Title:	pCR 28.104 Add MDA capability for handover optimization

- Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104 Management and orchestration; Management Data Analytics (MDA)

# 3	Rationale

Current handover procedures are mainly based on radio conditions for selecting the target gNB. Considering the resource capabilities of target gNB and service experience of UE, the selected target gNB may not be the optimal gNB. To address this handover optimization issue, TR 28.809 [1] and TS 28.104 [2] describe the handover optimization use case.

TS 28.104 [2] has defined the use case and requirement of handover optimization in chapter 7.2.5.2. This contribution is proposed to add MDA capability for handover optimization in draft TS 28.104 [2].

# 4	Detailed proposal

It is proposed to add the following chapter in TS 28.104 [2].

1st modified section

## 8.4.5	MDA assisted mobility management

#### 8.4.5.2		Handover optimization

##### 8.4.5.2.1	MDA type

The MDA type for is: MobilityManagement.HandoverOptimization.

##### 8.4.5.2.2	Enabling data

The enabling data for handover optimization analysis are provided in table 8.4.5.2.2-1.

Table 8.4.1.1.2-1: Enabling data for handover optimization analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurement | PDCP Data Volume of NR cells | Clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [4]. |
|  | Virtual resource usage of each neighbour cell, including compute, memory and storage. | VR usage of NF (clause 5.7.1 of TS 28.552 [4]. |
|  | Radio resource utilization of each neighbour cell | Radio resource utilization (clause 5.1.1.2 of TS 28.552 [4]. |
| MDT Data | UE measurements related to RSRP, RSRQ, SINR (serving cell and neighbour cells) and UE location information | RSRPs, RSRQs and UE location of M1 measurements for NR in TS 32.422 [6] and TS 32.423 [7]. |

##### 8.4.5.2.3	Analytics output

The specific information elements of the analytics output (MDA report) for handover optimization, in addition to the common information elements of the MDA reports (see clause 8.3), are provided in table 8.4.5.2.3-1.

Table 8.x.1.1-1: Analytics output for handover optimization

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| TargetCellList |  |  |  |
| > CellID | The identifier of the target cell. | M | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| > Virtual Resource ConsumptionStatistic | The indication of statistics virtual resource consumption. | M | type: VirResConsumption multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| > PredictedVirtual Resource Consumption | The indication of future predictions of virtual resource consumption. | M | type: VirResConsumption multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| >Radio Resource ConsumptionStatistic | The indication of current statistics radio resource consumption. | M | type: RadResConsumption multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| > PredictedRadio Resource Consumption | The indication of future predictions of radio resource consumption. | M | type: RadResConsumption multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| > PredictedQoE | The predicted QoE value of UE (e.g., PDCP Data Volume, MOS value, etc) in the target cell identified by cell ID. | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

2nd modified section

## 8.5	Data type definitions

### 8.5.1	VirResConsumption <<datatype>>

#### 8.5.1.1	Definition

This data type specifies the type of ResourceConsumption in the analytics output.

#### 8.5.1.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| StartTime | It indicates the start time that are used for prediction or statistic. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EndTime | It indicates the end time that are used for prediction or statistic. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| virtualCPUUsage | It indicates the predictions or statistics of the virtual CPU consumption in a certain time period. The value indicates the average percentage of the virtual CPU consumption. | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| virtualMemoryUsage | It indicates the predictions or statistics of the virtual memory consumption in a certain time period. The value indicates the average percentage of the virtual memory consumption. | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| virtualDiskUsage | It indicates the predictions or statistics of the virtual storage consumption in a certain time period. The value indicates the average percentage of the virtual storage consumption. | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |

### 8.5.2	RadResConsumption <<datatype>>

#### 8.5.2.1	Definition

This data type specifies the type of RadResConsumption in the analytics output.

#### 8.5.2.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| StartTime | It indicates the start time that are used for prediction or statistic. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EndTime | It indicates the end time that are used for prediction or statistic. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| DLPRBUsage | It indicates the predictions or statistics of average usage (in percentage) of physical resource blocks (PRBs) on the downlink in a certain time period. | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| ULPRBUsage | It indicates the predictions or statistics of average usage (in percentage) of physical resource blocks (PRBs) on the uplink in a certain time period. | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |

End of modified section