3GPP TSG-SA5 Meeting #142-e 	S5-222569d1

e-meeting, 04 - 12 April 2022

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Add potential requirements for intent conflict information

- Document for:	Approval

- Agenda Item:	6.5.3

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP draft TR28.912: “Study on enhanced intent driven management services for mobile networks”.

# 3	Rationale

It is possible that different intents (or expectations thereof) as submitted to the intent driven management system contain conflicts and contradictions. The intent driven management system should be able to identify such conflicts and contradictions as accordingly inform the consumers as needed.

This contribution proposes to add the requirements for such notifications

# 4	Detailed proposal

It proposes to make the following changes  28.912[1]

1st Change

### 4.n Key Issue#4.n:	Intent conflicts and intents containing contradictions

#### 4.n.1	Description

The MnS consumer may create an intent containing two or more intent expectations and/or targets. For example, a Radio Network related intent may express an intent with targets on radio network parameters (like, downlink transmit power, remote electrical tilt) or on KPIs (like, DL UE throughput, average RSRP, coverage area). On receiving and after analysing the intent, the MnS producer may realize that the expectations or targets are not aligned, i.e. the MnS producer may detect conflicts or contradictions in the intent.

A intent is considered to have a conflict if the requirements stated in one expectation is not consistent (i.e. conflicts) with the requirements stated in another expectation in the same or different intent. On the other hand, an intent is considered to have a contradiction if the targets stated in one intent expectation is not consistent (i.e. conflicts) with another target in the same intent expectation.

For example, consider two targets target_1=: throughput > threshold_1 and target_2 :=  interference <  threshold_2, and while trying to achieve target_1, target_2 gets degraded, so the producer will see that the targets are conflicting. The two are conflicts if they are in different expectations but they are a contradiction if they are in the same expectation.

When such conflicts or contradictions are detected, the MnS producer needs to notify the MnS consumer about the conflict or contradiction, specifying the expectations and targets which give rise to the conflict or contradiction. Additionally, the MnS producer may also notify the MnS consumer about the parameters and/or KPIs which are in conflict or contradiction with one another.

Thereby, the MnS consumer may modify or delete the intent.

#### 4.n.2	Potential Requirements

REQ-Intent_confl: The intent driven MnS shall have the capability to inform the MnS consumer about intent expectations or targets which may give rise to conflicts or contradictions.

End of changes