| 3GPP TR 28.912 V0.1.0 (2022-04) |  |
| --- | --- |
| Technica Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Study on enhanced intent driven management services for mobile networks (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Issue investigations and potential solutions for new areas	7

4.1 Key Issue#4.1: intent driven approach for RAN energy saving	7

4.1.1 Description	7

4.1.2 Potential solutions	7

4.2 Key Issue#4.2:	Intent conflicts and intents containing contradictions	7

4.2.1	Description	7

4.2.2	Potential Solution	8

5	Issue investigations and potential solutions for the requirements documented in TS 28.100	8

5.X Key Issue# Num: title	8

5.X.1 Description	8

6	Issue investigations and potential solutions for collaboration/alignment with other SDOs	8

6.X Key Issue# Num: title	8

6.X.1 Description	8

7 Conclusion and recommendation	9

Annex <X> (informative): Change history	10

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document investigates new areas for enhancing the intent driven management services of 5G network. It identifies and documents key issues and evaluates potential solutions, and provides recommendations for the normative work.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.312: "Management and orchestration; Intent driven management services for mobile networks"

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Issue investigations and potential solutions for new areas

Editor's note: this clause will contain issues and potential solutions related to the new scenario, requirement and generic capability for intent driven management.

4.1 Key Issue#4.1: intent driven approach for RAN energy saving

4.1.1 Description

Operators are aiming at decreasing power consumption in 5G networks to lower their operational expense with energy saving management solutions. Energy saving is achieved by executing the energy saving actions with suitable parameter configurations, e.g. energy saving state switch, start time and end time, the energy saving thresholds. However, the variety of combinations of energy saving actions lead to difficulties for the operators to derive energy saving actions, for example, different energy saving actions may be conflicted with each other, or the energy saving actions may conflict with other activities (e.g. network optimization actions). Moreover, it is not straightforward to evaluate the influence on service experience (e.g. UL/DL RAN UE throughput, latency) of energy saving actions beforehand, which makes it difficult to balance the energy saving effect and service experience, for example the energy saving actions may deteriorate the service experience.

As TS 28.312 [2] described, an intent focuses more on describing the "What" needs to be achieved but less on "How" that outcomes should be achieved, which not only relieves the burden of the consumer knowing implementation details but also leaves room to allow the producer to explore alternative options and find optimal solutions. So introducing the intent approach for energy saving, which can enable the 3gpp management system to analyze and select the optimal balance between the energy saving effect and service experience by utilizing some intelligence mechanisms. In intent driven approach, a MnS consumer expresses intent expectation for RAN energy saving in the specified area to a MnS producer, which may include the RAN energy saving target (e.g. target RAN energy consumption) and service experience (e.g. RAN UE throughput, latency), as well as the frequencies and RATs to be considered for energy saving. MnS producer analyzes and determines the optimal RAN energy saving solution (a set of energy saving actions) to satisfy MnS consumer's intent expectation for RAN energy saving.

It is important to investigate the model for intent expectation for RAN energy saving based on the generic intent model and radio network expectation defined in TS 28.312[2]

4.1.2 Potential solutions

TBD

4.2 Key Issue#4.2:	Intent conflicts and intents containing contradictions

4.2.1	Description

The MnS consumer may create an intent containing two or more intent expectations and/or targets. For example, a Radio Network related intent may express an intent with targets on radio network parameters (like, downlink transmit power, remote electrical tilt) or on KPIs (like, DL UE throughput, average RSRP, coverage area). On receiving and after analysing the intent, the MnS producer may realize that the expectations or targets are not aligned, i.e. the MnS producer may detect conflicts or contradictions in the intent.

A intent is considered to have a conflict if the requirements stated in one expectation is not consistent (i.e. conflicts) with the requirements stated in another expectation in the same or different intent. On the other hand, an intent is considered to have a contradiction if the targets stated in one intent expectation is not consistent (i.e. conflicts) with another target in the same intent expectation.

For example, consider two targets target_1=: throughput > threshold_1 and target_2 :=  interference <  threshold_2, and while trying to achieve target_1, target_2 gets degraded, so the producer will see that the targets are conflicting. The two are conflicts if they are in different expectations but they are a contradiction if they are in the same expectation.

When such conflicts or contradictions are detected, the MnS producer needs to notify the MnS consumer about the conflict or contradiction, specifying the expectations and targets which give rise to the conflict or contradiction. Additionally, the MnS producer may also notify the MnS consumer about the parameters and/or KPIs which are in conflict or contradiction with one another.

Thereby, the MnS consumer may modify or delete the intent.

REQ-Intent_conflict: The intent driven MnS shall have the capability to inform the MnS consumer about intent expectations or targets which may give rise to conflicts or contradictions.

4.2.2	Potential Solution

The MnS producer should notify the MnS consumer whenever such a conflict is detected. Additionally, the intent progress status should be marked as terminated with the reason as conflict detected.

5	Issue investigations and potential solutions for the requirements documented in TS 28.100

Editor's note: this clause will contain issues and potential solutions related to the solution to support Rel-17 leftover requirements

5.X Key Issue# Num: title

5.X.1 Description

5.X.2 Potential solutions

6	Issue investigations and potential solutions for collaboration/alignment with other SDOs

Editor's note: this clause will contain issues and potential solutions related to collaboration/alignment for intent driven management with other SDOs

6.X Key Issue# Num: title

6.X.1 Description

6.X.2 Potential solutions

7 Conclusion and recommendation

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-04 | SA5#142e | S5-222244 |  |  |  | TR 28.912_000 | 0.0.0 |
| 2022-04 | SA5#142e | S5-222255 S5-222256 S5-222620 S5-222569 |  |  |  | 1.pCR TR 28.912 Add scope 2.pCR TR 28.912 Add structure 3.pCR TR 28.912 Add key issue for intent driven approach for RAN energy saving 4.pCR TR 28.912 Add potential requirements for intent conflict information | 0.1.0 |