3GPP TSG-SA5 Meeting #142-e 	S5-222272

# e-meeting, 4 - 12 April 2022

# Source:	Huawei

# Title:	Key Issue on Management of NPN-SC

# Document for:	Approval

Agenda Item:	6.5.17

1	Decision/action requested

Discuss and approve on the proposal.

2	References

[1]	TR 28.907 Study on enhancement of management of non-public networks v0.0.0

3	Rationale

One of objectives of this FS_OAM_eNPN study item is study management of vertical as an authorized NPN service customer (SP-211436). This document proposes to add a Key Issue on the management of NPN service customer in draft TR 28.907 [1].

4	Detailed proposal

This document proposes the following changes in TR 28.907 [1].

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 28.557 "Management and orchestration; Management of non-public networks; Stage 1 and stage 2"

2nd Change

5.X	Key Issue #X: Management of NPN service customer

5.X.1	Description

As description in TS 28.557 [x], an NPN service customer is used to represent the role of communication service customer in NPN environment. NPN service customer can request and consume the management capabilities exposed by the mobile network operator. The mobile network operator would restrict the types (e.g. provisioning, fault supervision, performance assurance) of management capabilities and corresponding managed network resource exposed to an NPN service customer. The management of NPN service customers is not considered in TS 28.557 [x], including management of authorized capability of utilizing management services and management data. On the other hand, in some vertical industries, NPN service customer may be authorized to manage the terminal members of their own private network, e.g. monitoring smart distribution transformer terminal in smart grid.

3GPP management system may need to support the management of NPN service customer including,

- Maintaining of the related information for NPN service customer, including identity of NPN service customer, available management capabilities and other attributes (e.g. service area).

- Member management of NPN service customer, e.g. state monitoring of terminal members, group management of terminal members.

End of change