3GPP TSG-SA5 Meeting #142-e 	S5- 222343

# e-meeting, 4 - 12 April 2022

# Source:	China Unicom

# Title:	Add key issue management requirement for different POP’s network operation

# Document for:	Approval

Agenda Item:	6.5.13

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP TR 28.835 v0.0.0: “Management Aspects of 5G MOCN Network Sharing Phase2”

3	Rationale

It was approved to study the enhancement of management aspects for 5G MOCN Network Sharing [1]. One of the objectives of this new study item is studying the requirements of the interface between POP and MOP, and impact on interaction between MOP and NEP. For supporting the POP’s network operation, POP-NM needs to get different data (e.g. mobility management measurements focused by POP A, packet delay measurements focused by POP B) from MOP-NM.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

4.X	Issue #X:  Management requirement for different POP’s network operation

4.X.1	Description

For MOCN Networking Sharing, different POP needs different data for personalized operation and maintenance services (e.g. mobility management measurements focused by POP A, packet delay measurements focused by POP B). POP-NM needs to get different data from MOP-NM for POPs’ network operation.

Figure 4.X-1: Management architecture for MOCN

4.X.2	Potential solution

1. According to different POPs’ operation requirements, the 3GPP management system of the MOP shall have capability to report different configuration and performance measurements (e.g. Mobility management measurements focused by POP A, packet delay measurements focused by POP B) for each POP.

2. Ensuring alarm continuity, the 3GPP management system of the MOP shall have capability to provide full fault measurements for each POP.

End of Change