3GPP TSG SA WG5 Meeting 142-e		S5-222494

# e-meeting, 4 -12 April 2022

# Source:	Nokia

# Title:	Enhancing MDA type related to MDA capabilities

# Document for:	Approval

Agenda Item:	6.6.5

# 1	Decision/action requested

This contribution is for approval.

# 2	References

[x]	TR 28.809 Study on enhancement of management data analytics

# 3	Rationale

This contribution introduces a proposal to enhance the MDA type to enable the MDA MnS consumer to request optionally specific information elements out of a set of MDA output data related to a specific MDA capability, i.e., use cases defined in TR 28.809 [x].

# 4	Detailed proposal

1st Modified Section

## 8.4	Data definitions per MDA capability

### 8.4.1	Coverage related analytics

#### 8.4.1.1		Coverage problem analysis

##### 8.4.1.1.1	MDA type

The MDA type for coverage problem analysis is: CoverageAnalytics.CoverageProblemAnalysis. or optionally CoverageAnalytics.CoverageProblemAnalysis.InformationElementName

##### 8.4.1.1.2	Enabling data

The enabling data for coverage problem analysis are provided in table 8.4.1.1.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.1.1.2-1: Enabling data for coverage problem analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | SS-RSRP distribution per SSB (beam) of serving NR cell | SS-RSRP distribution per SSB (clause 5.1.1.22.1 of TS 28.552 [4]). |
|  | SS-RSRP distribution per SSB (beam) of neighbor NR cell | Editor’s note: to be defined in TS 28.552 |
|  | RSRP distribution of neighbor E-UTRA cell for an NR cell | Editor’s note: to be defined in TS 28.552 |
|  | Power headroom distribution for NR cell | Type 1 power headroom distribution (clause 5.1.1.26.1 of TS 28.552 [4]). |
|  | Wideband CQI distribution for NR cell | Wideband CQI distribution (clause 5.1.1.11.1 of TS 28.552 [4]). |
|  | Timing Advance distribution for NR cell | Editor’s note: to be defined in TS 28.552 |
|  | Number of UE Context Release Request (gNB-DU initiated) | Number of UE Context Release Request (gNB-DU initiated) (clause 5.1.3.5.1 of TS 28.552 [4]). |
|  | Number of UE Context Release Request per SSB (gNB-DU initiated) | Editor’s note: to be defined in TS 28.552 |
|  | Number of UE Context Release Requests (gNB-CU initiated) | Number of UE Context Release Request (gNB-CU initiated) (clause 5.1.3.5.2 of TS 28.552 [4]). |
|  | Number of UE Context Release Requests per SSB (gNB-CU initiated) | Editor’s note: to be defined in TS 28.552 |
|  | RSRP related measurements for ng-eNB | RSRP related measurements (clause 6.1 of TS 32.425 [12]). |
|  | UE power headroom related measurements for ng-eNB | UE power headroom related measurements (clause 6.3 of TS 32.425 [12]). |
|  | Wideband CQI distribution for ng-eNB | Wideband CQI distribution (clause 4.10.1.1 of TS 32.425 [12]). |
|  | Average sub-band CQI for ng-eNB | Average sub-band CQI (clause 4.10.1.2 of TS 32.425 [12]). |
|  | UE Rx – Tx time difference related measurements for ng-eNB | UE Rx - Tx time difference related measurements (clause 6.4 of TS 32.425 [12]). |
|  | AOA related measurements for ng-eNB | AOA related measurements (clause 6.5 of TS 32.425 [12]). |
|  | Timing Advance distribution for ng-eNB | Timing Advance Distribution (clause 4.10.2 of TS 32.425 [12]). |
|  | Number of UE CONTEXT Release Request initiated by ng-eNodeB | Number of UE CONTEXT Release Request initiated by eNodeB/RN (clause 4.1.5.1 of TS 32.425 [12]). |
| MDT reports | MDT reports containing RSRPs of the serving cell and neighbour cells, and UE location. | RSRPs and UE location of M1 measurements for NR in TS 32.422 [6] and TS 32.423 [7]. |
| RLF reports | RLF reports containing RSRPs of the last serving cell and neighbour cells, and UE location. | RLF data collection and RLF reporting in TS 32.422 [6], and rlf-Report-r16 in TS 38.331 [13]. |
| RCEF reports | RCEF reports containing RSRPs of NR cell where the RRC connection establishment failed and neighbour cells, and UE location. | RCEF data collection and RCEF reporting in TS 32.422 [6], and ConnEstFailReport-r16 in TS 38.331 [13]. |
| UE location reports | UE location information provided by the LMF services which can be used to correlate with the MDT reports. | The UE location information provided by LMF via service-based interface (see TS 23.273 [14]). |
| Geographical data | The geographical information (longitude, latitude, altitude) of the deployed RAN (NG-RAN and E-UTRAN). | Editor’s note: to be defined in TS 28.622/623 or 28.541. |
| Configuration data | The NRMs containing the attributes affecting the coverage for (NG-RAN and E-UTRAN). | NRCellDU IOC, NRSectorCarrier IOC, BWP IOC, CommonBeamformingFunction IOC, and Beam IOC in TS 28.541 [15]; EUtranGenericCell IOC in TS 28.658 [16]; SectorEquipmentFunction IOC, AntennaFunction IOC, and TMAFunction IOC in TS 28.662 [17]. |

##### 8.4.1.1.3	Analytics output

The specific information elements of the analytics output for coverage problem analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.1.1.3-1.

Table 8.4.1.1.3-1: Analytics output for coverage problem analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| CoverageProblemId | The identifier of the coverage problem. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CoverageProblemType | Indication of type of the coverage Problem. The allowed value is one of the enumerated values: WeakCoverage, CoverageHole, PilotPollution, Overshoot coverage, DlUlChannelCoverageMismatch, Other, Unknown. | M | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CoverageProblemAreas | Geographical location areas where the coverage problem occurred. | O | type: GeoArea (see TS 28.622, to be confirmed) multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ProblematicCells | The CGIs of cells where the coverage problem occurred. | M | type: Integer multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ProblematicRAT | Indication of the RAT(s) where the coverage problem occurred. The allowed value is one of the enumerated values: NR, E-UTRA, Both. | M | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| RecommendedActions | The recommended actions to solve the coverage problem. The recommended action may be (but not limited to): - creation of new beam(s), or cell(s); - change the transmission power of the NR sector carrier; - delete some unwanted beam(s) or cell(s). | M | type: RecommendedAction multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

### 8.4.2	SLS analysis

#### 8.4.2.1	Service experience analysis

##### 8.4.2.1.1	MDA type

The MDA type for Capability-Service experience analysis is: SLSAnalysis.ServiceExperienceAnalysis or optionally SLSAnalysis.ServiceExperienceAnalysis.InformationElementName

##### 8.4.2.1.2	Enabling data

The enabling data for service experience analysis are provided in table 8.4.2.1.2-1.

Table 8.4.2.1.2-1: Enabling data for service experience analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | Average e2e uplink/downlink delay for a network slice | Average e2e uplink/downlink delay for a network slice (in 6.3.1.8 in TS 28.554 [5]); |
|  | Integrated uplink/downlink delay in RAN | Integrated downlink delay in RAN (6.3.1.2 in TS 28.554 [5]); Integrated uplink delay in RAN (6.3.1.7 in TS 28.554 [5]); |
|  | Round-trip packet delay | Round-trip packet delay between PSA UPF and NG-RAN (5.4.8 in TS 28.552 [4]) |
|  | UL/DL throughput for network and Network Slice Instance | Upstream throughput for network and Network Slice Instance (6.3.3 in TS28.554 [5]); Downstream throughput for Single Network Slice Instance (6.3.4 in TS28.554 [5]) |
|  | RAN UE Throughput | RAN UE Throughput (6.3.6 in TS28.554 [5]) |
|  | Throughput at N3 interface | Upstream Throughput at N3 interface (6.3.4 in TS28.554 [5]); Downstream Throughput at N3 interface (6.3.5 in TS28.554 [5]); |
| QoE Data | The QoE data of the different services | QoE data (TS 26.247 [22] and TS 26.114 [23] can be acquired through the procedures defined in TS 28.405 [8]). |

##### 8.4.2.1.3	Analytics output

The specific information elements of the analytics output for service experience analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.2.1.3-1.

Table 8.4.2.1.3-1:  Analytics output for Service experience analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| ServiceExperienceId | The identifier indicates the analytics report is related with service experience analysis. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ServiceExperienceIssueType | Indication of the service experience issue type. The allowed value is one of the enumerated values: RAN issue, CN issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AffectedObjects | The managed object instances where the service experience is applicable, e.g., SubNetwork Instance, NetworkSlice Instance, S-NSSAI. | O | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ServiceExperienceStatistics | The statistics of the level of service experience for a service in a certain time period, e.g. there are five levels which are represented by 1, 2, 3, 4, 5 where level 1 represents the users are enduring bad experience while level 5 represents the users’ requirements are perfectly satisfied. | O | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ServiceExperiencePredictions | The predictions of the level of service experience for a service in a certain time period. | O | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

#### 8.4.2.2	Network slice throughput analysis

##### 8.4.2.2.1	MDA type

The MDA type for Capability-Network slice throughput analysis is: SLSAnalysis.NetworkSliceThroughputAnalysis

##### 8.4.2.2.2	Enabling data

The enabling data for network slice throughput analysis are provided in table 8.4.2.2.2-1.

Table 8.4.2.2.2-1: Enabling data for network slice throughput analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | UL/DL throughput for network and Network Slice Instance | Upstream throughput for network and Network Slice Instance as defined in 6.3.3 in TS28.554 [5]; Downstream throughput for Single Network Slice Instance as defined in 6.3.4 in TS28.554 [5] |
|  | RAN UE Throughput | RAN UE Throughput as defined in 6.3.6 in TS28.554 [5] |
|  | Throughput at N3 interface | Upstream Throughput at N3 interface as defined in 6.3.4 in TS28.554 [5]; Downstream Throughput at N3 interface as defined in 6.3.5 in TS28.554 [5]; |

##### 8.4.2.2.3	Analytics output

The specific information elements of the analytics output for network slice throughput analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.2.2.3-1.

Table 8.4.2.2.3-1:  Network slice throughput analysis output

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| NetworkSliceThroughputIssueId | Network slice throughput issue identifier | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceThroughputIssueType | Indication of the network slice throughput issue type The allowed value is one of the enumerated values: RAN issue, CN issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceThroughputUserStatistics | The statistics of the UL and/or DL network slice throughput in a certain time period. The value indicates the average percentage of users, for which the required SLS throughput satisfies | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceThroughputTimeStatistics | The statistics of the UL and/or DL network slice throughput in a certain time period. The value indicates the average percentage of time, during which the required SLS throughput satisfies | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceThroughputUserPredictions | The predictions of the UL and/or DL network slice throughput in a certain time period. The value indicates the average percentage of users, for which the required SLS throughput could be met | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceThroughputTimePredictions | The predictions of the UL and/or DL network slice throughput in a certain time period. The value indicates the average percentage of time, during which the required SLS throughput could be met. | O | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

#### 8.4.2.3	TBD

#### 8.4.2.4	E2E latency analysis

##### 8.4.2.4.1	MDA type

The MDA type for Capability-E2E latency analysis is: SLSAnalysis.E2ElatencyAnalysis or optionally SLSAnalysis.E2ElatencyAnalysis.InformationElementName

.

##### 8.4.2.4.2	Enabling data

The enabling data for E2E latency analysis are provided in table 8.4.2.4.2-1.

Table 8.4.2.4.2-1: Enabling data for E2E latency analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | Average e2e UL/DL delay for a network slice | Average e2e uplink delay for a network (6.3.1.8.1 in TS 28.554 [5]); Average e2e downlink delay for a network slice (6.3.1.8.2 in TS 28.554 [5]). |
|  | Integrated uplink/downlink delay in RAN | Integrated downlink delay in RAN (6.3.1.2 in TS 28.554 [5]); Integrated uplink delay in RAN (6.3.1.7 in TS 28.554 [5]); |
|  | Round-trip Packet Delay | Round-trip packet delay between PSA UPF and NG-RAN (5.4.8 TS 28.552 [4]) |

##### 8.4.2.4.3	Analytics output

The specific information elements of the analytics output for E2E latency analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.2.4.3-1.

Table 8.4.2.4.3-1:  Analytics output for E2E latency analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| E2ELatencyIssueId | The identifier indicates the output is for E2E latency issue analysis | M | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| E2ELatencyIssueType | Indication the type of the E2E latency issue. The allowed value is one of the enumerated values: RAN latency issue, CN latency issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AffectedObjects | The managed object instances of subnetwork, managed elements or network slices where the latency issue happens | O | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

#### 8.4.2.5	Network slice load analysis

##### 8.4.2.5.1	MDA type

The MDA type for Capability- Network slice load analysis is: SLSAnalysis.NetworkSliceLoadAnalysis or optionally SLSAnalysis.NetworkSliceLoadAnalysis.InformationElementName.

##### 8.4.2.5.2	Enabling data

The enabling data for network slice load analysis are provided in table 8.4.2.5.2-1.

Table 8.4.2.5.2-1: Enabling data for network slice load analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | Number of PDU sessions of network slice | Mean number of PDU sessions of network and network Slice Instance (6.4.1 in TS28.554 [5]) |
|  | Number of PDU Sessions successfully setup | Number of PDU Sessions successfully setup (5.1.1.5 in TS28.552 [4]) |
|  | Mean Number of PDU sessions | Number of PDU sessions(Mean) (5.3.1.1 in TS28.554 [5]) |

##### 8.4.2.5.3	Analytics output

The specific information elements of the analytics output for network slice load analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.2.5.3-1.

Table 8.4.2.5.3-1:  Analytics output for network slice load analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| NetworkSliceLoadIssueId | The identifier indicates the output is for Network slice instance load analysis | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceLoadIssueDomain | Indicates the domain of the network slice instance load issue The allowed value is one of the enumerated values: RAN issue, CN issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceLoadIssuePhase | Indicates the phase of the network slice instance load issue The allowed value is one of the enumerated values: historic network slice load issue, ongoing network slice load issue, potential network slice load issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceLoadIssueType | Indicates the type of the network slice instance load issue The allowed value is one of the enumerated values: overload network slice load issue, underutilized network slice load issue | M | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AffectedObjects | The managed object instances involved in the network slice instance load problem | O | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NetworkSliceLoadDistribution | Describes the detailed load distribution or predictive distribution, e.g. load distribution for a network slice instance at a certain location or in a certain time period. | O | type: list multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

### 8.4.3	TBD

### 8.4.4	MDA assisted energy saving

#### 8.4.4.1	MDA type

The MDA type for energy saving analysis is: MDAAssistedEnergySaving.EnergySavingAnalysis or optionally MDAAssistedEnergySaving.EnergySavingAnalysis.InformationElementName.

.

#### 8.4.4.2	Enabling data

The enabling data for coverage problem analysis are provided in table 8.4.4.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.4.2-1: Enabling data for energy saving analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | PNF Power Consumption: power consumed over the measurement period | Clause 5.1.1.19.2 of TS 28.552 [4] |
|  | PNF Energy consumption: energy consumed | Clause 5.1.1.19.3 of TS 28.552 [4] |
|  | SS-RSRP distribution per SSB (beam) of serving NR cell | Clause 5.1.1.22.1 of TS 28.552 [4]. |
|  | SS-RSRP distribution per SSB (beam) of neighbor NR cell | Clause 5.1.1.22.1 of TS 28.552 [4]. |
|  | PDCP Data Volume of NR cells: PDCP data volume delivered in the downlink and uplink; | Clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [4] |
|  | Traffic load variation: PRB utilization rate, RRC connection number, etc. | Clause 5.1.1.2 and 5.1.1.4 of TS 28.552 [4] |
|  | UE throughput: UE throughput in downlink and uplink | Clause 5.1.1.3 of TS 28.552 [4] |
|  | Delay related measurements of UPF | Clause 5.4 of TS 28.552 [4] |
|  | Data volume of UPF | Clause 5.4 of TS 28.552 [4] |
|  | Virtual resource usage of NF: The virtual CPU usage, virtual memory usage, virtual disk usage of virtual network functions; | Clause 5.7.1 of TS 28.552 [4] |
| MDT reports | The RSRPs of UE measurements. | RSRPs of M1 measurements in TS 32.422 [6] and TS 32.423 [7]. |
|  | The RSRQs of UE measurements. | RSRQs of M1 measurements in TS 32.422 [6] and TS 32.423 [7]. |
|  | The UE location information. | UE location of M1 measurements in TS 32.422 [6] and TS 32.423 [7]. |
| QoE Data | The measurements that are collected are DASH and MTSI measurements. | TS 28.406 [9] |
| Configuration data | MOIs of the cells, UPFs and SMFs. | TS 28.541 [15] |
| Network analytics data | The control plane analysis result from the NWDAF, e.g., observed service experience related network data analytics. | TS 23.288 [10] |

#### 8.4.4.3	Analytics output

The specific information elements of the analytics output for energy saving analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.4.3-1.

Table 8.4.4.3-1: Analytics output for energy saving analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| EnergyEfficiencyProblematicObject | Indication of NR cells or NFs where the energy efficiency issues occurred or potentially occur. | M | type: DN multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EnergyEfficiencyProblemType | Indication of type of the energy efficiency issues. The allowed value is one of the enumerated values: HighEnergyConsumption, LowEenergyEfficiency, Other, Unknown. | M | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TrafficLoadTrends | The predictions of the trends of traffic load in a certain time period. The predictions include the traffic load of the issue cell(s) and neighboring cell(s). | M | type:TrafficLoadTrend multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EnergySavingRecommendations | The recommendation shall contain the energy saving policy. For ES on NR cells. It may contain a set of - recommended NR Cell (ES-Cell) to enter energySaving state. - recommended candidate cells with precedence for taking over the traffic of the ES-Cell. - the time to enter and terminate the energy saving state. - the load threshold to enter and terminate the energy saving state for the ES-Cell For ES on UPFs. It contains a set of - recommended UPF (ES-UPF) to conduct energy saving; - recommended candidate UPFs with precedence for taking over the traffic of the ES-UPF. - the time to conduct energy saving for the ES-UPF | M | type: EsRecommendation multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| StatisticsOfCellsEsState | The statistic result of current energy saving state of the cells at a certain time, which can be used by consumers to make analysis (e.g., observed service experience analysis made by NWDAF) or to make decision (e.g., enter/exit the energy saving state based on the current energy saving state). | O | type: StatisticOfCellEsState multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End ofModified Section