| 3GPP TR 28.832 V0.1.0 (2022-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Study on management aspects of URLLC (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Concepts and overview	5

5	Key Issues Investigation and Potential Solutions	6

5.1	Key Issue #1: Classification of URLLC related RAN features from management perspective	6

5.1.1	Description	6

5.1.2	Potential solutions	6

5.Y	Key Issue #Y: DEF	8

5.Y.1	Description	8

5.Y.2	Potential solutions	8

6	Conclusion and Recommendation	8

6.X	Issue #X	8

6.Y	Issue #Y	8

Annex <X> (informative): Change history	9

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document investigates the potential requirements related to management of the URLLC features defined in RAN side. It documents key issues related to configuration management of URLLC features and to the performance measurements for URLLC service. It provides potential solutions and recommendations for the normative work.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.104: “Service requirements for cyber-physical control applications in vertical domains; Stage 1”

[3]	3GPP TS 22.261: “Service requirements for the 5G system; Stage 1”

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Concepts and overview

URLLC, Ultra Reliable Low Latency Communication, is a typical service of 5G network, which is mainly applicable to the scenarios having high requirements on latency and reliability. The deployment and commercialization of URLLC service are of great significance to network operators. Efforts to build URLLC service oriented to vertical industries may become a potential growth point to increase revenue.

In order to satisfy the requirements of URLLC service, 3GPP RAN groups have defined many features to decrease latency and increase reliability to guarantee the SLA of URLLC service. However, only principle and implementation mode of each feature are defined in RAN specifications, and it is not indicated that how to manage the features to enable URLLC service in actual network deployment. Consequently, the management of URLLC related features need to be studied to achieve the service requirements of URLLC defined in TS 22.261[3].

The characteristics of URLLC are mainly reflected in several aspects such as delay/latency, reliability and availability. The concepts and terms used in this document related to management of URLLC service are shown below:

| Term | Reference | Definition |
| --- | --- | --- |
| end-to-end latency | TS 22.261 [3] | The time that it takes to transfer a given piece of information from a source to a destination, measured at the communication interface, from the moment it is transmitted by the source to the moment it is successfully received at the destination. |
| reliability | TS 22.261 [3] | In the context of network layer packet transmissions, percentage value of the packets successfully delivered to a given system entity within the time constraint required by the targeted service out of all the packets transmitted. |
| survival time | TS 22.261 [3] | The time that an application consuming a communication service may continue without an anticipated message. |
| communication service availability | TS 22.261 [3] | Percentage value of the amount of time the end-to-end communication service is delivered according to a specified QoS, divided by the amount of time the system is expected to deliver the end-to-end service. |

The document studies on the management of URLLC related features defined in RAN groups, including sorting the features from latency, reliability and multiplexing aspects, investigating the requiremnets of configuration management and performance measurements in OAM.

5	Key Issues Investigation and Potential Solutions

5.1	Key Issue #1: Classification of URLLC related RAN features from management perspective

5.1.1	Description

URLLC is a set of service scenarios which require low-latency and high reliabile communications. Specific SLAs are defined for each scenario correspondingly. Different scenarios have different requirements, some of which focus on latency (e.g. Motion control) and some focus reliability (e.g. Discrete automation).

In order to satisfy the requirements of URLLC, many features have been defined by 3GPP RAN side to decrease latency and increase reliability to guarantee the SLAs of different URLLC service scenarios. These features with different functions and different effectives are distributed in different specifications, which bring complexity to the invocation and management when deploying URLLC service. Features related to URLLC need to be classified from the perspective of management, so that different features can be invoked according to different SLAs, and the management of URLLC-related features can be achieved.

5.1.2	Potential solutions

This document sorts out the features related to URLLC defined in RAN and classifies them from a management perspective. According to the characteristics of URLLC service, features are classified into the following two categories based on their effects : low latency and ultra reliability. Features belonging to low-latency category are mainly used to reduce data transmission delay, reliability features are mainly used to improve the reliability of transmission.Among the features belonging to low-latency category, some of them reduce the transmission latency from the effective mechanism and other features improve service priority to reduce URLLC latency in multi-service scenario.

| Feature | Category | Reference |
| --- | --- | --- |
| Mini-slot transmission | Low latency | TS 38.214 |
| Numerology/SCS | Low latency | TS 38.211 |
| UL configured grant | Low latency | TS 38.214 |
| DL SPS | Low latency | TS 38.213 |
| PDCCH monitoring | Low latency | TS 38.213 |
| Logical channel priority | Low latency | TS 38.321 |
| Short PUCCH | Low latency | TS 38.211 |
| UE processing capability#2 | Low latency | TS 38.214 |
| Span based PDCCH monitoring | Low latency | TS 38.212/TS 38.213 |
| UL configured grant enhancements | Low latency | TS 38.214 |
| DL SPS enhancements | Low latency | TS 38.213 |
| Sub-slot level HARQ-ACK (UCI enhancements) | Low latency | TS 38.213 |
| Two HAQR-ACK codebooks (UCI enhancements) | Low latency | TS 38.213 |
| Low SE MCS/CQI table | Ultra reliability | TS 38.214 |
| PDSCH repetitions | Ultra reliability | TS 38.214 |
| PUSCH repetitions | Ultra reliability | TS 38.214 |
| PUCCH repetitions | Ultra reliability | TS 38.213 |
| PDCCH aggregation level 16 | Ultra reliability | TS 38.213 |
| PDCP duplication | Ultra reliability | TS 38.323 |
| PUSCH repetitions enhancements | Ultra reliability | TS 38.214 |
| DCI format 0_2 and DCI format 1_2 | Ultra reliability | TS 38.213 |
| DL Multi-TRP for URLLC data channel repetitions | Ultra reliability | TS 38.213 |
| PDCP duplication enhancements | Ultra reliability | TS 38.323 |
| DL Preemption Indication (PI) | Low latency | TS 38.213 |
| Code Block Group (CBG) | Low latency | TS 38.214 |
| Inter UE: UL Cancellation Indication(CI) | Low latency | TS 38.213 |
| Inter UE: Power Boosting | Low latency | TS 38.213 |
| Intra UE: UL Prioritization | Low latency | TS 38.213 |

5.Y	Key Issue #Y: DEF

5.Y.1	Description

5.Y.2	Potential solutions

6	Conclusion and Recommendation

6.X	Issue #X

6.Y	Issue #Y

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-04 | SA5#142e | S5-222292 S5-222307 S5-222344 S5-222660 |  |  |  | 1. TR 28.832-0.0.0 initial skeleton 2. pCR 28.832 Add scope 3. pCR 28.832 Add Concepts and overview 4. pCR TR28.832 Add key issue on classification of URLLC related RAN features from management perspective | 0.1.0 |