| 3GPP TR 28.865 V0.1.0 (2022-04) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; Study on deterministic communication service assurance (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Overview and Concepts	7

4.1	Overview	7

4.2	Concepts	7

5	Key Issues and potential solutions	7

5.X	Key Issue #1: Provisioning of network functions related to deterministic communication service	7

5.X.1	Description	7

5.X.2	Potential solutions	8

5.X.2.a	Potential solution #<a>: <Potential Solution a Title>	8

5.X.2.a.1	Introduction	8

5.X.2.a.2	Description	8

5.X.3	Conclusion - Impact on normative work	8

5.Y	Key Issue #2: Service assurance for video monitoring	8

5.Y.1	Description	8

5.Y.2	Potential solutions	8

5.Y.2.a	Potential solution #<a>: <Potential Solution a Title>	8

5.Y.2.a.1	Introduction	8

5.Y.2.a.2	Description	8

5.Y.3	Conclusion - Impact on normative work	8

5.Z	Key Issue #3: Service assurance for PLC control	8

5.Z.1	Description	8

5.Z.2	Potential solutions	9

5.Z.2.a	Potential solution #<a>: <Potential Solution a Title>	9

5.Z.2.a.1	Introduction	9

5.Z.2.a.2	Description	9

5.Z.3	Conclusion - Impact on normative work	9

6	Conclusions and recommendations	9

Annex <X> (informative): Change history	9

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In drafting the TS/TR, pay particular attention to the use of modal auxiliary verbs! TRs shall not contain any normative provisions.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

TBD

1	Scope

The present document studies potential key issues and solutions for provisioning and assurance of deterministic communication services, e.g. management of related network functions, potential new service requirements and new performance measurements etc. The document provides conclusions and recommendations on the normative work.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 22.104: "Physical control applications in vertical domains".

[4]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

…

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Overview and Concepts

4.1	Overview

Editor's note: this clause will contain overview of requirements, related network functions and management aspects to support deterministic communication services.

5G network should satisfy diversified SLA requirements to support different vertical applications. Service requirements are more stringent for deterministic communication services as described in TS 22.261[2] and TS 22.104 [3], e.g., video monitoring for production environment in a factory, remote control etc. The latency/transmission time of a data package is bounded by a given threshold. There are two typical traffic classes or communication patterns for deterministic communication service: deterministic periodic communication and deterministic aperiodic communication. In clause 5 in TS 22.104 [3], performance requirements for some deterministic communication services are provided. In TS 23.501 [4], QoS characteristics are defined for some deterministic communication services, e.g. some 5QIs for delay critical GBR services. Communication service availability and reliability are more important for these services types. Service experience degradation or violation of the latency requirements such as unstable jitter or unexpected packet loss may result in service interruption or severe consequences. Stable and deterministic communication service experience should be assured.

There are some features in the 5G network to support deterministic communication services, e.g. URLLC related network functions for radio interface and 5GC network, 5GS Integration with TSN and Industrial IoT, high accuracy positioning etc. How to support deterministic communication services from management aspects are investigated in this present document, e.g. provisioning of the related network functions, solutions for the assurance of deterministic communication services such as video monitoring and PLC control etc.

4.2	Concepts

Editor's note: this clause will contain concept of deterministic communications, the generic aspects for provisioning and assurance of deterministic communication services.

5	Key Issues and potential solutions

Editor's note: this clause will contain the key issues and potential solutions for deterministic communication service assurance. Relation and potential enhancements to eCOSLA will also be studied for the related key issues.

5.X	Key Issue #1: Provisioning of network functions related to deterministic communication service

Editor's note: this clause will contain the description and potential solutions for provisioning of network functions related to deterministic communication service, e.g. URLLC related network functions, 5GS integration with TSN related network functions.

5.X.1	Description

Editor’s note: This clause provides a description of the key issue#1.

5.X.2	Potential solutions

5.X.2.a	Potential solution #<a>: <Potential Solution a Title>

5.X.2.a.1	Introduction

Editor's Note:	This clause describes briefly the potential solution for key issue#1 at a high-level.

5.X.2.a.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made for key issue#1.

5.X.3	Conclusion - Impact on normative work

Editor's Note:	This clause provides the conclusion from the aspect of impact on normative work for key issue#1.

5.Y	Key Issue #2: Service assurance for video monitoring

Editor's note: this clause will contain the description and potential solutions for the service assurance of video monitoring, e.g. service requirements, related measurements and service performance analysis. Video monitoring is one of the deterministic communication services in cyber-physical control scenarios described in TS 22.104.

5.Y.1	Description

Editor’s note: This clause provides a description of the key issue#2.

5.Y.2	Potential solutions

5.Y.2.a	Potential solution #<a>: <Potential Solution a Title>

5.Y.2.a.1	Introduction

Editor's Note:	This clause describes briefly the potential solution for key issue#2 at a high-level.

5.Y.2.a.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made for key issue#2.

5.Y.3	Conclusion - Impact on normative work

Editor's Note:	This clause provides the conclusion from the aspect of impact on normative work for key issue#2.

5.Z	Key Issue #3: Service assurance for PLC control

Editor's note: this clause will contain the description and potential solutions for the service assurance of PLC control, e.g. service requirements, related measurements and service performance analysis. PLC control is one of the deterministic communication services in cyber-physical control scenarios described in TS 22.104.

5.Z.1	Description

Editor’s note: This clause provides a description of the key issue#3.

5.Z.2	Potential solutions

5.Z.2.a	Potential solution #<a>: <Potential Solution a Title>

5.Z.2.a.1	Introduction

Editor's Note:	This clause describes briefly the potential solution for key issue#3 at a high-level.

5.Z.2.a.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made for key issue#3.

5.Z.3	Conclusion - Impact on normative work

Editor's Note:	This clause provides the conclusion from the aspect of impact on normative work for key issue#3.

6	Conclusions and recommendations

Editor's note: this clause will be used to document the conclusions and recommendation of the study, including potential requirements for the corresponding management services for deterministic communication service assurance.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2022-04 | SA5#142e | S5-222113 |  |  |  | Initial version of TR 28.865 | 0.0.0 |
| 04-2022 | SA5#142e | S5-222676 S5-222677 S5-222678 |  |  |  | Update to implement the agreed pCRs in SA5#142e: - S5-222676 pCR TR 28.865 Skeleton proposal - S5-222677 pCR TR 28.865 Add scope of FS_DCSA - S5-222678 pCR TR 28.865 Add overview of FS_DCSA | 0.1.0 |