3GPP TSG-SA5 Meeting #142-e S5-222784 # e-meeting, 4 - 12 April 2022 CR-Form-v12.1. CHANGE REQUEST. 32.277; CR; 0038; rev; 1; Current version:; 17.1.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests..

| Proposed change affects: |  |  | UICC apps |  |  |  |  | ME |  | Radio Access Network |  |  |  |  |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Introduction of Detailed message format for converged charging |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  |  |  |  |  | Date: |  |  | 2022-3-15 |  |  |  |
| Category: | B |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-17 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |  |  |  |
| Reason for change: |  | The charging information over Nchf for 5G ProSe converged charging is missing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce 5G ProSe information over Nchf |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No charging for 5G ProSe. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x.3(new) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |
| Other comments: |  | This CR needs to be implemented after CR#0037 (inside chapter 6.x of CR#0037) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1st modified section

6.x.3	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data for 5G ProSe converged charging.

The Operation types are listed in the following order: I [Initial] / U (Update)/T [Termination]/E [event]. Therefore, when all Operation types are possible it is marked as IUTE. If only some Operation types are allowed for a node, only the appropriate letters are used (e.g. IUT or E) as indicated in the table heading. The omission of an Operation type for a particular field is marked with "-" (e.g. I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.x.3.1 illustrates the basic structure of the supported fields in the Charging Data Request for 5G ProSe converged charging.

Table 6.x.3.1: Supported fields in Charging Data Request message

| Information Element | Node Type | Direct Discovery | Direct Communication |  |
| --- | --- | --- | --- | --- |
|  | Supported Operation Types | I/U/T/E | I/U/T/E |
| Session Identifier |  | --E | IUTE |  |
| Subscriber Identifier |  | --E | IUTE |  |
| NF Consumer Identification |  | --E | IUTE |  |
| Invocation Timestamp |  | --E | IUTE |  |
| Invocation Sequence Number |  | --E | IUTE |  |
| Retransmission Indicator |  | --- | --- |  |
| One-time Event |  | --E | --E |  |
| One-time Event Type |  | --E | --E |  |
| Notify URI |  | I-- | I-- |  |
| Triggers |  | --E | IUTE |  |
| Multiple Unit Usage |  | --E | IUTE |  |
| Service Information with ProSe Information |  |  |  |  |
| Announcing PLMN ID |  | ---E | - |  |
| Announcing UE HPLMN Identifier |  | ---E | - |  |
| Announcing UE VPLMN Identifier |  | ---E | - |  |
| Monitoring UE HPLMN Identifier |  | ---E | - |  |
| Monitoring UE VPLMN Identifier |  | ---E | - |  |
| Discoverer UE HPLMN Identifier |  | ---E | - |  |
| Discoverer UE VPLMN Identifier |  | ---E | - |  |
| Discoveree UE HPLMN Identifier |  | ---E | - |  |
| Discoveree UE VPLMN Identifier |  | ---E | - |  |
| Monitored PLMN Identifier |  | ---E | - |  |
| ProSe Application ID |  | ---E | - |  |
| Application ID |  | ---E | IUTE |  |
| Application Specific Data |  | - | IUTE |  |
| ProSe functionality |  | ---E | IUTE |  |
| ProSe Event Type |  | ---E | - |  |
| Direct Discovery Model |  | ---E |  |  |
| Validity Period |  | ---E |  |  |
| Role of UE |  | ---E |  |  |
| ProSe Request Timestamp |  | ---E |  |  |
| PC3 Protocol Cause |  | ---E |  |  |
| Monitoring UE Identifier |  | ---E |  |  |
| Requested Application Layer User ID |  | ---E |  |  |
| Requested PLMN Identifier |  | ---E |  |  |
| Time Window |  | ---E |  |  |
| Range Class |  | ---E |  |  |
| Proximity Alert Indication |  | ---E |  |  |
| Proximity Alert Timestamp |  | ---E |  |  |
| Proximity Cancellation Timestamp |  | ---E |  |  |
| Relay IP address |  | - | IUTE |  |
| ProSe UE-to-Network Relay UE ID |  | - | IUTE |  |
| ProSe Destination Layer-2 ID |  | - | IUTE |  |
| Coverage Info |  | - | IUTE |  |
| Radio Parameter Set Info |  | - | IUTE |  |
| Transmitter Info |  | - | IUTE |  |
| Time of First Transmission |  | - | IUTE |  |
| Time of First Reception |  | - | IUTE |  |
| Transmission Data Container |  | - | IUTE |  |
| Reception Data Container |  | - | IUTE |  |
| PC5 Radio Technology |  | ---E | - |  |

Table 6.x.3.2 illustrates the basic structure of the supported fields in the Charging Data Response for 5G ProSe converged charging.

Table 6.x.3.2: Supported fields in Charging Data Response Message

| Information Element | Node Type | Direct Discovery | Direct Communication |  |
| --- | --- | --- | --- | --- |
|  | Supported Operation Types | I/U/T/E | I/U/T/E |
| Session Identifier |  | --E | IUTE |  |
| Invocation Timestamp |  | --E | IUTE |  |
| Invocation Result |  | --E | IUTE |  |
| Invocation Sequence Number |  | --E | IUTE |  |
| Session Failover |  | - | IUTE |  |
| Supported Features |  | - | IUTE |  |
| Triggers |  | I--E | IUTE |  |
| Multiple Unit Usage |  | --E | IUTE |  |
| Result Code |  | --E | IUTE |  |
| Rating Group |  | --E | IUTE |  |
| Granted Unit |  | --E | IUTE |  |
| Tariff Time Change |  | --E | IUTE |  |
| Time |  | --E | IUTE |  |
| Total Volume |  | --E | IUTE |  |
| Uplink Volume |  | --E | IUTE |  |
| Downlink Volume |  | --E | IUTE |  |
| Service Specific Units |  | --E | IUTE |  |
| Validity Time |  | --E | IUTE |  |
| Final Unit Indication |  | --E | IUTE |  |
| Time Quota Threshold |  | --E | IUTE |  |
| Volume Quota Threshold |  | --E | IUTE |  |
| Unit Quota Threshold |  | --E | IUTE |  |
| Quota Holding Time |  | --E | IUTE |  |
| Triggers |  | --E | IUTE |  |

End of changes