3GPP TSG-SA5 Meeting #143-e	S5-223079

- May 09 – 17, 2022, e-Meeting								       		 			       Revision of S5-23abcd

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.538 | CR | 0007 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Revise EAS VNF instantiation procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eECM |  |  |  |  |  | Date: |  |  | 2022-04-29 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The EAS VNF instantiation procedure needs to be revised to align with the asynchronous provisioning operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Revise EAS VNF instantiation procedure according to the asynchronous provisioning operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EAS VNF instantiation procedure not working. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

#### 7.1.2.1	EAS VNF instantiation

Figure 7.1.2.1-1 depicts a procedure that describes how an ASP can consume provisioning MnS to instantiate the VNF instances. It is assumed that provisioning MnS consumer has subscribed to the producer of provisioning MnS to receive notifications.

Figure 7.1.2.1-1: EAS VNF instantiation

1. A consumer invokes createMOI operation in (see clause 11.1.1.1. in TS 28.532) on EASRequirements IOC containing the EAS deployment requirements.

2. Provisioning MnS producer creates the EASRequirements MOI.

3. Provisioning MnS producer returns an output parameter with status = “OperationSucceeded” to indicate the EASRequirements MOI has been created.

4. A consumer invokes createMOI operation in on LCMJob IOC with lcmOperation = “INSTANTIATE” to request the provisioning MnS producer to instantiate the NF instance. The LCMJob IOC contain the following attributes needed for VNF instantiation:

- nfForInstantiation: indicates the type of network function (e.g., “EAS”, “EES”, “ECS”, “PCF”, “UPF”, “NEF”, … etc,) to be instantiated.

- deploymentRequirements: indicate the DN of of the EASRequirements MOI, obtained from step 3.

- vnfLcmInfo: contain the software image information (e.g., software image location, minimum RAM, disk requirements), QoS requirements (e.g., maxLatency, maxBitRate, minBitRate).

- lcmJobStatus.timer: specifies the maximum duration given to an LCM job.

5. Provisioning MnS producer creates the LCMJob MOI.

6.  Provisioning MnS producer returns an output parameter with status = “OperationSucceeded” to to indicate the LCMJob MOI has been created.

7. Provisioning MnS producer analyses the input attributes to determine actions for VNF instantiation, including downloading the VNF software image from the software image location, and setting the timer to lcmJobStatus.timer.

8. Provisioning MnS producer invokes the InstantiateNsRequest operation (see clause 7.3.3 in ETSI GS NFV-IFA 013 [6]) to request NFVO via the Os-Ma-nfvo interface to instantiate VNF instance.

9. Provisioning MnS producer receives from NFVO the NS lifecycle change notification indicating the start of instantiation procedure (see clause 7.3.3.4 of ETSI GS NFV-IFA 013 [6]).

10. Provisioning MnS producer sends a notifyMOIAttributeValueChange on LCMJob MOI with lcmStatus.status = “STARTED” to indicate the VNF instantiation has started.

If the timer has timed out, then performs the following step:

11.1 Provisioning MnS producer sends a notifyMOIAttributeValueChange on LCMJob MOI with lcmStatus.status = “FAILED” and lcmStatus.resultStateInfo = "TIMER_EXPIRED" to indicate the VNF instantiation has timed out.

If the timer has not timed out, then performs the following steps:

11.2. Provisioning MnS producer receives from NFVO the NS lifecycle change notification indicating the result of instantiation procedure (see clause 7.3.3.4 of ETSI GS NFV-IFA 013).

If VNF instantiation has succeeded, then performs the following steps:

11.3. Provisioning MnS producer sends a notifyMOIAttributeValueChange on LCMJob MOI with lcmStatus.status = “FINISHED” to indicate that the VNF instantiation is completed.

11.4. Provisioning MnS producer creates the EASFunction MOI for the instantiated VNF.

11.5. Provisioning MnS producer sends a notification notifyMOICreation to indicate that the EASFunction MOI has been created.

Otherwise (VNF instantiation has failed) perform the following step:

11.6. Provisioning MnS producer sends a notifyMOIAttributeValueChange on LCMJob MOI with lcmStatus.status = “FAILED” to indicate that the NF instantiation has failed.

End of Modified Sections