3GPP TSG-SA5 Meeting #143-e	S5-223081

- May 09 – 17, 2022, e-Meeting								       		 			       Revision of S5-23abcd

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.538 | CR | 0009 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add EAS VNF update procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eECM |  |  |  |  |  | Date: |  |  | 2022-04-29 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The EAS VNF update procedure needs to align with the asynchronous provisioning operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add EAS VNF update procedure according to the asynchronous provisioning operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EAS VNF update is missing from LCM. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

#### 7.1.2.3	EAS VNF update

Figure 7.1.2.3-1 depicts the solution that describes how a provisioning MnS consumer can consume provisioning MnS to update the VNF instances. It is assumed that provisioning MnS consumer has subscribed to the producer of provisioning MnS to receive notifications.

Figure 7.1.2.3-1: EAS VNF update

1. A consumer invokes createMOI operation in (see clause 11.1.1.1. in TS 28.532) on LCMJob IOC with lcmOperation = “UPDATE” to request the provisioning MnS producer to update the VNF instance. The LCMJob IOC contain the following attributes needed for NF instantiation:

- mEForLcm: specifies the Managed Element, which contains the VNF instance to be updated.

- deploymentRequirements: indicate the DN of of the EASRequirements MOI.

NOTE: The EASRequirements MOI has been updated to reflect the EAS VNF update.

- vnfLcmInfo: contain the software image information (e.g., software image location, minimum RAM, disk requirements), QoS requirements (e.g., maxLatency, maxBitRate, minBitRate).

- lcmJobStatus.timer: specifies the maximum duration given to a LCM job.

2. Provisioning MnS producer creates the LCMJob MOI.

3.  Provisioning MnS producer returns an output parameter with status = “OperationSucceeded” to the consumer.

4. Provisioning MnS producer invokes the UpdateNsRequest operation (see clause 7.3.3 in ETSI GS NFV-IFA 013 [5]) to request NFVO via the Os-Ma-nfvo interface to update VNF instance.

5. Provisioning MnS producer receives from NFVO the NS lifecycle change notification indicating the start of instantiation procedure (see clause 7.3.3.4 of ETSI GS NFV-IFA 013 [5]).

6. Provisioning MnS producer sends a notifyMOIAttributeValueChange on LCMJob MOI with lcmStatus.status = “STARTED” to indicate the NF instantiation has started.

7. Provisioning MnS producer receives from NFVO the NS lifecycle change notification indicating the result of instantiation procedure (see clause 7.3.3.4 of ETSI GS NFV-IFA 013 [5]).

If VNF update has succeeded, then performs the following steps:

8.1. Provisioning MnS producer sends a notifyMOIAttributeValueChange on LCMJob MOI with lcmStatus.status = “FINISHED” to indicate that the NF update is completed.

8.2 Provisioning MnS producer modifies the attributes in EASFuncton MOI.

8.2. Provisioning MnS producer sends a notification notifyMOIAttributeValueChange on EASFunction MOI to indicate that the EASFunction MOI has been modified.

Otherwise (VNF update has failed) perform the following step:

8.4. Provisioning MnS producer sends a notifyMOIAttributeValueChange on LCMJob MOI with lcmStatus.status = “FAILED” to indicate that the VNF update has failed.

End of Modified Sections