- 3GPP TSG-SA5 Meeting #143e	S5-223269

- e-meeting, 9-17 May 2022

- Source:	Huawei

- Title:	pCR 28.903 adding key issue for EAS LCM management

- Document for:	Approval

- Agenda Item:	6.5.23.1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	3GPP TS 28.903-010 “Study on alignment with ETSI MEC for Edge computing management”

[2]	S5-222551 Collection of Rel-18 3GPP SA5 OAM WoP

# 3	Rationale

Add key issue for EAS LCM management.

# 4	Detailed proposal

1st modified section

- 5.X	Key Issue#2: EAS LCM management

- 5.X.1	Description

3GPP TS 28.538 [x] provides the EAS lifecycle management procedure by interworking with ETSI NFV MANO NFVO, which considers EAS as a VNF. The interface exposed by NFVO as specified in ETSI NFV IFA013 [y] only includes NS (network service) lifecyc management interface, in order to create a new EAS VNF instance, it has to be done through the management of the NS. This gives additional burden when deploy a new EAS, e.g., a network service needs to be created first. EAS as an application server to support edge computing needs to be deployed as quick and simple as possible. Meanwhile, ETSI MEC provides a more application friendly interface for the lifecycle management of edge applications.

This key issue aims at investigating on potential solutions on alignment with ETSI MEC for EAS lifecyc management.

- 5.X.2	Potential solutions

#### 5.x.2.1	Application instance lifecycle management interface produced by ETSI MEC

ETSI GS MEC 010-2 [3] provides the application instance lifecycle management interface, which includes application instantiation, termination and operation (i.e. start or stop). The application instance lifecycle management interface can be produced by MEO (when ETSI NFV is not involved) or MEAO (when ETSI NFV is involved).

The application instance lifecycle management interface includes the following operations as described in [3]:

•	Create application instance identifier.

•	Application instantiation.

•	Application instance terminate.

•	Delete application instance identifier.

•	Query application instance information.

•	Change application instance state.

•	Query application lifecycle operation Status.

•	Subscribe to notifications relating to application lifecycle management.

- 5.X.2	Potential solutions