3GPP TSG-SA5 Meeting #143-e 	S5-223270

- e-meeting, 9-17 May 2022

- Source:	Huawei

- Title:	pCR Add alarm analytics solution-stage 2

- Document for:	Approval

- Agenda Item:	6.6.5

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	3GPP TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-110 “Management and orchestration; Management Data Analytics”

# 3	Rationale

This contribution is proposed to add MnS component for fault management related issues analysis capability of MDAS in [2].

# 4	Detailed proposal

It is proposed to apply following changes in [2].

1st modified section

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.500: "Telecommunication management; Self-Organizing Networks (SON); Concepts and requirements".

[3]	3GPP TS28.535: “Management services for communication service assurance; Requirements”.

[4]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[5]	3GPP TS 28.554: "5G end to end Key Performance Indicators (KPI)".

[6]	3GPP TS 32.422: "Telecommunication management; Subscriber and equipment trace; Trace control and configuration management".

[7]	3GPP TS 32.423: "Telecommunication management; Subscriber and equipment trace; Trace data definition and management".

[8]	3GPP TS 28.405: "Telecommunication management, Quality of Experience (QoE) measurement collection; Control and configuration".

[9]	3GPP TS 28.406: "Telecommunication management; Quality of Experience (QoE) measurement collection; Information definition and transport".

[10]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[11]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[12]	3GPP TS 32.425: "Telecommunication management; Performance Management (PM); Performance measurements Evolved Universal Terrestrial Radio Access Network (E-UTRAN)".

[13]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[14]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[15]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[16]	3GPP TS 28.658: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[17]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM); Information Service (IS)".

[18]	3GPP TS 32.156: "Telecommunication management; Fixed Mobile Convergence (FMC) Model Repertoire".

[19]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[20]	3GPP TS 28.511: "Telecommunication management; Configuration Management (CM) for mobile networks that include virtualized network functions; Procedures".

[21]	3GPP TS 28.531: "Management and Orchestration; Provisioning".

[22]	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[23]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[x]	ITU-T Recommendation X.733: " Information Technology - Open Systems Interconnection - Systems Management: Alarm Reporting Function ".

2nd modified section

- 7.2.3.x	Alarm analytics

##### 7.2.3.x.1	Discription

3GPP management system shall be able to provide the alarm related analysis.

##### 7.2.3.x.2	Use cases

In 5G network, millions of alarms are generated due to the network complexity. Since the topological relations between different network elements and logical relations between different generated alarms, a series of alarms caused by a same fault should be correlated with each other. In addition, a fault may give rise to the network performance deterioration. For example, an alarm in a lower layer of the communication protocol stack or an alarm related to virtualized resource in the virtualization deployment environment may cause multiple alarms in higher layers.

Large amount of alarms brings difficulties in network operation and maintenance. Therefore, the alarms and deteriorated performance measurements of the same fault should be correlated and analysed.

To satisfy the expectations above, MDA is expected to provide alarm analytical capabilities which can be used in single-domain or cross-domain to achieve rapid and accurate alarm ananlysis.

##### 7.2.3.x.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ALARM_ MDA-1 | MDA capability for Alarm analytics shall be able to provide the alarm related analysis of demarcation to single-domain or cross-domain based on collected data. | Alarm analytics |
| REQ-ALARM_ MDA-2 | MDA capability for Alarm analytics shall have the capability to provide alarm analysis report according to requsts. | Alarm analytics |
| REQ-ALARM_MDA-3 | MDA capability for Alarm analytics should have the capability to provide the analytics output with following information describing the alarm related fault: - Alarm-analysis Identifier - The raised time and cleared time of the Alarm related Fault - The possible root cause(s) - Severity level - Affected objects (MOIs, NFs, etc.) | Alarm analytics |

3rd modified section

### 8.4.3.x	Alarm Analytics

##### 8.4.3.x.1	MDA type

The MDA type for Alarm Analytics is: FaultManagement.AlarmAnalytics.

##### 8.4.3.x.2	Enabling data

The enabling data for alarm analytics are provided in table 8.4.3.x.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.3.x.2-1: Enabling data for alarm analytics

| Data category | Description | References |
| --- | --- | --- |
| Alarm notifications | The alarm information related to NFs. | Clause 11.2 of TS 28.532 [11] |
| Performance measurements | RAN related performance measurements and KPIs. | TS 28.552 [4] and TS 28.554 [5] |
|  | CN related performance measurements and KPIs. | TS 28.552 [4] and TS 28.554 [5] |
| Configuration data | The NRMs containing the attributes affecting the network (NG-RAN or/and 5GC). | TS 28.541 [15] |

##### 8.4.3.x.3	Analytics output

The specific information elements of the analytics output (MDA report) for Alarm Analytics, in addition to the common information elements of the analytics outputs (see clause X.Y), are provided in table 8.4.3.x.3-1.

Table 8.4.3.x.3-1: Analytics output for Alarm analysis

| Information | Description | Support qualifier | Properties |
| --- | --- | --- | --- |
| AlarmAnalysisId | The identifier of this analytics. When reports, this identifier can be used to provide the information to management system to maintain. | M | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| PossibleRootCause | Indicates the root cause of the fault. This field holds the information of the possible cause(s) in alarm analytics report. This information relies on domains and the network context. | O | type: RootCauseData multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| AffectedObjects | The managed object instances where the issue happens. | M | type: DN multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| PerceivedSeverity | This field holds the value to indicate relative level of urgency for operator attention. NOTE: the value can be Critical, Major, Minor, Warning, Indeterminate, Cleared, see ITU-T Recommendation X.733 [x]. | M | type: enumeration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

4th modified section

- 8.5.x	RootCauseData <<dataType>>

- 8.5.x.1	Definition

This data type specifies the type of RootCauseData.

- 8.5.x.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| Id | It provides the identifier of the root cause. | M | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Description | It provides the description of the possible cause, which provides information of the reason that cause the fault. | M | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End modified section