3GPP TSG-SA5 Meeting #143-e 	S5-223271

- e-meeting, 9-17 May 2022

- Source:	Huawei

- Title:	pCR update EsRecommendationsOnUPF and EsRecommendationsOnNRcell

- Document for:	Approval

- Agenda Item:	6.6.5

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	3GPP TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-110 “Management and orchestration; Management Data Analytics”

# 3	Rationale

In the EsRecommendationsOnUPF, the multiplicity of EsUPFs and CandidateUPFs are both multiple. If there are many instances of EsUPF and many instances of CandidateUPFs are present, it is hard to know which candidate UPF(s) will take over the traffic from which ES-UPF.

In addition, the EsRecommendation data type as defined in 8.5.5, includes

| EsRecommendationsOnUPFs | It contains the energy saving recommendations on UPFs. | M | type: EsRecommendationsOnUPF multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| --- | --- | --- | --- |

The multiplicity of EsRecommendationsOnUPFs is also multiple.

It is suggested in EsRecommendationsOnUPF, changing the multiplicity of EsUPFs from multiple to 1. So the CandidateUPFs (1 or many) will just take over traffic from the ES-UPF. Also in some cases, the tranffic does not need to be token over by other UPFs, the CandidateUPFs should be optional.

Similar changes for EsRecommendationsOnNRcell.

# 4	Detailed proposal

It is proposed to apply following changes in [2].

1st modified section

- 8.5.7	EsRecommendationsOnUPF <<dataType>>

- 8.5.7.1	Definition

This data type specifies the type of energy saving recommendations on UPFs.

- 8.5.7.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| EsUPF | It provides the DN of UPF (ES-UPF) which is recommended to conduct energy saving. | M | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CandidateUPFs | It provides the DN of candidate UPFs which are recommended with precedence for taking over the traffic of ES-UPF. | O | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| ConductTime | It indicates the recommended time to conduct energy saving for the ES-UPF | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

2nd modified section

- 8.5.6	EsRecommendationsOnNRcell <<dataType>>

- 8.5.6.1	Definition

This data type specifies the type of energy saving recommendations on NR cells.

- 8.5.6.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| EsNRcell | It provides the DN of NR cell (ES-Cell) which is recommended to enter energySaving state. | M | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| CandidateNRcells | It provides the DN of candidate NR cells which are recommended with precedence for taking over the traffic of ES-Cell. | M | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| EnterTime | It provides the recommended time to enter the energy saving state for the ES-Cell. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| EndTime | It provides the recommended time to terminate the energy saving state for the ES-Cell | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TrafficThresholds | It provides the recommended traffic threshold information. The ES-Cell can enter the energy saving state when the traffic is below the threshold value defined in the thresholdValue. | M | type: ThresholdInfo (see TS 28.622) multiplicity: * isOrdered: False isUnique: False defaultValue: None isNullable: False |

End modified section