3GPP TSG-SA5 Meeting #143-e 	S5-223272

- e-meeting, 9-17 May 2022

- Source:	Huawei

- Title:	pCR update TrafficProjections date type

- Document for:	Approval

- Agenda Item:	6.6.5

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	3GPP TR 28.809 Management and orchestration; Study on enhancement of Management Data Analytics (MDA)

[2]	3GPP TS 28.104-110 “Management and orchestration; Management Data Analytics”

# 3	Rationale

The output elements included in TrafficProjections should not be all mandatory. For example, in the domain specifc case, like CN, only CN domain NFs related analysis result will be present in the report.

# 4	Detailed proposal

It is proposed to apply following changes in [2].

1st modified section

- 8.5.11	TrafficProjections <<dataType>>

- 8.5.11.1	Definition

This data type specifies the traffic projection for a slice.

- 8.5.11.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| ProjectionTime | The time duration for which the projections are made | M | type: ProjectionDuration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| UPFProjections | This specifies the traffic projection of a UPF in the slice. It shall be present only if the analysis target contains CN part. | CM | type: UPFProj multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBProjections | This specifies the traffic projection of a gNB in the slice. It shall be present only if the analysis target contains AN part. | CM | type: gNBProj multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| SMFProjections | This specifies the projected number of PDU session of a SMF in the slice. It shall be present only if the analysis target contains CN part. | CM | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| AMFProjections | This specifies the projected number of registered subscriber of an AMF in the slice. It shall be present only if the analysis target contains CN part. | CM | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End modified section