- 3GPP TSG-SA5 Meeting #143e	S5-223473rev1

- 09 - 17 May 2022, E-meeting

- Source:	Intel

- Title:	pCR 28.104 Enhance the MDA NRM

- Document for:	Approval

- Agenda Item:	6.6.5

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TS 28.104-110 “Management and orchestration; MDA”.

# 3	Rationale

This pCR is to enhance the MDAFunction IOC with the attribute indicating the supported MDA capabilities, and enhance the MDARequest IOC with reportingTarget attribute. The analyticsWindow is redundant from the startTime and stopTime, so it is also removed.

# 4	Detailed proposal

Start of modification

### 9.2.2	Inheritance

Figure 9.2.2-1: Inheritance Hierarchy

Next modification

### 9.3.1	MDAFunction

#### 9.3.1.1	Definition

The IOC MDAFunction represents the MDA function which supports one or more MDA capabilities.

#### 9.3.1.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| supportedMDACapabilities | M | T | F | F | T |
| Attribute related to role |  |  |  |  |  |

9.3.1.3	Attribute constraints

None.

#### 9.3.1.4	Notifications

The common notifications defined in clause 9.6 are valid for this IOC, without exceptions or additions.

Next modification

### 9.3.2	MDARequest

#### 9.3.2.1	Definition

The IOC MDARequest represents the MDA output request created by an MnS consumer.

The attribute requestedMDAOutputs contains one or multiple MDAOutputPerMDAType elements, and each MDAOutputPerMDAType element supports filtering of MDA output for a certain MDA type.

#### 9.3.2.2	Attributes

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| requestedMDAOutputs | M | T | T | F | T |
| reportingMethod | M | T | T | F | T |
| reportingTarget | M | T | T | F | T |
| analyticsScope | M | T | T | F | T |
| startTime | M | T | T | F | T |
| stopTime | M | T | T | F | T |
| analyticsWindow | M | T | T | F | T |
| Attribute related to role |  |  |  |  |  |

#### 9.3.2.3	Attribute constraints

None.

#### 9.3.2.4	Notifications

The common notifications defined in clause 9.6 are valid for this IOC, without exceptions or additions.

Next modification

### 9.5.1	Attribute properties

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| mDAType | It indicates the type of MDA type (corresponding to the MDA capability). AllowedValues: the value of MDA type defined for each MDA capability in clause 8. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| requestedMDAOutputs | It indicates the requested analytics outputs for an MDA request | type: MDAOutputPerMDAType multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mDAOutputIEFilters | It provides the filters for the analytics output information elements of an MDA type for an MDA request. | type: MDAOutputIEFilter multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| mDAOutputIEName | It indicates the analytics output information element name of an MDA type for an MDA request. AllowedValues: the analytics output information element names for each MDA type as specified in clause 8. | type: string multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| filterValue | It indicates the filter value for analytics output information element for an MDA request. The MDA output information element is only requested and reported when its value equals to the value of this attribute. allowedValues: depends on the definitions of the analytics output information element (see clause 8) indicated by mDAOutputIEName attribute. | type: string multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| threshold | It indicates the threshold for analytics output information element for an MDA request. | type: TBD multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True Editor’s note: it is TBD to whether reuse the ThresholdInfo data type defined in 28.622. |
| analyticsPeriod | It indicates a list of times, which may determine a time-period related to a time schedule for analytics period. | type: DateTime (see TS 32.156 [18]) multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| timeOut | It indicates a time until which an MDA MnS consumer needs to obtain an MDA output. Beyond this time the MDA output is no loner needed by the MDA MnS consumer. | type: DateTime (see TS 32.156 [18]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| reportingMethod | It indicates the reporting method of the analytics output selected by the MnS consumer. allowedValues: File, Streaming, Notification. Editor’s note: the detailed solution for Notification based solution is FFS. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| reportingTarget | It indicates the reporting target of the MDA outputs. Allowed values: URI | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| analyticsScope | It indicates the scope of the analytics requested by the MnS consumer. | type: AnalyticsScopeType multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| managedEntitiesScope | It indicates the scope of the analytics by the DNs of the managed entities. It carries the DN(s) of SubNetwork MOI(s), ManagedElement MOI(s), and/or the MOI(s) of the derivative IOCs of ManagedFunction (see TS 28.622 [19]). For each MOI provided by this attribute, the MOI itself and all of its subordinated MOIs are in the scope of analytics. | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| areaScope | It indicates the scope of the analytics by the geographical area information. | type: GeoArea (see TS 28.622) multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| startTime | It indicates the start time of the analytics requested by the MnS consumer. | type: DateTime (see TS 32.156 [18]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| stopTime | It indicates the stop time of the analytics requested by the MnS consumer. | type: DateTime (see TS 32.156 [18]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| analyticsWindow | It indicates the time duration related with the analytics output towards the MDA MnS consumer. | type: TimeWindow multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| supportedMDACapabilities | It indicates the MDA capabilities supported by the MDA function. AllowedValues: the value of MDA types defined for the MDA capabilities in clause 8. | type: String multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |

End of modifications