- 3GPP TSG-SA5 Meeting #143e	S5-223488rev2

- 09 - 17 May 2022, E-meeting

- Source:	Intel, NEC

- Title:	pCR 28.908 Add AI/ML workflow for 5GS

- Document for:	Approval

- Agenda Item:	6.5.5.1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.908-000 “Study on AI/ML management”

# 3	Rationale

The AI/ML techniques are widely used in 5GS (including 5GC, NG-RAN and management system), and in the SA5 study item AI/ML management (see the SID in S5-216427), the following AI/ML management capabilities are to be studied to enable and support AI/ML in 5GS:

- Validation of AI/ML model and AI/ML-enabled function

- Testing of AI/ML model and AI/ML-enabled function (before deployment)

- Deployment of AI/ML model (new or updated model) and AI/ML-enabled function

- Configuration of AI/ML-enabled function

- Performance evaluation of AI/ML-enabled function

The AI/ML workflows involving these AI/ML management capabilities could be generic for different domains (5GC, NG-RAN and management system) and for different AI/ML-enabled functions.

The generic AI/ML workflows are helpful to illustrate where the AI/ML management/operations are needed, therefore they are useful to derive the use cases and requirements for AI/ML management.

This pCR is to add the generic AI/ML workflows.

# 4	Detailed proposal

Start of modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[3]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[a]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence / Machine Learning (AI/ML) management".

Start of modification

# X	Concepts and overview

## X.1	AI/ML workflow for 5GS

AI/ML techniques are widely used in 5GS (including 5GC, NG-RAN and management system), and the generic workflow for AI/ML management in 5GS, including the operational steps in the lifecycle of an AI/ML entity, is shown in Figure X.1.1-1.

Figure X.1.1-1: Workflow for management of AI/ML

Editor’s note: the detailed sequence of the figure may need to be revisited.

The workflow involves the training phases and inference phases, and the operation steps for each phase are are briefly described below:

Training phase:

AI/ML Training: Learning by the Machine from the training data to generate the (new or updated) AI/ML entity (see TS 28.105 [a]) that could be used for inference. The AI/ML Training also includes the validation of the generated AI/ML entity to evaluate the performance variance of the AI/ML entity when performing on the training data and validation data. If the validation result does not meet the expectation (e.g., the variance is acceptable), the AI/ML entity needs to be re-trained. This is the initial step of the workflow. The AI/ML Training MnS is specified in TS 28.105 [a].

AI/ML Testing: Testing of the validated AI/ML entity with testing data to evaluate the performance of the trained AI/ML entity for selection for inference. When the performance of the trained AI/ML entity meets the expections on both training data and validation data, the AI/ML entity is finally tested to evaluate the performance on testing data. If the testing result meets the expectation, the AI/ML entity may be counted as a candidate for verification and/or deployment, otherwise the AI/ML entity may need to be further (re)trained.

AI/ML Verification: Verification of the AI/ML entity about whether it works in the AI-enabled function or the target node (which is the same as, or can simulate, the target function or node where the AI/ML entity is to be used for inference). In some cases, this step may be skipped, for instance in case the input data and output data, data types and formats, have been unchanged from the last AI/ML entity.

Inference phase:

AI/ML Configuration: Configuration of the AI/ML entity to control as well as activation and deactivation of the AI/ML entity.

AI/ML Monitoring: Monitoring of the activated AI/ML entity that is being used for inference, including performance evaluation and fault supervision.

End of modifications