3GPP TSG SA WG5 Meeting 143-e	S5-223509

- electronic meeting, online, 9– 17th  May 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.405 | CR | 0009 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Signalling Based Activation for NR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eQoE |  |  |  |  |  | Date: |  |  | 2022-04-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The feature was missing in the specification |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Signalling Based Activation for NR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not aligned with RAN and CT groups resulting in interoperability problems |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.z (new), 4.z.1 (new), 4.z.1.2 (new), 4.z.1.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.405 CR 0011 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

## 4.z	Signalling based activation in NR

### 4.z.1	Activation of measurement collection for a UE in NR

Activation of measurement collection for a UE can be done after UE is registered or before UE Registration procedure.

#### 4.z.1.1 Activation of QoE measurement task after completion of UE registration procedure

Figure 4.z.1-1 and the text below describes the activation of QoE measurement collection after after completion of UE registration procedure.

Figure 4.z.1-1: QMC activation and reporting in NR after UE is registered

- The MnS Consumer sends createMOI request for QMCJob to UDM that controls the impacted gNB(s), and includes the parameters: serviceType, areaScope, sliceScope, qoECollectionEntityAddress, qoETarget, qoEReference and contAppLayerMeasConfigRef.

- The UDM inserts subscriber related data and forwards it to the AMF.

- The AMF forwards the configuration parameters serviceType, areaScope, sliceScope, qoECollectionEntityAddress, qoEReference and ContainerForApplicationLayerMeasurementConfiguration in message UE Context Modification Request to the impacted gNB.

- The gNB checks if the UE capability matches the criteria for serviceType in the QoE measurement configuration information.

- If the UE has the wanted UE capability, the gNB starts a UE request session and stores the associated qoECollectionEntityAddress, sends the message RRCReconfiguration to the UE, and includes the following: serviceType, measConfigAppLayerId and measConfigAppLayerContainer.

- The access stratum in the UE sends an unsolicited response to the application level and includes the following: serviceType, measConfigAppLayerId and appMeasConfigFile.

- When the application for the specified serviceType starts, the QMC is initiated.

- The application layer sends the AT command +CAPPLEVMRNR [7] including measConfigAppLayerId and qoEMeasurementStatus that indicates that a session is started to the access stratum.

- The UE sends the message MeasurementReportAppLayer including measConfigAppLayerId and qoEMeasurementStatus to the gNB.

- The gNB sends a notification to the MnS consumer.

- When the QMC is completed or at the end of period for periodic report, the recorded information is collected in a QMC report [6], [7]. The parameters qoEReference, Client Id [6] and [7] in the reporting container (that represent the UE request session), and recordingSessionId which is generated by the application in the UE are needed in the QMC collection entity for post processing purposes.

- The application layer sends the AT command +CAPPLEVMRNR [7] including measConfigAppLayerId, qoEMeasurementStatus indicating that session has stopped and the QMC report including recordingSessionId to the access stratum.

- The UE sends the message MeasurementReportAppLayer including measConfigAppLayerId, qoEMeasurementStatus and the QMC report including recordingSessionId to the gNB.

- The gNB sends the QMC report to the MCE associated to the qoEReference, recordingSessionId and sliceScope which contains only the S-NSSAI used.

#### 4.z.1.2 Activation of QoE measurement task before UE Registration procedure to the network

At UE Registration the configuration parameters for the QoE measurement collection for a job is propagated to the UE.

When UDM activates the QoE measurement collection for a job to the AMF, the following configuration parameters shall be included in the message: serviceType, areaScope, sliceScope, qoECollectionEntityAddress, qoETarget, qoEReference and containerForApplicationLayerMeasurementConfiguration.

The AMF receives and stores QoE measurement collection job as part of API: Numd_SDM . Then the same procedure for activation of measurement collection job is applied as after completion of UE registration procedure, except that instead of UE Context Modification Request the message Initial Context Setup Request is sent, described in see 4.z.1.2.

Figure 4.z.1-2: QMC activation and reporting in NR before UE Registration procedure to the network

End change