# 3GPP TSG-SA5 Meeting #	S5- 224098

# June 2022, E-meeting

# Source:	ZTE Corporation,China Mobile

# Title:	pCR 28.909 Add ANLS evaluation of autonomous network

# Document for:	Approval

# Agenda Item:	6.7.2.1

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1] 	3GPP draft TR 28.909: “Management and orchestration; Study on evaluation of autonomous network levels v0.1.0”.

3	Rationale

This contribution proposes to add ANLS evaluation method of autonomous network level  in 5.X.3 Potential solutions for process of ANL evaluation based on concept for autonomous network level evaluation described in TR 28.909 [1].

- Detailed proposal

It proposes to make the following changes to TR 28.909 [1].

1st Change

5.X.3		Potential solutions for process of ANL evaluation

5.X.3.X ANLS evaluation of autonomous network

5.X.3.X.1 Process of ANLS evaluation

The following steps describe the ANLS evaluation :

Figure 5.X.3.X.1-1 Process of ANLS evaluation

- Identify tasks involved in the ANLS evaluation.

Each key tasks needs to be focused for autonomous network level evaluation.

Tasks involved in the the ANLS evaluation are marked as Task1,Task2,...,Taskn.

- Evaluate the score of the tasks.

Tasks are scored based on the considerations including the degree of involvement of human and telecom system and KEI. The task score range is [0,1) and the precision is left to implementation.

The corresponding scores of Task1,Task2,...,Taskn are marked as S1,S2,...,Sn.

- Assign weight of each involved task.

The weight can be assigned based on implementation difficulty and operational cost.

The corresponding weights of Task1,Task2,...,Taskn  are marked as W1,W2,...,Wn and the sum of the weights is 1, i.e. k=1nWk = 1.

- Calculate ANLS

Summarize Sk∗Wk of all the tasks involved in the evaluation(i.e. k=1nSk∗Wk). The precision is left to implementation. The score is marked as Ssum.

Finally, ANLS is derived based on the autonomous network level and Ssum.

5.X.3.X.2 Example of ANLS evaluation

Take ANLS evaluation of radio coverage optimization Level 2 as an example.

- Identify tasks involved in the ANLS evaluation.

Tasks involved in the evaluation can be:

-	Tasks accomplished by collaboration of human and telecom system

-	Tasks accomplished by human but required to be accomplished by collaboration of human and telecom system for ANL upgrading(e.g. task E in level 2 in Figure 4.1.1-1).

-	Tasks accomplished by human but not required to be accomplished by collaboration of human and telecom system for ANL upgrading(e.g. Task A in level 2 in Figure 4.1.1-1) are left to implementation.

Tasks that need not be involved in the evaluation can be:

-	Tasks fully accomplished by the telecom system.

In this example, Task C, D, E, F,G,H and I are involved in the ANLS evaluation. These seven tasks are marked as Task1,Task2,...,Task7.

- ~ 4) Table 5.X.3.X.2-1shows the following evaluation process:

- Table 5.X.3.X.2-1 Example of ANLS evaluation of radio coverage optimization Level 2

| All the tasks of radio coverage optimization | A | B | C | D | E | F | G | H | I | J |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tasks involved in ANLS evaluation | / | / | C | D | E | F | G | H | I | / |
| Sk | / | / | 0.9 | 0.9 | 0.9 | 0.2 | 0.9 | 0.1 | 0.5 | / |
| Wk | / | / | 30% | 15% | 15% | 10% | 15% | 5% | 10% | / |
| Ssum | k=17Sk∗Wk= 0.795, e.g. round to 0.8 |  |  |  |  |  |  |  |  |  |
| ANLS | 2.8 |  |  |  |  |  |  |  |  |  |

End of changes