3GPP TSG-SA5 Meeting #144-e 	S5-224104

# e-meeting, 27 June- 1 July 2022

# Source:	AsiaInfo

# Title:	Add use case of Intent driven management for network slice feasibility check

# Document for:		Approval

Agenda Item:	6.7.4.1

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1] 	3GPP TR 28.836: “Study on intent-driven management for network slicing v0.2.0”.

3	Rationale

This document is to add use case of Intent driven management for network slice feasibility check regarding network slice instance and network slice subnet instance.

4	Detailed proposal

The start of the change

5.y	Intent driven management for network slice feasibility check

5.y.1 	Use case of Intent driven management to express expectation of network slice feasibility check

5.y.1.1	Introduction

To satisfy the requirement of checking the feasibility of provisioning a network slice, the intent driven MnS producer receive the intent expectation of network slice feasibility check and provide the management capabilities to check the feasibility of provisioning a network slice subnet(s) of the network slice.

5.y.1.2 	Pre-condition

CSP provides the intent driven management service to CSC to create a communication service.

Network operator is capable to provide network slice feasibility check in management system to invoke the proper intent driven MnS.

5.y.1.3	Description

In order to enable a group of end users to check the feasibility of provisioning a network slice provided by a 3GPP network, a MnS Consumer expresses its intent for the network slice feasibility check with proper network characteristics to network operator.

The operator provides the expectation of network slice feasibility check with a set of network characteristics. The intent diven management service producer is capble to translate the intent of network slice feasibility check capability to relevant requirements e.g., in terms of resources. Regarding the intent MnS request received, the intent driven-management service provider may decide management capabilities to meet the requirements of checking the feasibility of provisioning a network slice subnet(s) of the network slice, for example analysing network constituents to ensure that their capabilities, e.g., resources, management services, etc. are (or will be) adequate to provision network slice instance. The intent driven management service producer is also capable to provide the report of intent fulfilment of network slice feasibility check addressing the certain management capabilities of intent.

5.y.1.4	Post-condition

The network slice feasibility check expectation expressed by intent for a specified group of end users is fulfilled.

5.y.1.5	Requirements

REQ-Intent_Expose_Slice-CON-1: The intent driven MnS shall have capability enabling MnS consumer to express intent containing an expectation for network slice feasibility check.

5.y.2 	Use case of Intent driven management to express expectation of network slice subnet feasibility check

5.y.2.1	Introduction

To satisfy the requirement of checking the feasibility of provisioning a network slice subnet, the intent driven MnS producer receive the intent expectation of network slice subnet feasibility check and provide the management capabilities to check the feasibility of provisioning a network slice subnet(s) of the network slice instance.

5.y.2.2 	Pre-condition

NOP provides the intent driven management service to create a network slice subnet instance.

Network operator is capable to provide network slice subnet feasibility check in management system to invoke the proper intent driven MnS.

5.y.2.3	Description

In order to enable a group of end users to check the feasibility of provisioning a network slice subnet provided by a 3GPP network, a MnS Consumer expresses its intent for the network slice subnet feasibility check with proper network characteristics to network operator.

The operator provides the expectation of network slice subnet feasibility check with a set of network characteristics. The intent diven management service producer is capble to translate the intent of network slice subnet feasibility check capability to relevant requirements e.g., in terms of resources. Regarding the intent MnS request received, the intent driven-management service provider may decide management capabilities to meet the requirements of checking the feasibility of provisioning a network slice subnet(s) of the network slice instance, for example obtain information from the network (e.g., current or predicted load level information, current or predicted resource usage information from management data analytics services). The intent driven management service producer is also capable to provide the report of intent fulfilment of network slice subnet feasibility check addressing the certain management capabilities of intent.

5.y.2.4	Post-condition

The network slice subnet feasibility check expectation expressed by intent for a specified group of end users is fulfilled.

5.y.2.5	Requirements

REQ-Intent_Expose_Slice-CON-2: The intent driven MnS shall have capability enabling MnS consumer to express intent containing an expectation for network slice subnet feasibility check.

The end of the change