3GPP TSG-SA5 Meeting #144-e 	S5-224107

# e-meeting, 27 June - 1 July 2022

# Source:	Huawei

# Title:	pCR 28.834 Add concepts from ETSI NFV

# Document for:	Approval

Agenda Item:	6.8.5.1

# 1	Decision/action requested

For approval

# 2	References

[1]	3GPP TR 28.834 V0.1.0 Study on Management of Cloud Native Virtualized Network Functions

# 3	Rationale

This contribution proposes to add background information to explain the ETSI NFV definition of cloud native VNF.

# 4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	ETSI GS NFV-EVE 011 “Specification of the Classification of Cloud Native VNF implementations”

2nd change

# 4	Concepts and background

Editor's note: this clause will contain concepts and background of relevant studies in other SDOs or industry parties.

## 4.x	Cloud-native related concepts from ETSI NFV

ETSI NFV has described in ETSI GS NFV-EVE 011 [x] a set of characteristics which relate to cloud-native virtualized network functions (VNFs). A cloud-native VNF may have one or several of the following characteristics:

Redundancy – Resiliency may be achieved on VNF level through redundancy of VNFCs, or resiliency may be achieved on NS level through redundancy of VNFs.

Fault Monitoring and Failure Detection – Cloud-native VNFs are expected to be equipped with appropriate mechanisms to monitor and support the general operational health of the VNF.

Scaling – One of the mechanisms is to support scaling by adding/removing VNFC instances, in response to changed demand loads. Another mechanism is to add/remove VNF instances to an NS and share the load between those.

Decomposition – Composition of VNFs using NFV granular functions enables instantiating and deploying only the essential VNFC functions as needed for the service. NFV micro-services and container technologies may be used for composition of VNFs.

Location independence – Cloud-native VNFs can be instantiated in any location that meets the performance, latency or regulatory requirements of the network service.

Use of APIs – Interaction between VNFC or between VNFs is through APIs.

Automated Instantiation and Configuration – Cloud-native VNF should be automatically configured or self-configured where possible.

Load Balancing – Load balancer systems distribute the traffic between VNFs or between components (VNFCs) inside a VNF instance.

Automated Resource Management – For the separation of infrastructure resource management and VNF resource management, the two are logically decomposed and are handled in separate management components or through an infrastructure planning process.

End of changes