3GPP TSG-SA5 Meeting #144e	S5-224137

e-meeting 27 June - 1 July 2022

# Source:	China Unicom

# Title:	Add solution for URLLC performance measurements related to UL resource load with power boosting

# Document for:	Approval

Agenda Item:	6.8.3 Study on Management Aspects of URLLC

1	Decision/action requested

The group is asked to approve the proposal.

2	References

[1]	3GPP TR 28.832 v0.2.0: “Management Aspects of URLLC”

3	Rationale

It was approved in SP-220146 to study the management aspects of URLLC and one of the objectives is to investigate performance measurements related to URLLC. In order to achieve the objective mentioned above, some performance measurements related to URLLC is proposed in this contribution.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

5	Key Issues Investigation and Potential Solutions

5.X	Key Issue #X: Performance measurements related to URLLC

5.X.1	Description

5.X.2	Potential solutions

In order to measure resource multiplexing and preemption under multi-service coexistence scenarios to reflect resource load and resource allocation rationality, new measurements and KPIs should be added to effectively evaluate resource load of URLLC services under eMBB and URLLC multiplexing scenarios. Referring to TS 38.213, for resource multiplexing and preemption under multi-service coexistence scenarios, the uplink service scenario defines PB (Power Boosting) feature. Accordingly, the following measurement is proposed.

#### 5.X.2.1 UL PB Time Domain Proportion

a)	This measurement provides the proportion of time domain resources that invoke the power boosting（PB） feature in the statistical period.

b)	SI

c)	This measurement is obtained as: PPBT=PBTNUTT∗100,

where

PPB(T) denotes the proportion of time domain resources that invoke the PB feature during the time period T, with value range: 0-100%;

PB(T) is the number of sampling occasions that invoke the PB feature during the time period T,  PB(T)=iPBi(T);

PBiT is the invoking PB feature result of sampling occasion i,  when the number of physical resource blocks (PRBs) that invoke the PB feature at sampling occasion i is greater than 0, PBi(T) = 1, and when the number of PRBs that invoke the PB feature at sampling occasion i is equal to 0, PBi(T) = 0;

NUT(T) is the number of sampling occasions with UL data scheduled during the time period T, NUT(T)=iNUT,i(T)；

NUT,i(T) is the UL data scheduled result of sampling occasion i, when there is UL data scheduled at sampling occasion i, NUT,i(T)=1, and when there is no UL data scheduled at sampling occasion i, NUT,i(T)=0;

T denotes the time period during which measurement is performed;

i denotes sampling occasion (e.g. 1 slot) during time period T.

d)	A single integer value from 0 to 100.

e)	RRU.PbUtUl, which indicates the proportion of time domain resources that invoke the PB feature

f)	NRCellDU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for evaluating the resource load of URLLC services under eMBB and URLLC multiplexing scenarios.

End of changes