3GPP TSG-SA5 Meeting #144-e 	S5- 224162

# e-meeting, 27 June - 1 July 2022

# Source:	China Unicom

# Title:	Add key issue remote access requirement of MOP-SR-DM for each POP

# Document for:	Approval

Agenda Item:	6.8.6

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP TR 28.835 v0.2.0: “Management Aspects of 5G MOCN Network Sharing Phase2”

3	Rationale

It was approved to study the enhancement of management aspects for 5G MOCN Network Sharing [1]. One of the objectives of this new study item is studying the requirements of POP and MOP. For supporting the POP’s network operation,  each POP needs to get and view some individual requirement data from MOP directly. So MOP-SR-DM needs to provide remote access and relevant system user permissions for each POP.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

5.X	Issue #X:  Remote access requirement of MOP-SR-DM for each POP

5.X.1	Description

For MOCN Networking Sharing and POP’s network operation requirements, each POP needs to get and view some individual requirement data from MOP-SR-DM directly. In TS 32.130[4], the data of MOP-SR-DM is transmitted to POP-NM through MOP-NM, which cannot meet the real-time acquisition and viewing data requirements of each POP. This data of MOP-SR-DM contain configuration data, performance data and alarms here. According to the POP’s real-time data requirements, MOP-SR-DM needs to allow each POP to be able to access remotely and provide relevant system user permissions for each POP.

End of Change