3GPP TSG-SA5 Meeting #144-e 	S5-224168

# e-meeting, 27 June – 1 July 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.313 | CR | draftCR | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 draftCR 28.313 Add beam specific inter-RAT handover counters related to MRO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2022-06-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Add new PMs to support analytics per beam-cell-pair related to inter-system mobility. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce new PMs related to handover failure per beam-cell-pair related to MRO for inter-system mobility. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Use case for beam handover inter-system mobility cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.552 draftCR S5-224166 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First changes

#### 7.1.2.3	MnS Component Type C definition

##### 7.1.2.3.1	Performance measurements

Performance measurements related MRO are captured in Table 7.1.2.3.1.-1:

Table 7.1.2.3.1-1. MRO related performance measurements

| Performance measurements | Description | Related targets |
| --- | --- | --- |
| Number of handover events | Includes all successful and unsuccessful handover events (see clause 5.1.1.6 in TS 28.552 [5]). | Total handover failure rate |
| Number of handover failures | Includes unsuccessful handover events with failure causes (see clause 5.1.1.6 in TS 28.552 [5]). | Total handover failure rate |
| Number of intra-RAT handover events | Includes all successful and unsuccessful intra-RAT handover events (see clauses 5.1.1.6.1 and 5.1.1.6.2 in TS 28.552 [5]). | Total intra-RAT handover failure rate |
| Number of intra-RAT handover failures | Includes unsuccessful intra-RAT handover events with failure causes (see clauses 5.1.1.6.1 and 5.1.1.6.2 in TS 28.552 [5]). | Total intra-RAT handover failure rate |
| Number of inter-RAT handover events | Includes all successful and unsuccessful inter-RAT handover events (see clause 5.1.1.6.3 in TS 28.552 [5]). | Total inter-RAT handover failure rate |
| Number of inter-RAT handover failures | Includes unsuccessful inter-RAT handover events with failure causes (see clause 5.1.1.6.3 in TS 28.552 [5]). | Total inter-RAT handover failure rate |
| Number of intra-RAT too early handover failures | Detected when an RLF occurs after the UE has stayed for a long period of time in the cell (see clause 5.1.1.25.1 in TS 28.552 [5]). |  |
| Number of intra-RAT too late handover failures | Detected when an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure (see clause 5.1.1.25.1 in TS 28.552 [5]). |  |
| Number of intra-RAT handover failures to wrong cell | Detected when an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure (see clause 5.1.1.25.1 in TS 28.552 [5]). |  |
| Number of intra-RAT too early handover failures per source beam | Detected when an RLF occurs after the UE has stayed for a long period of time in the cell (see clause 5.1.1.25.5 in TS 28.552 [5]). |  |
| Number of intra-RAT too late handover failures per source beam | Detected when an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure (see clause 5.1.1.25.5 in TS 28.552 [5]). |  |
| Number of intra-RAT handover failures to wrong cell per source beam | Detected when an RLF occurs shortly after a successful handover from a source cell to a target cell or a handover failure occurs during the handover procedure (see clause 5.1.1.25.5 in TS 28.552 [5]). |  |
| Number of intra-RAT too early handover failures per source beam | Detected when an RLF occurs after the UE has stayed for a long period of time in the cell (see clause 5.1.1.25.5 in TS 28.552 [5]). |  |
| Number of inter-RAT too early handover failures | Detected when an RLF occurs after the UE has stayed in an E-UTRAN cell which connects with 5GC for a long period of time (see clause 5.1.1.25.2 in TS 28.552 [5]). |  |
| Number of inter-RAT too late handover failures | Detected when an RLF occurs shortly after a successful handover from an E-UTRAN cell which connects with EPC to a target cell in a E-UTRAN cell which connects with 5GC (see clause 5.1.1.25.2 in TS 28.552 [5]). |  |
| Number of inter-RAT too late handover failures per source beam | Detected when an RLF occurs after the UE has stayed in an NG-RAN cell for a long period of time; the UE attempts to reconnect to a cell belonging to an E-UTRAN node (see clause 5.1.1.25.x in TS 28.552 [5]). |  |
| Number of unnecessary handover to another RAT | Detected when a UE is handed over from NG-RAN to other system (e.g. UTRAN) even though quality of the NG-RAN coverage was sufficient for the service used by the UE (see clause 5.1.1.25.3 in TS 28.552 [5]). |  |
| Number of unnecessary handover to another RAT per source beam | Detected when a UE is handed over from NG-RAN to other system (e.g. UTRAN) even though quality of the NG-RAN coverage was sufficient for the service used by the UE (see clause 5.1.1.25.y in TS 28.552 [5]). |  |
| Number of inter-RAT handover ping pong | Detected when an UE is handed over from a cell in a source system (e.g. NG-RAN) to a cell in a target system different from the source system (e.g. E-UTRAN), then within a predefined limited time the UE is handed over back to a cell in the source system, while the coverage of the source system was sufficient for the service used by the UE (see clause 5.1.1.25.4 in TS 28.552 [5]). |  |
| Number of inter-RAT handover ping pong per source beam | Detected when an UE is handed over from a cell in a source system (e.g. NG-RAN) to a cell in a target system different from the source system (e.g. E-UTRAN), then within a predefined limited time the UE is handed over back to a cell in the source system, while the coverage of the source system was sufficient for the service used by the UE (see clause 5.1.1.25.z in TS 28.552 [5]). |  |

End of changes