# 3GPP TSG-SA5 Meeting #144e 	S5-224226

# e-meeting, 27 June – 01 July 2022

# Source:	Huawei

# Title:	DP on scenarios of KQIs of video uploading, remote controlling and cloud VR

# Document for:	Discussion

Agenda Item:	6.9.4.1

1	Decision/action requested

In this document the detailed description of KQI scenarios are discussed.

2	References

[1]	 3GPP TR 28.863 v0.2.0: "Study on Key Quality Indicators (KQIs) for 5G service experience"

[2]	 3GPP TS 26.247 "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)"

[3]	 3GPP TS 26.118: "Virtual Reality (VR) profiles for streaming applications".

3	Discussion

In the SI of KQI_5G the KQIs of the following scenarios will be studied:

- Video uploading

- Remote controlling

- Cloud VR

This document will discuss the detailed description of scenarios and make a comparison of the scenarios discussed in SA4.

3.1      Video Uploading

The scenario of video uploading include video generation, video uplink transportation and video playing. Video generation related to the process of generating the video. Video uplink transportation related to the video uplink to the server. Video playing related to the video presentation at the client.

Figure 1: Video uploading scenario

In TS 26.247 [2] from SA4 defines the QoE matrics of the service of Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH). DASH enables to provide services to deliver continuous media content over Hypertext Transfer Protocol (HTTP) in a sense that all resources that compose the service are accessible through HTTP-URLs and the HTTP/1.1 protocol may be used to deliver the metadata and media data composing the service. In TS 26.247 [2] in chapter 10 the QoE matrics for Progressive Download and DASH are defined. There are QoE matrics for media content in MPD information and QoE matrics for media playing in Play List. These QoE matrics could be used as the source of the KQI of video generation and video playing in the scenario of video uploading.

3.2      Remote controlling

In remote controlling scenarios, real-time interaction services are involved. URLLC slicing is used in 5G SA networking to meet ultra-low latency requirements. Real-time interactive services are mainly small-packet transmission services, which do not require network bandwidth but require ultra-low latency. For industrial-grade URLLC services, unqualified delay may cause service failures instead of performance problems. Therefore, it is important to analyze the delay fulfillment boundary. The network side needs to ensure that the delay is lower than the boundary instead of the absolute value of the delay. For example, if the SLA requirement is 10 ms, the evaluation focuses on ensuring that most service measurements are within the 10 ms boundary, or the proportion that exceed the delay boundary is as low as possible.

The scenario of remote controlling in this TR means PLC (Programmable Logic Controller) controlling. The process is in Figure 2. It is not in the  scope of SA4 (the main objectives of the 3GPP TSG SA WG4 (SA4) are the specifications of codecs for speech, audio, video, graphics and other media types related to emerging services such as extended realities (XR) and gaming, as well as the system and delivery aspects of such contents.). We will mainly focus on the network support for PLC controlling. The KQIs for remote controlling should be studied and defined in SA5.

Figure2 Remote controlling scenario

3.3      Cloud VR

Cloud VR (Cloud Virtual Reality) may become one of the most preferred eMBB services for many commercial 5G operators. It introduces the concepts and technologies of cloud computing and cloud rendering to VR service applications. With a high-speed and stable network, the cloud display output and audio output are encoded, compressed, and then transmitted to users' terminals. In this way, VR service content can be migrated to the cloud and rendered to the cloud. Cloud VR service experience is affected by various factors, such as media quality, network quality, and terminal quality. Therefore, the development of measurable and manageable service experience indicators will be a key basis for carriers to ensure the satisfaction of consumers and enterprises. This KQI modeling system applies to scenarios such as VR games, VR 360-degree video, VR live broadcast, giant-screen cinema, and VR education.

In TS 26.118 [3] VR matrics are introduced as the scenarios in the Figure 9.2.1-1: Client reference architecture for VR metrics in [3] from OP1 to OP5 as listed below.

OP1 (DASH access engine):

-	A sequence of transmitted network requests, each defined by its transmission time, contents, and the TCP connection on which it is sent

-	For each network response, the reception time and contents of the response header and the reception time of each byte of the response body

-	The projection/orientation metadata carried in network manifest file if applicable

-	The reception time and intended playout time for each received segment

OP2 (file decoder):

-	Media resolution

-	Media codec

-	Media frame rate

-	Media projection, such as region wise packing, region wise quality ranking, content coverage

-	Mono vs. stereo 360 video

-	Media decoding time

OP3 (sensor):

-	Head pose

-	Gaze direction

-	Pose timestamp

-	Depth

OP4 (render)：

-	The media type

-	The media sample presentation timestamp

-	Wall clock counter

-	Actual presentation viewport

-	Actual presentation time

-	Actual playout frame rate

-	Audio-to-video synchronization

-	Video-to-motion latency

-	Audio-to-motion latency

OP5 (VR application):

-	Display resolution

-	Max display refresh rate

-	Field of view, horizontal and vertical

-	Eye to screen distance

-	Lens separation distance

-	OS support, e.g. OS type, OS version

Furthermore, not all of the above listed matrics are defined as QoE matrics. The defined metrics are from the following three aspects:

- Comparable quality viewport switching latency

- Rendered viewports metric

- VR Device information

Note that the metric functionality is based on the QoE metrics concept in 3GP-DASH [2], but further extended to also cover VR-specific metrics.

The KQIs for cloud VR could be studied based on the QoE matrics in TS26.118 and other source like application layer measurements. If any more matrics are needed for OAM, LS could be sent to SA4 to consider defining the new QoE matrics.

4	Detailed proposal

It is proposed that the detailed description of the scenarios of video uploading, remote controlling and cloud VR be added in TR 28.863.