# 3GPP TSG-SA5 Meeting #144e	 S5-224271

# 27 June to 1 July 2022, E-meeting

# Source:	China Mobile

# Title:	Add potential solution related with number of notifications for NWDAF Data Collection

# Document for:	Approval

# Agenda Item:	6.7.6.2

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TR 28.864-020: " Study on Enhancement of the management aspects related to NWDAF".

[2]				3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

3	Rationale

In  [1],  the key issue #4 of performance management for NWDAF data collection is proposed. This pCR is to provide the potential solution related with the number of notifications for NWDAF data collection.

4	Detailed proposal

Start of 1st Change

4.4	Key Issue #4: Performance Measurement for NWDAF Data Collection

4.4.1	Description

The Data Collection feature permits NWDAF to retrieve data from various data sources (e.g., NF such as AMF, SMF, PCF, UDM and AF; OAM), as a basis of the computation of network analytics [2].

The retrieval of data has impacts on the performance of both the data source and NWDAF. For the data source, all the data required by NWDAF needs to be generated or prepared accordingly before they are retrieved by NWDAF. And for the NWDAF, it needs to distinguish every piece of data received from different data sources and of data received from the same data source but for different network analytic purposes.

The operator may notice that the data collection is the major task of one NWDAF instance, therefore, less computation resource can be allocated to that NWDAF instance, and more storage and network resources may be allocated if necessary. Or on the extreme cases, a new NWDAF instance needs to be provided or we may find out that deploying a DCCF is the optimum solution. For making a decision, the measurement data is needed, such as how many data is collected by a NWDAF instance or by all NWDAF instances related to some areas of interests, so that the operator could have the understanding and estimation of the working load and working status of the NWDAF instance.

Moreover, it is also beneficial to have the granular measurement of data collection, such as measuring the data collected from different type of data sources. This measurement may provide information about if it is possible to optimize the deployment of NWDAF. For example, the NWDAF instance may be geographically deployed closer to its major data source to reduce the latency and save network resources.

In this key issue, the potential solutions are provided to define the new performance measurement reflecting the data collection performed by NWDAF.

4.4.2	Potential solutions

4.4.2.i	Potential solution #<i>: number of notifications for NWDAF Data Collection

4.4.2.i.1	Introduction

Data collection may be performed via DCCF, MFAF and ADRF, when such NFs are deployed, or via NWDAF (hosting DCCF and/or ADRF). DCCF, MFAF, and NWDAF hosting DCCF shall use at least one of the following services from OAM, and NFs (including AFs directly or via NEF) to collect data. The services includes (see TS 23.288 [2]):

-	the Generic management services, the Performance Management services or the Fault Supervision services, offered by OAM in order to collect OAM global NF data.

-	the Exposure services offered by NFs in order to retrieve data and other non-OAM pre-computed metrics available in the NFs.

-	Other NF services in order to collect NF data (e.g. NRF)

-	DCCF data management service to retrieve data using DCCF.

The number of notifications received by DCCF, MFAF, and NWDAF for these services can be a potential performance metric for NWDAF Data Collection.

4.4.2.i.2	Description

This proposed performance measurements of the NWDAF are defined as the number of notifications received by DCCF, MFAF, and NWDAF. Since the Data Source and the services used are different, the number of notifications is defined based on the Data Source.

The number of notifications received by NWDAF from NFs for Data Collectioncan be measured as follows:

- When the NWDAF receives the Nnf_EventExposure_Notify service operation from NF (See TS 23.288 [2]), each accepted notifications is added to the relevant counter.

The number of notifications received by NWDAF from AF via NEF for  Data Collection can be measured as follows:

- When the NWDAF receives the Nnef_EventExposure_Notify service operation from NEF (See TS 23.288 [2]), each accepted notifications is added to the relevant counter.

When the DCCF receives a request for data, it determines the status of data collection from the Data Source. If the requested data is not already being collected from a Data Source, the DCCF sends a new subscription/request towards the Data Source with the notification target specified as the DCCF. If the requested data is partially covered by existing subscriptions with the Data Source, the DCCF sends the Data Source a request to modify the subscription. Data Source can be determined by DCCF to be NF instance if the Data Source is not indicated in the Data Consumer's request.The number of notifications received by DCCF from NFs for Data Collectioncan be measured as follows:

- When the DCCF receives the Nnf_EventExposure_Notify service operation from NF (See TS 23.288 [2]), each accepted notifications is added to the relevant counter.

The DCCF may also select an ADRF if the data is to be stored in an ADRF and an ADRF endpoint is not indicated in the Data Consumer's request. The number of notifications received by DCCF from ADRF for Historical Data Collection can be measured as follows:

- When the DCCF receives the Nadrf_DataManagement_RetrievalNotify  service operation from ADRF (See TS 23.288 [2]), each 	accepted notifications is added to the relevant counter.

The number of notifications received by DCCF from NWDAF for Historical Data Collection can be measured as follows:

- When the DCCF receives the Nnwdaf_DataManagement_Notify service operation from NWDAF (See TS 23.288 [2]), each 	accepted notifications is added to the relevant counter.

The data collection procedure using the DCCF supports two options via the DCCF or via MFAF. For the second option, The number of notifications received by MFAFfrom NFs for Data Collectioncan be measured as follows:

- When the MFAFreceives the Nnf_EventExposure_Notify service operation from NF (See TS 23.288 [2]), each accepted notifications is added to the relevant counter.

The number of notifications received by MFAF from ADRF for Historical Data Collection can be measured as follows:

- When the MFAF receives the Nadrf_DataManagement_RetrievalNotify  service operation from ADRF (See TS 23.288 [2]), each 	accepted notifications is added to the relevant counter.

The number of notifications received by MFAF from NWDAF for Historical Data Collection can be measured as follows:

- When the MFAF receives the Nnwdaf_DataManagement_Notify service operation from NWDAF (See TS 23.288 [2]), each 	accepted notifications is added to the relevant counter.

Editor's Note:	This clause provides details of the potential solution and any assumptions made.

End of Modified Sections