3GPP TSG-SA5 Meeting #144-e 	S5-224309

# e-meeting, 27 June - 1 July 2022

# Source:	Microsoft

# Title:	pCR 28.834 Add use case on LCM: configuration of cloud-native VNFs using de-facto standards

# standards

# Document for:	Approval

Agenda Item:	6.8.5.1

# 1	Decision/action requested

The group is asked to discuss and approve.

# 2	References

[1]	3GPP TR 28.834-010 “Study on Management of Cloud Native Virtualized Network Functions”.

# 3	Rationale

This contribution proposes to add the use case and potential requirements on configuration of cloud native VNFs as part of the set of use cases of LCM of cloud-native VNFs using de-facto standards.

# 4	Detailed proposal

It proposes to make the following changes to TR 28.834.

1st Change

# 5	Potential use cases and requirements

Editor's note: this clause will contain the use cases and potential requirements.

## 5.X	Use case# Num: Life cycle management of cloud-native VNFs using de-facto standards

## 5.X.1	Configuration of cloud-native VNF

### 5.X.1.1	Description

Configuration of an NF allows updating the attributes in the MOI and maintaining its operational state. The initial configuration data can be provided as part of instantiation or may be provided later.

| Use Case | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | To enable the authorized consumer to request configuration of a cloud-native VNF using de-facto standards |  |
| Actors and Roles | An authorized MnS consumer of NF provisioning management system requests configuration of a cloud-native VNF. NF provisioning management service producer. |  |
| Telecom resources | 3GPP management system, cloud-native VNF, cloud infrastructure management system |  |
| Assumptions |  |  |
| Pre-conditions | The cloud-native VNF to be configured has been instantiated and its MOI has been created. |  |
| Begins when | An authorized consumer in 3GPP management system needs to configure a cloud-native VNF instance. |  |
| Step 1 (M) | The authorized consumer requests the NF provisioning management service producer to modify the attribute(s) of the MOI of the cloud-native VNF. |  |
| Step 2 (M) | The NF provisioning management service producer configures the 3GPP NF instance, per the MOI attribute modification request received from the consumer. |  |
| Step 3 (M) | The NF provisioning management service producer modifies the attributes of the MOI and informs the consumer that the cloud-native VNF instance has been configured successfully. |  |
| Ends when | Ends when all steps identified above are completed or when an exception occurs. |  |
| Exceptions | One of the steps identified above fails. |  |
| Post-conditions | The instance of 3GPP NF has been configured. |  |
| Traceability | N/A |  |

### 5.X.1.2	Requirements

None.

Editor's note: Use cases for LCM of cloud-native VNFs using ETSI MANO are planned for submission in the next meeting.

Editor’s note: Use cases related to FCAPS are planned for submission in the next meeting.

End of Changes