3GPP TSG-SA5 Meeting #144-e 	S5-224328

# e-meeting, 27 June - 1 July 2022

# Source:	3GPP RAN3

# Title:	Reply LS on Report Amount for M4, M5, M6 and M7 measurements

# Document for:	Information, Discussion

Agenda  6.1

3GPP TSG-RNGAN WG3 Meeting #116-e	R3-224084

E-meeting, 9-19 May 2022

Title:	Reply LS on Report Amount for M4, M5, M6 and M7 measurements

Response to:	S5-214523

Release:	Rel-17

Work Item:	NR_ENDC_SON_MDT_enh-Core

Source:	RAN3

To:	SA5

Cc:

Contact person:	Man Zhang

Zhang(dot)Man4(at)zte(dot)com(dot)cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# 1	Overall description

At RAN3#114-e, RAN3 has introduced the report amount IE for M4, M5, M6 and M7 measurements, based on the reply LS from SA5 (S5-214523) and RAN2 (R2-2108966). The value range of the report amount for M4, M5, M6 and M7 measurements in NGAP/XnAP/E1AP/F1AP are set as that of M1 measurement configuration, i.e., ENUMERATED (1, 2, 4, 8, 16, 32, 64, infinity, …). At RAN3#116-e, the following was further discussed:

- Introduction of report amount in E-UTRAN

RAN3 has discussed whether the report amount for M4, M5, M6, M7 measurements for E-UTRAN should be introduced in Rel-17 but achieved no consensus. The proponents think it is beneficial to introduce the report amount in MDT measurements, not only for NR but also for LTE. So it was proposed to introduce the report amount IE for M4, M5, M6 and M7 measurements in S1AP and X2AP.

RAN3 would like to ask SA5 whether addition of report amount for M4, M5, M6 and M7 is feasible and beneficial in E-UTRAN (like it is for NG-RAN).

# 2	Actions

To SA5

ACTION: 	RAN3 would like to ask SA5 to consider the situation and question above and send feedback to RAN3.

# 3	Dates of next RAN3 meetings

RAN3 #117-e	          15-25 August 2022	       Electronic Meeting

RAN3 #117-bis-e          10-18 October 2022        Electronic Meeting