|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2022-2024 |  |  | SG12-TD123 |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 12 |  |
|  |  |  |  | Original: English |  |
| Question(s): |  | 13/12 |  | Geneve, 7-17 June 2022 |  |
| TD |  |  |  |  |  |
| Source: |  | Editors GSTR-5GQoE |  |  |  |
| Title: |  | Draft new Technical Report ITU-T GSTR-5GQoE: QoE requirements for real-time multimedia services over 5G networks (for agreement) |  |  |  |
| Contact: |  |  | Pablo Pérez Nokia Spain |  | Tel: +34 616 219 675 E-mail: pablo.perez@nokia-bell-labs.com |
| Contact: |  |  | Jaime Ruiz Nokia Spain |  | E-mail: jaime_jesus.ruiz_alonso@nokia-bell-labs.com |
| Contact: |  |  | Diego González-Morín Nokia Spain |  | E-mail: diego.gonzalez-morin@nokia-bell-labs.com |
| Contact: |  |  | Femi Adeyemi-Ejeye University of Surrey United Kingdom |  | E-mail: femi.ae@surrey.ac.uk |
| Contact: |  |  | Kjell Brunnström RISE Research Institute of Sweden Sweden |  | E-mail: kjell.brunnstrom@ri.se |
| Contact: |  |  | Narciso García Universidad Politécnica de Madrid Spain |  | E-mail: narciso.garcia@upm.es |
| Contact: |  |  | Jesús Gutiérrez Universidad Politécnica de Madrid Spain |  | E-mail: jesus.gutierrez@upm.es |
| Contact: |  |  | Maria Martini Kingston University London United Kingdom |  | E-mail: m.martini@kingston.ac.uk |
| Contact: |  |  | Ian Wagdin British Broadcasting Corporation United Kingdom |  | E-mail: ian.wagdin@bbc.co.uk |
| Contact: |  |  | Margaret Pinson National Telecommunication and Information Administration United States |  | E-mail: mpinson@ntia.gov |
| Contact: |  |  | Lucjan Janowski AGH University of Science and Technology Poland |  | E-mail: janowski@kt.agh.edu.pl |

| Abstract: | This Technical Report defines a scope for the analysis of QoE in 5G services and several use cases where this scope is applicable. This is an amended version addressing the comments from Q13/12 session on 13th June 2022 (with change control). This report has been produced by the 5G-KPI working group of the Video Quality Experts Group (VQEG). |
| --- | --- |

|  |  | International Telecommunication Union |  |  |
| --- | --- | --- | --- | --- |
| ITU-T | Technical Report |  |
| TELECOMMUNICATION STANDARDIZATION SECTOR OF ITU |  | (17 June 2022) |
|  | GSTR-5GQoE QoE requirements for real-time multimedia services over 5G networks |  |  |  |

Summary

This technical report defines a scope for the analysis of QoE in 5G services and several use cases where this scope is applicable. Such use cases are: Tele-operated Driving, Wireless Content Production, Mixed Reality Offloading, and First Responder Networks.

Addressing this set of use cases is challenging for three different reasons:

- Their requirements and Quality of Experience (QoE) expectations may be different from the ones typically present in most QoE-related research and Recommendations, which typically address communication services for consumer-type users (e.g., telephony, videoconference, video delivery / streaming, gaming, etc.).

- The experience and expectations of the use case owners may not be applicable to cellular wireless networks, even when QoS policies are applied. E.g., a wireless content production studio will not have the same channel capacity as a wired network, neither from bandwidth nor from reliability points of view. Therefore, totally new impairments or artifacts may appear when moving the use case from wired links to 5G.

- Professional and vertical applications typically have less users than the video consumer ones (there are fewer content producers than content consumers), or the video transmission is just one of the pieces of a much more complex ecosystem (as in the automotive industry).

For each of the services, the document describes:

- Its main characteristics and reference architecture.

- The relevant QoE indicators to be considered on the service.

- A reference implementation, including the order-of-magnitude values of the service Key Performance Indicators, and

- An analysis of the key factors to evaluate the QoE of the service.

Keywords

5G; Tele-operated Driving; Content Production; Mixed Reality; First Responders

Change Log

This document contains Version 1 of the ITU-T Technical Report on “QoE requirements for real-time multimedia services over 5G networks” approved at the ITU-T Study Group 12 meeting held in Geneva, 7-17 June 2022.

| Editor: | Pablo Pérez Nokia Spain | Tel: +34 91 330 4000 Fax: +34 91 330 5000 Email: pablo.perez@nokia-bell-labs.com |
| --- | --- | --- |

CONTENTS

Page

1 Scope 1 2 References 2 3 Terms and definitions 3 3.1 Terms defined elsewhere 3 3.2 Terms defined here 3 4 Abbreviations 3 5 Introduction 4 6 General considerations 5 6.1 5G QoS model 5 6.2 Identifying relevant QoE indicators 7 6.3 QoE assessment for verticals 8 7 Tele-operated Driving (ToD) 10 7.1 Service implementation 11 7.2 Relevant QoE indicators 12 7.3 Key factors to evaluate user QoE 12 8 Wireless Content Production (WCP) 13 8.1 Service implementation 15 8.2 Relevant QoE indicators 16 8.3 Key factors to evaluate user QoE 17 9 Mixed Reality Offloading 18 9.1 Service implementation 18 9.2 Relevant QoE Indicators 20 9.3 Key Factors to Evaluate User QoE 20 10 First Responder networks 21 10.1 Service implementation 22 10.2 Relevant QoE Indicators 24 10.3 Key Factors to Evaluate User QoE 24

List of Tables

Page

Table 1: 5G QoS profile definition 6 Table 2: ToD taxonomy. Source: [5GAA XWG5-200029] 10 Table 3: Service KPIs for Tele-operated Driving 11 Table 4: Relevant 5QIs for Tele-operated Driving 12 Table 5: Service KPIs for Wireless Content production 15 Table 6: Relevant 5QIs for Wireless Content production 16 Table 7: Service KPIs for Mixed Reality Offloading 19 Table 8: Relevant 5QIs for Mixed Reality Offloading 20 Table 9: Service KPIs for First Responder Networks 23 Table 10: Relevant 5QIs for First Responder Networks 24 Table A-1: Uplink and downlink streams weights for scenarios A and B 26 Table A-2: Set of peak throughput and round trip latencies KPIs for the offloading scenarios A and B 26

List of Figures

Page

Figure 1: QoS flows within 5G QoS model [3GPP TS 23.501] 6 Figure 1: Example architecture of a ToD system 11 Figure 2: Example of multi-camera source use case for a live video production [EBU TR 056] 14 Figure 3: Live Audio in Production use case [EBU TR 056] 14 Figure 4: Proposed 5G architecture for MR offloading 19 Figure 5: Elements of First-responder network 21 Figure 6: Demonstration from CONCERTO EU project, emergency services @ Hospital of Perugia, Italy [Martini 2015]. 22

Technical Report ITU-T GSTR-5GQoE

Technical Report ITU-T
QoE requirements for real-time multimedia services over 5G networks

Summary

This technical report defines a scope for the analysis of QoE in 5G services and several use cases where this scope is applicable. Such use cases are: Tele-operated Driving, Wireless Content Production, Mixed Reality Offloading, and First Responder Networks.

Addressing this set of use cases is challenging for three different reasons:

- Their requirements and Quality of Experience (QoE) expectations may be different from the ones typically present in most QoE-related research and Recommendations, which typically address communication services for consumer-type users (e.g., telephony, videoconference, video delivery / streaming, gaming, etc.).

- The experience and expectations of the use case owners may not be applicable to cellular wireless networks, even when QoS policies are applied. E.g., a wireless content production studio will not have the same channel capacity as a wired network, neither from bandwidth nor from reliability points of view. Therefore, totally new impairments or artifacts may appear when moving the use case from wired links to 5G.

- Professional and vertical applications typically have less users than the video consumer ones (there are fewer content producers than content consumers), or the video transmission is just one of the pieces of a much more complex ecosystem (as in the automotive industry).

For each of the services, the document describes:

- Its main characteristics and reference architecture.

- The relevant QoE indicators to be considered on the service.

- A reference implementation, including the order-of-magnitude values of the service Key Performance Indicators, and

- An analysis of the key factors to evaluate the QoE of the service.

# Scope

The scope of this Technical Report is addressing live multimedia services which are provided over wireless IP networks (5G and beyond).

The proposed scope for this Technical Report is addressing services which are required on 5G-and-beyond, either because they are related to vertical use cases, or because they rely on 5G-specific capacities. They should have the following properties:

- Based on IP wireless communication.

- Involve multimedia (mostly video) transmission.

- Require (or, at least, expect) specific support from network features so that they are not expected to be provided over-the-top.

- In particular, they have specific requirements for latency or expectation on timely delivery.

- In most cases, they will be vertical or niche applications, which makes it challenging for them to be covered by a dedicated ITU-T QoS/QoE Recommendation.

For all the targeted use cases, the work item will study the specific QoE requirements, as well as the required performance and features from the network. By addressing them in parallel, it will be possible to find synergies between them and, more relevantly, extract the common information that can be used to also analyze other use cases that may arise outside the scope of this work item

# References

[ITU-T P.10/G.100] Vocabulary for performance, quality of service and quality of experience (2017).

[ITU-T G.1035] Influencing factors on quality of experience for virtual reality services (2020).

[5GAA XWG5-200029] Tele-operated Driving (ToD) – Use Cases and Technical Requirements, 5GAA Technical Report, July 2020

[5GAA T-200xxx] 5GAA White Paper, C-V2X Use Cases Volume II: Examples and Service Level Requirements, Oct. 2020

[5GAA S-200137] 5GAA, Study of spectrum needs for safety related intelligent transportation systems – day 1 and advanced use cases, Jun 2020

[3GPP TS 23.287] 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services (Release 17); V17.1.0 (2021-09)

[3GPP TR 22.827] 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Audio-Visual Service Production Stage 1(Release 17), December 2019

[EBU R 118] Tiering of cameras for use in television production, April 2017

[EBU TR 056] EBU Technical Report, 5G for professional media production and contribution, October 2020

[ITU-R BT.2137-0] ITU-Recommendation, BT.2137-0: Technologies applicable to Internet Protocol interfaces for programme production, December 2020

[3GPP TS 23.501] 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; System architecture for the 5G System (5GS); Stage 2 (Release 17), V17.2.0 (2021-09)

[3GPP TR 26.928] Extended Reality (XR) in 5G; (Release 16); V16.0.0 (2020-03)

[3GPP TR 26.998] Support of 5G Glass-type Augmented Reality / Mixed Reality (AR/MR) devices

[3GPP TS 22.261] 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Service requirements for the 5G system; Stage 1 (Release 17), V18.8.0 (2021-09)

[RDG V4] National Rail and Underground Closed Circuit Television (CCTV) Guidance document

[P. Basilio] High speed cameras for motion analysis in sports science. Journal of Human Sport and Exercise 11.1 (2016): 53-73.

[Perez 2021] Pérez, Pablo, et al. "A parametric quality model to evaluate the performance of tele-operated driving services over 5G networks." Multimedia Tools and Applications (2021): 1-17.

[Villegas 2019] Villegas, A., Pérez, P., & González-Sosa, E. (2019, June). Towards a distributed reality: a multi-video approach to xr. In Proceedings of the 11th ACM Workshop on Immersive Mixed and Virtual Environment Systems (pp. 34-36).

[Dima 2020] Dima, E., Brunnström, K., Sjöström, M., Andersson, M., Edlund, J., Johanson, M., & Qureshi, T. (2020). Joint effects of depth-aiding augmentations and viewing positions on the quality of experience in augmented telepresence. Quality and User Experience, 5(1), 1-17.

[Razaak 2014] Razaak, M., Martini, M.G. and Savino, K., 2014. A study on quality assessment for medical ultrasound video compressed via HEVC. IEEE Journal of biomedical and health informatics, 18(5), pp.1552-1559.

[Martini 2015] Martini, M.G., Iacobelli, L., Bergeron, C., Hewage, C.T., Panza, G., Piri, E., Vehkaperä, J., Amon, P., Mazzotti, M., Savino, K. and Bokor, L., 2015, August. Real-time multimedia communications in medical emergency-the CONCERTO project solution. In 2015 37th Annual International Conference of the IEEE Engineering in Medicine and Biology Society (EMBC) (pp. 7324-7327). IEEE.

[Henry 2021] Henry, M., Villemain, O. and Mertens, L., 2021. Ultrafast Ultrasound Imaging. In Multimodality Imaging Innovations In Adult Congenital Heart Disease (pp. 27-47). Springer, Cham.

[Janowski 2019] Lucjan Janowski; Ludovic Malfait; Margaret H. Pinson, "Evaluating experiment design with unrepeated scenes for video quality subjective assessment," Quality and User Experience, 4, 2 (2019).

[Lévêque 2021] Lévêque, L., Outtas, M., Liu, H., & Zhang, L. (2021). Comparative study of the methodologies used for subjective medical image quality assessment. Physics in Medicine & Biology, 66(15), 15TR02.

# Terms and definitions

## Terms defined elsewhere

This Technical Report uses the following terms defined elsewhere:

3.1.1	Quality of Experience (QoE) [ITU-T P.10/G.100]: The degree of delight or annoyance of the user of an application or service.

3.1.2	QoE influencing factors [ITU-T P.10/G.100]: Include the type and characteristics of the application or service, context of use, the user's expectations with respect to the application or service and their fulfilment, the user's cultural background, socio- economic issues, psychological profiles, emotional state of the user, and other factors whose number will likely expand with further research.

3.1.3	QoE assessment [ITU-T P.10/G.100]: The process of measuring or estimating the QoE for a set of users of an application or a service with a dedicated procedure, and considering the influencing factors (possibly controlled, measured, or simply collected and reported). The output of the process may be a scalar value, multi-dimensional representation of the results, and/or verbal descriptors.

3.1.4	quality of service (QoS) [ITU-T P.10/G.100]: The totality of characteristics of a telecommunications service that bear on its ability to satisfy stated and implied needs of the user of the service.

3.1.5	Tele-operated Driving (ToD) [5GAA XWG5-200029]: a Remote Vehicle (RV) user or operator engaging in the act of driving by taking any of the following roles: dispatcher, indirect controller or direct controller.

## Terms defined here

None

# Abbreviations

| 3D | Three-Dimensional |
| --- | --- |
| 5G | Fifth Generation (of cellular telephony) |
| AR | Augmented Reality |
| AT | Augmented Telepresence |
| CCAM | Cooperative Connected and Automated Mobility |
| CCTV | Close Circuit Television camera |
| DDT | Dynamic Driving Task |
| DL | Downlink |
| DR | Distributed Reality |
| ECG | Electrocardiogram |
| eMBB | Enhanced Mobile Broadband |
| FPS | Frames per second |
| gNB | g-NodeB |
| GOP | Group Of Pictures |
| HEVC | High-Efficiency Video Coding |
| IEM | In-Ear Monitor |
| IP | Internet Protocol |
| KPI | Key Performance Indicator |
| LAiP | Live Audio in Production |
| LAVP | Live Audiovisual Production |
| LTE | Long Term Evolution |
| MCPTT | Mission-Critical Push-to-Talk |
| MEC | Multi-access Edge Computing |
| mMTC | Massive Machine-Type Communication |
| MR | Mixed Reality |
| NPN | Non-Public Network |
| NR | New Radio |
| OBU | On-Board Unit |
| OEDR | Object and Event Detection and Response |
| PTZ | Pan-Tilt-Zoom |
| QoE | Quality of Experience |
| QoS | Quality of Service |
| RGB | Red-Green-Blue |
| ToD | Tele-operated Driving |
| UI | User Interface |
| UL | Uplink |
| URLLC | Ultra-Reliable Low Latency Communication |
| VR | Virtual Reality |
| WCP | Wireless Content Production |
| XR | eXtended Reality |

# Introduction

The fourth generation of mobile networks (LTE) has made it possible to generalize the consumption of video content in mobile. These services are mostly provided over-the-top, using the same internet connection as other applications in the mobile device, since they target end users (individual consumers) with a global outreach.

With the fifth generation of mobile technology (5G), wireless networks increase their capabilities in terms of achieved throughput (enhanced Mobile Broadband, eMBB), latency (Ultra-reliable low latency communication, URLLC), and device density (massive Machine Type Communications, mMTC). But, besides that, it provides improved flexibility to apply these new capabilities to a specific subset of devices, using approaches such as network slicing or non-public networks (NPNs). This opens a new set of video-related services for professional users and vertical industries which, in previous technological generations, were only possible using wired networks, legacy non-IP based wireless links, or even were not possible whatsoever. Examples of those services are Tele-operated Driving (ToD), Wireless Content Production (WCP), or cloud-enabled Augmented and/or Virtual Reality (more generally, Mixed Reality, MR).

Addressing this set of use cases is challenging for three different reasons:

- Their requirements and Quality of Experience (QoE) [ITU-T P.10/G.100] expectations may be different from the ones typically present in most QoE-related research and Recommendations, which typically address communication services for consumer-type users (e.g., telephony, videoconference, video delivery / streaming, gaming, etc.).

- The experience and expectations of the use case owners may not be applicable to cellular wireless networks, even when QoS policies are applied. E.g., a wireless content production studio will not have the same channel capacity as a wired network, neither from bandwidth nor from reliability points of view. Therefore, totally new impairments or artifacts may appear when moving the use case from wired links to 5G.

- Professional and vertical applications typically have less users than the video consumer ones (there are fewer content producers than content consumers), or the video transmission is just one of the pieces of a much more complex ecosystem (as in the automotive industry).

Notwithstanding the characteristics of the professional and vertical use cases, 5G networks also provide the possibility to enhance consumer-oriented services (streaming, gaming, etc.) not only by having higher bandwidth than previous wireless generations, but also by providing added-value functionality such as traffic prioritization, network slicing or edge computing.

With these premises, this Technical Report is addressing services which are required on 5G-and-beyond, either because they are related to vertical use cases, or because they rely on 5G-specific capacities. Addressed services have the following properties:

- Based on IP wireless communication.

- Involve multimedia (mostly video) transmission.

- Require (or, at least, expect) specific support from network features so that they are not expected to be provided over-the-top.

- With specific requirements for latency or expectation on timely delivery.

- In most cases, they will be vertical or niche applications, which makes it challenging for them to be covered by a dedicated ITU-T QoS/QoE Recommendation.

- In most cases, they are task-based applications, performed by a professional or dedicated type of user, with functional requirements, rather than purely aesthetic or perceptual.

For all the targeted use cases, the work item will study the specific QoE requirements, as well as the required performance and features from the network. By addressing them in parallel, it will be possible to find synergies between them and, more relevantly, extract the common information that can be used to also analyse other use cases that may arise outside the scope of this work item.

# General considerations

This section provides background information which apply to all the use cases covered by the document.

## 5G QoS model

The System architecture for the 5G System [3GPP TS 23.501] defines the QoS model used by 5G. The model is summarized in the following figure:

Figure 1: QoS flows within 5G QoS model [3GPP TS 23.501]

In the figure, the UE is the user terminal, AN is the access network (the antenna and base station) and UPF is the part of the 5G core which resolves user data. When a UE connects to the network, it establishes one or several data flows (“PDU sessions”) with one or several UPF entities. “Data packets from applications” are typically IP packets that are exchanged between the UE network stack and the internet (or a private IP network), connected directly to the UPF. The traffic coming from the UE is tunneled across the whole 5G network and it “emerges” to the public internet always at the same point: the UPF.

Each packet of this PDU session belongs to one “QoS Flow”, which is tagged with a QoS Flow Indicator (QFI). There must be at least a default QoS flow in each session, but there could be many. The assignment of IP packets to individual QoS flows is done:

- By “Packet Detection Rules” (PDRs) in the Downlink (network to UE), which are decided by the core.

- By “QoS rules” in the Uplink (UE to network), which must be either explicitly negotiated between the UE and the network or derived by the UE based on received downlink traffic (“Reflective QoS”).

Both PDRs or QoS rules (explicit or reflective) use Packet Filter Sets to assign IP packets to QoS Flows based on IP header fields: source/destination address, port number, protocol ID, Type of Service, etc.

Each QFI is assigned a QoS management policy or “QoS profile”, summarized in the following table.

Table 1: 5G QoS profile definition

| 5G QoS Identifier (5QI) | Resource Type | Non-GBR | GBR | Delay-Critical GBR |
| --- | --- | --- | --- | --- |
|  | Priority Level | X | X | X |
|  | Packet Delay Budget (PDB) | X | X | X |
|  | Packet Error Rate (PER) | X | X | X |
|  | Average Window |  | X | X |
|  | Maximum Data Burst Volume |  |  | X |
| GBR- specific | Guaranteed Flow Bit Rate (GFBR) |  | X | X |
|  | Maximum Flow Bit Rate (MFBR) |  | X | X |
|  | Maximum Packet Loss Rate |  | X | X |
|  | Notification Control |  | X | X |
| Aggregate Maximum Bit Rate (AMBR) | Per Session | X |  |  |
|  | Per UE | X |  |  |
|  | Per Slice | X | X | X |
| Allocation and Retention Priority (ARP) |  | X | X | X |
| Reflective QoS Attribute (RQA) |  | X |  |  |

The definition of the QoS profile is mostly done by the assignment of a 5G QoS identifier (5QI). A 5QI defines the QoS policies which will be applied to the specified QoS flow. It comprises the following components:

- Resource type. Three resource types are identified in 5G: Guaranteed Bit Rate (GBR), Delay-Critical GBR, and non-Guaranteed Bit Rate (non-GBR). As seen in the table, some QoS parameters apply only to specific resource types.

- The total delay (PDB) and error rate (PER) allowed for the flow in the whole 5G network.

- The average time window used to compute bit rate statistics (for GBR flows).

- The maximum volume of data that is expected (and allowed) in each burst.

5QIs can be defined explicitly for each flow. However, the [3GPP TS 23.501] defines some standard values for different types of services. In this document, the standard 5QIs which apply to each of the use cases under study will be presented.

GBR flows require additional definition beyond its 5QI: the service QoS guarantees in terms of bitrate (GFBR) and packet loss rate, as well as the maximum bitrate allowed in the flow (MFBR). Besides, the Notification Control specifies how to handle the situation when the 5G network is not able to fulfil the guaranteed QoS conditions.

Finally, some additional QoS parameters are defined:

- AMBR, to limit the maximum bitrate that a session, UE or slice can use (to prevent individual users to overuse the shared resources of the network).

- ARP, which specifies the priority to decide whether a new QoS Flow may be accepted or needs to be rejected, as well as which existing QoS Flow to pre-empt, in the case of resource limitations.

- RQA, to indicate whether Reflective QoS is enabled for a (non-GBR) QoS Flow.

## Identifying relevant QoE indicators

Most Recommendations involving auditory, visual, or audiovisual quality aim at modelling or predicting the quality of experience perceived by the user. This model may involve analysing the audiovisual signal involved, either as perceived by the user or on its encoded form (bitstream); or just analysing the interaction of the audiovisual communication with the network quality of service (QoS). For the purpose of this document, two sets of relevant recommendations can be considered:

- Planning models. These models allow dimensioning the network, by estimating the expected QoE that can be achieved given generic characteristics of the content (e.g. resolution, codec, compression bit rate) and the network (e.g. throughput, loss rate, etc.). Examples are ITU-T P.107, and P.1070-1072 series.

- Monitoring models. These models allow predicting the subjective quality as perceived by a user from the actual bitstream that would be received by the user device, either in compressed or in decoded format. Examples are ITU-T P.1201-1204 series.

Those models are built on:

- A target QoE indicator representing the subjective opinion of the users about the system. This QoE indicator is typically evaluated in terms of the Mean Opinion Score (MOS) of a panel of users evaluating the system under test.

- A set of technical factors which influence on the QoE indicator. Those factors are related to

  - Implementation of the system (e.g. image resolution, codec).

  - Restrictions on the communication channel, i.e. communication QoS indicators: throughput, loss rate, latency, etc.

- A mathematical model which relates variations in the technical factors (causes) with variations in the QoE indicator (effect).

When analyzing other types of services, especially the ones related to professional usage and vertical use cases, additional considerations are needed.

First, the basic QoE indicator is normally not a MOS value rating aesthetic quality, but an indicator of the effectiveness of the system for its designed task. Each use case will have a different key quality indicator (KQI). Identifying this QoE indicator is key to be able to properly build a QoE analysis of the system.

Understanding the system implementation is also key for a proper analysis of the QoE. The systems covered by many Recommendations comprise a very limited of potential systems: voice calls on telephones, video consumption on screens, and alike. However, vertical use cases may have much different setups, involving non-conventional streams (e.g. control signals) and devices (e.g. vehicles).

Restrictions on the communication channel still apply. A relevant difference with respect to over-the-top communication is that the 5G network may guarantee certain levels of QoS, according to the model described in the previous section. In this sense, network planning models may be more relevant than network monitoring ones, unlike in over-the-top scenarios.

Besides, there is an additional set of relevant technical factors involving energy consumption, from two different perspectives. On the one hand, the energy efficiency on mobile devices (bits per joule) affects battery life, which might be more relevant than other performance indicators for mobility use cases. On the other hand, there is trade-off between the complexity of the algorithms used to implement each use cases (for compression, scene understanding, video processing, etc.) and the performance they offer. And the complexity is also related to the energy consumption of those algorithms, either on mobile or on infrastructure computing power. Therefore, there is an energy-distortion trade-off that needs to be considered in the evaluation of the use cases.

Finally, it is worth noting that the development of mathematical models for QoE, either for planning or monitoring purposes, is out of the scope of this document. Some QoE models have been proposed by the research community for some of the use cases, and they will be reference where appropriate. Besides, this document should serve as guidelines to develop network planning and monitoring strategies for the mentioned use cases.

## QoE assessment for verticals

The objective for QoE assessment is identifying how the variation of different conditions, either technical (e.g. video resolution or bitrate) or non-technical (e.g. the type of task) can affect the overall QoE. For this, subjective tests are performed to assess the response of a panel of subjects to the variation of such conditions.

QoE assessment should be done by the type of subjects that are typically going to use the system. From the perspective of subjective evaluation, subjects can be classified into (based on [ITU-T P.805]):

- Untrained subjects (naïve). They are neither experienced in subjective testing methodology, nor are they experts in technical implementations of the system under test. They should have not participated in any subjective test in the previous 6 months.

- Application experts. They are experienced in subjective testing and they are able to describe their subjective impressions in detail. However, they neither have a background in technical implementations of the system under test, nor do they have detailed knowledge of the influence of these implementations on subjective quality

- System experts. They have a background in technical implementations of the equipment under test and do have detailed knowledge of the influence of particular implementations on subjective quality.

In most scenarios described in this document, users will have deep knowledge of the use case, and therefore they will be system experts (if they also have knowledge of the underlying technical implementation) or application experts (otherwise).

In most scenarios, the relevant QoE indicator will not be a perceptual property, but a functional one: driving safety, medical situation evaluation, etc. This requires that the evaluation is done by the proper type of users (application or system experts), and the evaluation methodology reflects the actual use case.

Regarding the evaluation methodologies, there are two possible approaches:

- Active tests: create an evaluation scenario where the actual task is performed.

- Passive tests: evaluate pre-recorded videos of content which is relevant to the use case.

Active tests must be specifically designed for each use case by subject matter experts. The exact definition of the task is outside the scope of this document. However, several guidelines can be taken from existing ITU-T Recommendations, for different types of tasks:

- Recognition tasks [ITU-T P.912]. This is appropriate methodology when there is a question whose answer can be objectively considered as correct or incorrect. The recommendation provides guidelines to several types of tasks, including how to perform statistical analysis of the results:

  - Multiple choice. The video is shown above a list of verbal labels representing the possible answers.

  - Single answer. There is an unambiguous answer to an identification question, e.g. alphanumeric character recognition.

  - Timed task method. A viewer may be asked to watch for a particular action or object to be recognized in the video clip. When the viewer perceives that the target has occurred, a timer button can be pushed.

- Conversational tasks are covered by several recommendations: [ITU-T P.805] for voice conversations, [ITU-T P.920] for audiovisual conversations, [ITU-T P.1312] for multiparty voice telemeetings.

- Interactive tasks can be based on [ITU-T P.809], design for the evaluation of interactive game scenes. Two types of test stimuli are considered:

  - Short interactive (90-120 seconds). It is possible to assess the interaction quality (e.g., the impact of delay on the control), but the assessment of more complex player experience features, highly depends on the player and the game content

  - Long interactive. It is reasonable to use a duration of 10-15 minutes to ensure that players get emotionally attached to a game scenario while aiming to measure emotions and other QoE aspects such as flow.

It is important to mention that these active tests Recommendations will probably not cover completely the requirements of the use case that need to be tested. They can be used as guidelines to develop a subjective evaluation methodology for the use case, but other important aspects may need to be evaluated which exceed the scope of such Recommendations. Once more, a deep knowledge of the use case is required to

Passive tests should be based on conventional multimedia evaluation methodologies such as [ITU-T P.913]. The selection of source content and added impairments should be representative of the actual use case (e.g. a ToD test should use content captured from a vehicle under a representative set of driving conditions).

When characterizing a network for a particular video application, realism is critical and camera impairments are a major constraint. The subjective test must include sufficient variety of subject matter and environmental conditions. This can be done by using unrepeated scene experiment design as described in [Janowski 2019]. For instance, an unrepeated scene experiment design can characterize the full range of camera impairments; a conventional matrix experiment design, where all combinations of source sequence and impairments are tested, cannot.

The result of passive tests must be taken with proper precaution. Application and system experts tend to provide severe judgement in the evaluation of systems with passive tests. However, in practice, they can adapt to bad quality conditions and still perform the desired task.

Nonetheless, passive tests are useful to understand the effect of the different technical factors into the perceived QoE, as well as to prototype active tests.

# Tele-operated Driving (ToD)

The availability of 5G networks is becoming a reality, and it brings the opportunity to support the deployment of new Cooperative Connected and Automated Mobility (CCAM) services. One of the most challenging CCAM uses is Tele-operated Driving (ToD). ToD can be seen as a side effect of tackling with potential issues that autonomous driving cannot solve by itself. In this case human intervention might be required to drive the car. Such intervention can be feasible by making use of faster 5G infrastructure, a dedicated protocol, video data channels, and cockpit setup under the supervision of a control center.

Depending on the level of implication of the remote operator in the act of driving, different ToD types may be defined (Table 2) [5GAA XWG5-200029]. For the purpose of this document, the types under consideration are the ones where part of or all of the dynamic driving tasks are performed by a remote driver on a sustained basis (ToD Type 2 or 3).

Table 2: ToD taxonomy. Source: [5GAA XWG5-200029]

| ToD Type (Role of ToD operator when engaging in the act of driving) | Act of Driving |  |  |
| --- | --- | --- | --- |
|  | Strategic Operation (Travel planning, route and itinerary selection) | Dynamic Driving Task (DDT) |  |
|  |  | Tactical Operation (Object and Event Detection and Response OEDR) | Operational Operation (Sustained lateral and longitudinal vehicle motion control) |
| 0 (No Role) | In-vehicle user or system | In-vehicle user or system | In-vehicle user or system |
| 1 (Dispatcher) | ToD operator | In-vehicle user or system | In-vehicle user or system |
| 2 (Indirect Controller) | ToD operator | ToD operator | In-vehicle user or system |
| 3 (Direct Controller) | ToD operator | ToD operator | ToD operator |

Although several implementations of the ToD service are possible, a typical reference architecture assumes that there are four cameras boarded on the vehicle, one at each side, as well as other sensors (positioning system, radar, lidar, etc.). The information coming from those sources is processed by an On-Board Unit (OBU) and sent through a 5G network to the remote driver cockpit. The cockpit has several screens, or maybe a virtual environment using VR, where the video from the cameras is displayed, as well as the information from the sensors. Besides, the cockpit contains remote control (driving wheel, throttle) whose information is also sent back to the car.

Figure 2: Example architecture of a ToD system

## Service implementation

Typical communication performance requirements, for vehicle speed < 50 km/h, are described in Table 3 [5GAA T-200xxx, Perez2021]. Service latency is critical, and it covers the video and data stream (UL) and the messages from the ToD operator to the Host Vehicle (DL). Uplink data rate accounts for 1 to 4 cameras, plus additional telemetry data. Note that UL latency includes video encoding + decoding delay, which might be a significant part of the end-to-end latency budget.

Table 3: Service KPIs for Tele-operated Driving

| ToD Type | Latency (UL) | Latency (DL) | Data Rate (UL) | Data Rate (DL) |
| --- | --- | --- | --- | --- |
| 2 | 100-200 ms | 200 ms | 4-36 Mbps | 25 kpbs |
| 3 | 100-200 ms | 20-100 ms | 4-36 Mbps | 400 kbps |

It is assumed that those services are going to be provided on dedicated spectrum. Due to the requirements for throughput, density, and coverage, they will probably need to be deployed on the mid bands, allocating a minimum of 30 MHz for urban areas, and up to 100 MHz for rural areas [5GAA S-200137]. Specific enhancements of 3GPP standards are being proposed to support this, as well as other V2X scenarios [3GPP TS 23.287].

Table 4 shows the relevant 5QIs for ToD services, from the ones provided in [3GPP TS 23.501]. The uplink, whose most relevant information is video, should use the 5QIs proposed for real-time video transmission: 2, 3, or 7. The downlink, where the information is mostly related to control commands, should use V2X messaging 5QIs: 79 or 85.

Table 4: Relevant 5QIs for Tele-operated Driving

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | GBR | 40 | 150 ms | 10-3 | N/A | 2000 ms | Conversational Video (Live Streaming) |
| 3 |  | 30 | 50 ms | 10-3 | N/A | 2000 ms | Real Time Gaming, V2X messages |
| 7 | Non-GBR | 70 | 100 ms | 10-3 | N/A | N/A | Video (Live Streaming) Interactive Gaming |
| 79 |  | 65 | 50 ms | 10-2 | N/A | N/A | V2X messages |
| 85 | Delay-critical GBR | 21 | 5 ms | 10-5 | 255 bytes | 2000 ms | V2X messages (Remote Driving) |

## Relevant QoE indicators

Tele-operated Driving is a task-based media experience, whose quality is mostly evaluated through the ability of the remote driver to safely control the vehicle. Safety is therefore the most relevant driver for ToD QoE.

More in detail:

- Video quality and latency have safety impacts on ToD with human remote operators.

- The relation between video quality and ToD performance is complex and should be studied.

- Video quality and latency must be addressed as part of the functional safety standardization.

- Different ToD types may have different video quality requirements.

- There are strong influences from the environments and the vehicle speed. For instance, it is not the same to deploy a tele-operated valet parking than to be able to perform ToD on a highway.

The relationship between network throughput, latency and losses can be used to build a parametric planning model for ToD, such as the one proposed in [Perez2021].

## Key factors to evaluate user QoE

Two kinds of factors are critical to evaluate the QoE for ToD services: technical factors, regarding the specificity of the data streams that are used for the tele-operation; and human factors.

From the technical perspective, the main components of the final QoE are:

- The end-to-end latency, including coding and network delays.

- The quality of the video signal from the cameras, including coding quality and the effect of transmission errors.

- The quality of the sensors and control signals, especially the potential effect of transmission errors.

The end-to-end latency is critical for a safe ToD execution. The tolerance to latency is directly related with the driving speed and the safety distance between the vehicle and any possible obstacle or hazard. The higher the latency, the lower the maximum speed and the higher the required safety distance.

Three factors are relevant for the evaluation of the video quality for a given camera: the speed of driving, the camera location and the current manoeuvre in which the car is involved. The speed of driving affects the video quality because differences between one frame and the next increases with speed, so the compression algorithms requires more bandwidth to keep the same video quality. Camera location is also critical for quality as lateral cameras are less relevant for some type of driving and also present more differences between one frame to the next compared with the front camera, so we need to consider the relevance of the camera information and the quality that can be provided for a given bitrate. Finally, the manoeuvre sometimes imply that laterals or rear cameras become more relevant in some scenarios.

The quality of the rest of the data streams (sensors and actuators) will be strongly dependent on implementation on each ToD system.

Regarding the subjective assessment of QoE (human factors), there are two elements to be considered:

- Evaluation of ToD systems must be based on a task-oriented methodology. I.e., the relevant quality is not related to aesthetics or perceptual fidelity, but with the ability of the remote driver to execute the ToD task in a safe manner.

- The subjective experience of the driver will be influenced by the operation cockpit. Different options (e.g., multi-screen vs VR implementations) may show different QoE results for a similar task and signal quality.

# Wireless Content Production (WCP)

Wireless content production covers several uses cases and broadly falls under the following categories: newsgathering, live broadcast of events, live audio in production and non-live production (media file transfer). Non-live production will not be covered by this document.

Newsgathering is vital for broadcast companies worldwide, offering news coverage of any kind of event that may interest the public. This includes planned events such as politics and economy or disasters that occur without prior notice and therefore cannot be planned, and reporters may have to capture audio and video data to be sent back to production facilities as they happen. Typically, a single camera or multiple cameras simultaneously feedback to a curator in a central production facility, depending on the circumstance [3GPP TR 22.827, EBU TR 056].

Live audio-visual production (LAVP) involves capturing and distributing both video and audio data to an audience in real-time. There are different scales of live video production. These range from using mobile handsets on social media to live contribution using a single camera up to the typical multi-camera broadcast of a high-profile event (including outside broadcast). 5G is one of the enablers that can add value to a production budget. For example, in remote live TV production, wireless cameras and microphones could be deployed in a political chamber to provide an automated feed of a meeting or a small cultural venue to cover an arts event. While in large sporting and cultural events such as the football matches or musical concerts, where many cameras are needed, you can have a mix of both wireless and wired cameras, wireless microphones, telemetry and remote control. Currently most of these sources will be mixed locally on site, but in both cases it is desirable that captured data is sent to a data centre or studio where live content selection can occur, and 5G might offer a reliable connection [3GPP TR 22.827, EBU TR 056].

Figure 3: Example of multi-camera source use case for a live video production [EBU TR 056]

Live audio in production (LAiP) for events such as concerts or theatrical events where one or more artists perform before an audience. Producing and capturing of a live event for live programme audio or subsequent use of the cultural and creative content, involve many wireless audio streams.  The technical crew, the production team, and the security staff are normally backstage ensuring the successful realization of the live event and are normally connected via an intercom system using microphones, loudspeakers and in-ear monitoring. For the live audio content, reception of the wireless communication service is only required locally, limited to the event area, and all audio processing such as audio mixing is done in real time during operation. A number between 5 and 300 simultaneously active wireless audio links can be expected. Each wireless audio source is streamed to or from a central audio mixing console. Therefore, the setup and local network must be fast to optimize the usage of resources (equipment, radio spectrum, working time of the artists and crew) [3GPP TR 22.827, EBU TR 056].

Figure 4: Live Audio in Production use case [EBU TR 056]

## Service implementation

Wireless Content Production services are provided by connecting different media production elements (microphones, IEMs, cameras, etc.) to the 5G network. A wide range of implementations is possible, starting from connecting the system to the commercial mobile network (a typical case for newgathering) to using dedicated slices or non-public networks (NPNs).

Table 5 shows a set of KPIs for the different use cases. When applicable, it is based on a reference configuration using NR midband (3.8 GHz) with 100 MHz of bandwidth, which would result in around 120 – 200 Mbps of uplink [3GPP TR 22.827]. This limits the maximum UL throughput that can be considered for the service.

In this respect, for LAVP, codec decision is critical. I-frame-only could exceed target data rate, IBBP is too much latency, so a form of IPP is probably required. The impact of network errors on the video stream is dependent on codec decision and group of pictures (GOP) length, since packet sizes are constant [ITU-R BT.2137-0].

For LAiP, the quality of the audio is dependent on different user data rates per audio stream need to be supported for different audio demands (e.g. compressed vs. uncompressed audio). E.g. 5 Mb/s is a target maximum desired for studio usage (24 bit @ 192 kHz).

A relevant KPI for all services is end-to-end latency, even though requirements vary strongly between services. For newsgathering, it is enough to have an audio return channel from the news curator allowing 2-way conversation. For LAVP, low and consistent latency is critically, so that constant frame rate can be achieved, as well as frame-level synchronization among all the cameras. In LAiP, latency requirements are even more strict to guarantee minimum mouth-to-ear delay for the performers.

Table 5: Service KPIs for Wireless Content production

| Use case | Number of UEs | Latency (RTT) | Synchro / jitter | Data Rate per UE (UL) | Data Rate per UE (DL) | PER |
| --- | --- | --- | --- | --- | --- | --- |
| News Gathering | 1 | 1000 ms | 100 ms | 15-50 Mbps | 0.1-0.3 Mbps | 10-6 |
| LAVP | 1-10 | 100 ms | 6 ms | 80-200 Mbps | 20 Mbps | 10-8 |
| LAiP | 5-300 | 4 ms | 0.01 ms | 0.1 – 5 Mbps | 0.1 – 5 Mbps | 10-6 |

Table 6 shows the relevant 5QIs for WCP services, from the ones provided in [3GPP TS 23.501]. Depending on the specific use case, different 5QIs will apply. News gathering will use less restrictive QoS requirements for the uplink, e.g. the ones defined for non-conversational video (4, 8, 9), while requiring voice only for the downlink (e.g. 1). LAVP has more demanding QoS requirements, mostly due to the latency limits. 5QIs designed for conversational video or gaming could be applied. Finally, LAiP requirements are so far not supported by any standardized 5QI.

Table 6: Relevant 5QIs for Wireless Content production

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GBR | 20 | 100 ms | 10-2 | N/A | 2000 ms | Conversational Voice |
| 2 |  | 40 | 150 ms | 10-3 | N/A | 2000 ms | Conversational Video (Live Streaming) |
| 3 |  | 30 | 50 ms | 10-3 | N/A | 2000 ms | Real Time Gaming |
| 4 |  | 50 | 300 ms | 10-6 | N/A | 2000 ms | Non-Conversational Video (Buffered Streaming) |
| 6 | Non-GBR | 60 | 300 ms | 10-6 | N/A | N/A | Video (Buffered Streaming) |
| 7 |  | 70 | 100 ms | 10-3 | N/A | N/A | Voice, Video (Live Streaming) Interactive Gaming |
| 8 |  | 80 | 300 ms | 10-6 | N/A | N/A | Video (Buffered Streaming) |
| 9 |  | 90 |  |  |  |  |  |

## Relevant QoE indicators

Wireless Content Production covers a set of use cases where connectivity between production devices and the production centre is done via a 5G network. In most cases, it replaces (or complements) previous solutions, based either on wired connectivity or on ad-hoc wireless technologies. Using 5G mobile technology increases the flexibility of the system, making it possible to deploy production pipelines in locations where it was not possible before, or with a more cost-effective solution. In such cases, this may be at expenses of the audiovisual quality. Balancing these factors is quite critical for the QoE evaluation of these services.

For instance, connectivity is the most important factor for newsgathering QoE. Another factor is the quality of the captured audio and video data. However, connectivity takes priority over the visual aesthetics, and therefore, quality compromises can be made. Latency is also a critical factor with 2-way communications needed between studio and location services. Increasing there is a need to control remote contribution facilities and this require lower latency connectivity. Ideally latency should be fixed and not drift over time and be able to be synchronized across multiple sources.

In Live Audiovisual Production, quality is evaluated through ability of the production team to deliver a visual experience for broadcast, based on the programme’s running order. This can be impacted by the following:

- Video quality and latency from capture sources.

- Multi camera feeds synchronization. Camera feeds have to be synchronized to ensure switching and /or composing mismatches.

- Different LAVP implementations may have different video quality requirements. Typical wired solutions use uncompressed video but where wireless links are used they need to meet minimal contractual quality as set out by broadcast regulators.

- Camera accessories such as lens also have an impact on video quality.

- Often there is a contractual or legal obligation to provide a level of quality. Broadcasters can be penalized for breaking these obligations and may lose future rights if the quality of the broadcast is not seen to be high enough by a rights holder.

In LAiP, the two most critical requirements are reliability of connection and mouth-to-ear latency between microphone and in-ear monitoring (IEM) equipment used by the artists or presenters. The tolerance for disturbance to QoE for this application is extremely low as there is no opportunity of recovery (e.g. no possibility to ask the singer to repeat).

## Key factors to evaluate user QoE

The two main factors to evaluate for the QoE of wireless content production services: technical factors, and human factors.

From the technical perspective, the main components of the final QoE are:

- Reliable connectivity.

- Audio and video signal quality.

- For LAVP: low and, more importantly, predictable end-to-end latency and multi-camera synchronization.

- For LAiP: extremely low mouth-to-ear latency.

Since Wireless Content Production covers live content for professional use, a reliable connectivity is the most critical element. Fast and efficient deployment of communication links are crucial for newsgathering. This is directly related to the location of the audio and video capture and the connectivity within the area. So reduced bandwidth links should be taken into consideration, to emulate areas with poor reception capacity. For LAVP and LAiP, reliability is particularly crucial for the successful coordination of activities of different members of the production.

Video and audio quality are also critical, particularly for LAVP and LAiP. In the case of newsgathering, since connectivity is crucial, quality compromises can be made using compression algorithms to ensure the video can be easily transmitted using the bandwidth available for upload at the location of capture

As we move to IP based production solutions there are also requirements for return path AV at lower bandwidth and control signals such as tally or PTZ control.

To evaluate the video quality for a given camera, the following factors should be considered: image sensor size, sensor resolution, video raster resolution, frame rate, digital signal processing and optical path. While the image sensor size is fixed on any given camera, the other factors can be easily changed and thus affect the perceptual quality of the resulting video. Objective metrics are desirable to avoid the costs of subjective assessments and/or expert viewing. These are defined for Europe in [EBU R 118].

In a multi-camera feed setup, capture to production latency will have a direct impact on audio-visual and camera synchronization and this could lead to audio and video mismatch, multiple feed mismatch and compositing artefacts.

In a live production, the end-to-end latency that can be tolerated by a live performer between their microphone and IEM typically ranges from 0.125 to 2 ms and cannot exceed 4 ms.

Regarding the subjective assessment of QoE (human factors), there are two elements to be considered:

- The QoE of the system is provided to content production professionals: news curators, video producers, performers, etc. Therefore, QoE evaluation should be done by such professionals.

- Although conventional quality evaluation methodologies can be of use (e.g., ITU-T P.800, P.910, P.913, etc), a task-oriented methodology could provide better ecological validity, e.g.

  - The ability of the remote curator to weave a storyline from audio and video data received, from new sources to fit in with other parts of the editorial workflow.

  - A live vision engineer confirming the quality of the incoming video feed before passing on to be made available for selection in a live broadcast by editorial production staff.

  - The ability of the members of the production team and live performers to be able to communicate effectively

Besides, objective metrics would be desirable to avoid the costs of subjective assessments and/or expert viewing.

# Mixed Reality Offloading

The fast-paced development of Mixed Reality (MR), composed by Augmented Reality (AR) and Virtual Reality (VR) technologies, have risen the computation demands of MR applications. High rendering resolution is not enough to provide the users with fully immersive experiences: other novel algorithms such as hand tracking, occlusion handling or object recognition are becoming fundamental for both AR and VR. However, robust examples of such algorithms require high-end GPU-enabled computing platforms which hinder the goal of reducing the size and weight of current state-of-the-art AR/VR devices. The global deployment of 5G networks enables the chance of offloading some or all of these heavy-duty algorithms to a Multi-Access Edge Computing (MEC) platform. Successful MEC-delivered AR/VR can lead to lighter, more comfortable and more affordable devices.

Mixed Reality Offloading is then a solution which, using 5G-network capabilities, enables running state-of-the-art MR algorithms while wearing a lightweight head-mounted-display (HMD) –an Edge-dependent User Equipment (UE) [3GPP TR 26.998]. This approach is also called XR (eXtended Reality) Distributed Computing [3GPP TR 26.928].

Mixed Reality Offloading assumes that the immersive environment where the application takes place is either the real world around the user (AR) or a completely virtual environment (VR). There is an alternative scenario where the immersive environment is a real-time capture of a remote location, sent via a 360-degree video stream or a similar means, onto which virtual elements are added. This scenario, called Augmented Telepresence (AT) [Dima 2020] or Distributed Reality (DR) [Villegas 2019], is not covered by this technical report.

## Service implementation

AR/VR applications run at rates of 60 Hz and above. These rates set severe latency constraints that must be fulfilled to avoid visual artifacts or user’s simulator sickness that can completely ruin the experience. Processing latencies have different effects in AR and VR:

- In AR, processing latencies lead to virtual objects to drift, having incorrect positions and dynamics. Besides, AR demands synchronized coherence between the real and virtual worlds.

- In VR these latencies lead to very severe simulator sickness which might force the user to leave the experience. In some VR applications in which the real world is incorporated (e.g., users’ hands are segmented and rendered in the virtual scene) these latencies also produce incorrect drifts between the real and virtual content.

Ideally, the maximum delay between the sensor capture process and the rendered frame should be bounded to the frame update period (16.6 ms for 60 Hz). Besides, recent MR devices incorporate several high-definition cameras, including both RGB and depth feeds, which generate a wide stream of sensor data. Consequently, any successful MEC-delivered AR/VR will require a careful network design to fulfil with the latency and data rates. We propose a set of KPIs from a thorough mathematical analysis using two different scenarios for both AR and VR.

- Full offloading scenario: the sensor feed (RGB and depth) is streamed to the MEC and processed. The new ultra-high-definition frame is rendered, sent and displayed on the device.

- Algorithm offloading scenario: a machine-learning-based heavy-duty algorithm is offloaded from the device. In this scenario, part or full of the sensor feed is streamed to the MEC and processed, and the result is sent back to the device. The result is used to render the new frame. For simplification, we consider this result to be a low-definition segmentation mask, common in scene recognition or hand segmentation algorithms.

Table 7 shows the KPIs for both scenarios. More detail on the derivation of such KPIs is presented in Annex A. Note that KPIs are aligned with the ones described in [3GPP TS 22.261].

Table 7: Service KPIs for Mixed Reality Offloading

| Scenario | Latency (RTT) | Average Data Rate (UL) | Average Data Rate (DL) | Peak Data Rate (UL) | Peak Data Rate (DL) |
| --- | --- | --- | --- | --- | --- |
| A | 2-3 ms | 180 Mbps | 1000 Mbps | 1250-1700 Mbps | 2800-3000 Mbps |
| B | 3.5-6 ms | 180 Mbps | 18 Mbps | 750-900 Mbps | 250-300 Mbps |

The estimated KPIs impose tight demands which require a well-designed configuration. The latency requirements impose the MEC to be placed as close to the gNB as possible, avoiding any backhauling connectivity. User prioritization is also a key component in a successful AR/VR offloading architecture: packets should be allocated as soon as they become available to provide high-throughput and low latency transmissions. Besides, the gNB should be configured with at least a subcarrier spacing of 120 KHz and a bandwidth of 400 MHz. Consequently, high-frequency bands are the most suitable for the proposed offloading scenarios: mmWave is a key enabler in MEC-enabled AR/VR. Figure 5 shows a possible network architecture for MR offloading.

Figure 5: Proposed 5G architecture for MR offloading

Table 4 shows the relevant 5QIs for mixed reality offloading, from the ones provided in [3GPP TS 23.501]. As Mixed Reality is one the critical drivers for 5G, specific 5QIs have been defined (80, 87-90). It is worth noting that, even if they are the most restrictive ones so far defined in terms of packet delay budget, they might be insufficient to fulfil the requirements of the scenarios as we have defined them.

Table 8: Relevant 5QIs for Mixed Reality Offloading

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 80 | Non-GBR | 68 | 10 ms | 10-6 | N/A | N/A | Low Latency eMBB applications Augmented Reality |
| 87 | Delay-critical GBR | 25 | 5 ms | 10-3 | 500 bytes | 2000 ms | Interactive Service - Motion tracking data |
| 88 |  | 25 | 10 ms | 10-3 | 1125 bytes | 2000 ms | Interactive Service - Motion tracking data |
| 89 |  | 25 | 15 ms | 10-4 | 17000 bytes | 2000 ms | Visual content for cloud/edge/split rendering |
| 90 |  | 25 | 20 ms | 10-4 | 63000 bytes | 2000 ms | Visual content for cloud/edge/split rendering |

## Relevant QoE Indicators

The QoE in both AR and VR is measured in terms of the level of immersion into the virtual experience [ITU-T G.1035]: how the augmented objects are perceived as part of the real-world experience (AR) or, similarly, how the user feels fully immersed in the virtual environment (VR).

However, the immersion is related to several aspects of the overall AR/VR experience:

- The quality and resolution of the rendered frames or augmented elements directly affect the immersion.

- The responsiveness during the interaction with virtual objects and 3D UIs is tightly related to the quality of experience. This indicator is crucial in the applications in which the user has to accomplish a specific task.

- The correct alignment and relative positions of the virtual and real objects, including the correct resolution of occlusions, is a key factor of AR applications.

- The sense of embodiment, which measures how the user perceives himself within the virtual experience, is a key factor of any VR application.

- Motion-to-photon latency is another key indicator for the quality of the experience.

- The manner each of these metrics affect the experience is tightly related to the specific use case or application.

## Key Factors to Evaluate User QoE

The correlation between the network KPIs and the user QoE can be roughly achieved by directly measuring the performance of the network. Network KPIs can serve as proxy to estimate certain factors of the end user's QoE.

Two-way end-to-end latency is the most critical technical factor influencing MR Offloading QoE. Latencies above a modelled threshold indicate:

- High motion-to-photon latency, which causes simulator sickness and degrades considerably the experience in both AR and VR.

- Low responsiveness in human-virtual interaction.

- Decreased feeling of embodiment in the cases in which the hand/body tracking or segmentation algorithms are offloaded.

End-to-end latency is influenced by network round-trip-time, peak uplink and downlink throughputs, and coding and processing delay.

Additionally, mean network throughput directly affects the number and quality of visual elements that are rendered in the MEC and streamed to the end device.

Besides, the level of QoE degradation is tightly correlated to the type of MR application. To estimate a highly detailed QoE-KPIs correlation, is necessary to evaluate the QoE indicators in a subjective manner. The relation between the QoE indicators and the network KPIs is application dependent and so it is the evaluation methodology. As an example, the QoE indicators might not be the same in an industrial training task than in an immersive video game.

# First Responder networks

A relevant use cases of 5G networks is the ability to provide connectivity to first responders to attend emergency situations. Whenever a medical or security emergency happens, an effective connectivity and coordination of the different teams involved may make the difference to save lives. To support the work of the emergency services, this communication should be provided by dedicated network resources, either specific frequency bands or prioritized network slices.

Figure 6 shows an example of first responder network. In general, an emergency situation requires the coordination between on-site teams (e.g. first responders in an ambulance), infrastructure elements (e.g. CCTVs) and central stations (e.g. the hospital). Different kinds of first responders and public safety officers may have different requirements, depending on the emergency scenario. For instance, attending a traffic accident, a natural disaster or a criminal threat will involve different personnel and systems.

For the purpose of this document, we will use two specific examples:

- On-site medical emergencies: communication between an ambulance and the hospital.

- Infrastructure elements: deployment of CCTVs for live monitoring of an area (e.g. railway infrastructure).

Figure 6: Elements of First-responder network

In medical emergency multiple data flows are transmitted from emergency site/ambulance to the hospital (uplink). Typically voice from emergency room staff or medical specialists in the hospital guiding operation can be transmitted in the downlink. Video can be used for the downlink to visually show paramedics how specific operation should be performed. Access to medical records in the hospital or National Health System may be required (also downlink).

The figure below (from a demonstration from the CONCERTO European project) illustrates the different sources of data flows in a connected ambulance.

Figure 7: Demonstration from CONCERTO EU project, emergency services @ Hospital of Perugia, Italy [Martini 2015].

Closed Circuit Television (CCTV) systems are used on rail infrastructure including inter-city rail or metro rail stations, on trains and at car parks [RDG V4]. They are usually deployed for a variety of reasons including:

- crime prevention and prosecution

- investigation of rail related accidents and incidents

- to reassure staff and members of the public

- to aid decisions on train movements

- to meet statutory requirements and obligations.

5G is one of the enablers that can add value to CCTV systems and workflows. For example, wireless CCTV cameras can be easily deployed in various locations of the rail infrastructure. In addition, it can provide both the bandwidth and reliable connectivity needed for real time very high-quality video remote monitoring of rail and non-rail assets. Currently most rail CCTV systems are deployed with local storage and then transferred later when required bandwidth is available.

## Service implementation

In a medical emergency scenario, there are several uplink flows that need to be transmitted from the incident scenario or the ambulance (uplink):

- Ambient video, from camera on ambulance and from body worn / head mounted cameras from paramedics. High resolution video is required for specific tasks, such as the observation of eyes.

- Medical data, such as ECG or ultrasound video. The specific case of ultrasound video is important because it requires high data rate. Frame rate is typically 25-40 fps, but higher temporal resolution is required to identify specific cardiac mechanical events: ultrafast cardiac imaging [Henry 2021] is considering recently 500-600fps for specific applications. The typical space resolution is 640 x 480 pixels, but higher resolutions are also considered (e.g., 1280 x 768, 1400 x 1050). This results in an uplink data rate from 92 Mbps (uncompressed ultrasound video), which can be reduced with compression, e.g., starting from 658 kbps (HEVC medium quality) [Razaak 2014].

In the downlink direction, fast access to patients’ medical records from hospital/national health system is also required.

There are different sub-scenarios:

- Data transmission from the emergency site.

- Data transmission from the ambulance, while in motion (high-speed possible).

- Guided intervention from the hospital. In this scenario, an external expert (e.g. a specialist in the hospital) needs to support the paramedic in real-time. Besides the mentioned flows, it requires low-latency bidirectional communication.

A typical deployment of CCTV for railway infrastructure monitoring will contain a dense number of IoT cameras. Each camera will produce an upstream video flow, starting from low resolution and low frame rate (e.g. 640 x 480 pixels, one frame per second) to high or ultra-high definition at 30 fps. This translates into a data rate range from about 200 kbps up to 20 Mbps. Donwlink rate is negligible.

Table 9: Service KPIs for First Responder Networks

| Use case | Number of UEs per cell | Latency (RTT) | Data Rate per UE (UL) | Data Rate per UE (DL) | PER |
| --- | --- | --- | --- | --- | --- |
| Medical Emergency | 1-5 | 100 - 1000 ms | 10 – 100 Mbps | 0.1 – 0.3 Mbps | 10-8 |
| Wireless CCTV | 5-50 | 1000 ms | 0.1 – 20 Mbps | - | 10-6 |

Different network configurations may be applied to the described scenarios. However, first responder networks require coverage over wide areas, either permanently (CCTV monitoring of a large infrastructure) or temporarily (medical emergency). For such reason, low to mid frequency ranges are proposed (900 MHz to 2.8 GHz).

Some first responder use cases include specifically regulated frequencies and even dedicated network services. An example of this is Mission-Critical Push-to-Talk (MCPTT), which provides “walkie-talkie” videoconference functionality on top of cellular network. MCPTT is not specifically covered in this document, although it presents similarities with the described use cases.

Since the First Responder Networks use case is particularly complex, a wide range of 5QIs are applicable to it (Table 10). They can be divided in three groups:

- 5QIs regarding video communication, for the uplink of the Medical Emergency scenario: 2, 7; and voice communication for the downlink: 1, 7.

- Specific 5QIs for push-to-talk communication and mission-critical video (65, 66, 67, 69, 70), specifically designed for First Responder communications.

- 5QIs for buffered video, to connect wireless CCTV cameras: 8, 9. If the real-time access of this video became critical during an emergency scenario, then the traffic should be moved to conversational video (or even mission critical) 5QIs.

Table 10: Relevant 5QIs for First Responder Networks

| 5QI Value | Resource Type | Default Priority Level | Packet Delay Budget | Packet Error Rate | Default Maximum Data Burst Volume | Default Averaging Window | Example Services |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | GBR | 20 | 100 ms | 10-2 | N/A | 2000 ms | Conversational Voice |
| 2 |  | 40 | 150 ms | 10-3 | N/A | 2000 ms | Conversational Video (Live Streaming) |
| 65 |  | 7 | 75 ms | 10-2 | N/A | 2000 ms | Mission Critical user plane Push To Talk voice (e.g. MCPTT) |
| 66 |  | 20 | 100 ms | 10-2 | N/A | 2000 ms | Non-Mission-Critical user plane Push To Talk voice |
| 67 |  | 15 | 100 ms | 10-3 | N/A | 2000 ms | Mission Critical Video user plane |
| 7 | Non-GBR | 70 | 100 ms | 10-3 | N/A | N/A | Voice, Video (Live Streaming) |
| 8 |  | 80 | 300 ms | 10-6 | N/A | N/A | Video (Buffered Streaming) |
| 9 |  | 90 |  |  |  |  |  |
| 69 |  | 5 | 60 ms | 10-6 | N/A | N/A | Mission Critical delay sensitive signalling (e.g. MC-PTT signalling) |
| 70 |  | 55 | 200 ms | 10-6 | N/A | N/A | Mission Critical Data (e.g. example services are the same as 5QI 6/8/9) |

## Relevant QoE Indicators

First-responder use cases are strongly focused on the required task. For the ones described in this document, it is either the coordination in the attendance of the patients or the surveillance of the railway infrastructure. The quality of the system is evaluated through ability of the first responders to make swift decisions, based on video and data provided.

This can be impacted by the following:

- Video resolution and compression quality.

- Video Frame Rate is key in two scenarios:

  - Ultrasound video, as described before.

  - CCTV monitoring of fast-moving trains. Lower frame rates provide slow sampling rates especially for fast moving objects. For example, a 15 FPS forward facing camera running on a 11 m/s train can only sample objects within 0.7 m [P. Basilio]

- Camera capture conditions may be challenging, especially on moving scenarios (trains, ambulances) and outdoors: camera stability, weather and light conditions, etc.

- Streaming from vehicles at high speed has impact on several parameters of the access network quality (cell changes, beamforming, coherence time, etc).

## Key Factors to Evaluate User QoE

From the technical perspective, the main components of the final QoE are:

- Capture-to-viewing latency, including coding and network delays.

- The quality of the video signal from the cameras and sensors, including spatial resolution, frame rate, coding quality and the effects of transmission errors.

- For the specific use case of the guided intervention from hospital, the two-way latency of the whole interaction loop.

In a multi-camera CCTV feed setup, these components are crucial for an effective execution. Capture -to-viewing latency will have a direct impact on camera synchronization and this could lead, multiple feed mismatch and delay. To evaluate the video quality for a given camera, the following factors should be considered: image sensor size, sensor resolution, frame rate, digital signal processing and optical path. While the image sensor size is fixed on any given camera, the other factors can be easily changed and thus affect the perceptual quality of the resulting video.

Objective metrics are desirable to avoid the costs of subjective assessments and/or expert viewing.

Regarding the subjective assessment of QoE: Evaluation of first responder systems must be based on a task-oriented methodology. Due to the high level of specialization of the users of these systems, experts are required in all the phases of the subjective evaluation: design, execution, and evaluation. A comparative study of the methodologies used for subjective medical image quality assessment can be found on [Lévêque 2021].

Annex A
Derivation of KPIs for Mixed Reality Offloading

Assuming that a single RGB and single depth feeds are compressed and transmitted, after compression, each pixel is estimated to weight one bit. These assumptions are used to estimate the uplink and downlink frame sizes summarized in Table A-1.

Table A-1: Uplink and downlink streams weights for scenarios A and B

| Scenario A & B |  |  |  |
| --- | --- | --- | --- |
| Uplink |  |  |  |
| Data Feed | Frame Size | Frequency | Mean Throughput |
| RGB 1080p | 2 Mbit | 60 Hz | 120 Mbps |
| Depth 1024x1024 | 1 Mbit |  | 60 Mbps |
| Total | 3 Mbit |  | 180 Mbps |
| Scenario A |  |  |  |
| Downlink |  |  |  |
| Data Feed | Frame Size | Frequency | Mean Throughput |
| Rendered Left Eye 4K | 8.3 Mbit | 60 Hz | 498 Mbps |
| Rendered Right Eye 4K | 8.3 Mbit |  | 498 Mbps |
| Total | 16.6 Mbit |  | 996 Mbps |
| Scenario B |  |  |  |
| Downlink |  |  |  |
| Data Feed | Frame Size | Frequency | Mean Throughput |
| Low resolution mask | 0.3 Mbit | 60 Hz | 18 Mbps |

Using the uplink and downlink stream sizes shown in Table A-1, a simple optimization algorithm is applied to estimate an initial set of network latency and peak throughput KPIs for both scenarios. The estimated KPIs are included in Table A-2.

Table A-2: Set of peak throughput and round trip latencies KPIs for the offloading scenarios A and B

| Scenario A |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | Data rate | Processing – 5 ms |  | Processing – 10 ms |  |
|  |  | Peak Throughput | Latency | Peak Throughput | Latency |
| 3 Mbit | 180 Mbps | ~1.25 Gbps | ~3 ms | ~1.7 Gbps | ~ 2 ms |
| 16.6 Mbit | 996 Mbps | ~2.8 Gbps |  | > 3 Gbps |  |
| Scenario B |  |  |  |  |  |
| Size | Data Rate | Processing – 5 ms |  | Processing – 10 ms |  |
|  |  | Peak Throughput | Latency | Peak Throughput | Latency |
| 3 Mbit | 180 Mbps | ~ 0.75 Gbps | ~6 ms | ~0.9 Gbps | ~3.5 ms |
| 18 Mbit | 18 Mbps | ~ 0.25 Gbps |  | ~0.3 Gbps |  |

.

___________________

PDU sessions can also be established at layer 2 using Ethernet packets, with similar functionality.