3GPP TSG-SA5 Meeting #144-e	S5-224424

# e-meeting, 27 June - 1 July 2022

# Source:	Nokia, Nokia Shanghai Bell

# Title:	pCR 28.831 Add potential requirements for key issue Targeted notification subscription

# Document for:	Approval

Agenda Item:		6.8.2.3 - FS_eSBMAe_WoP#4

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below

# 2	References

[1]		3GPP TS 28.831: " Management and orchestration; Study on basic Service-Based Management Architecture (SBMA) enabler enhancements"

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TR 28.831[1].

Begin of modifications

### 4.2.4	Potential requirements

Potential requirements to address the issues identified above are

[Req-1] A subscription shall enable to scope a single object instance.

[Req-2] A subscription shall enable to scope a complete subtree of objects starting at a specified base object.

[Reg-3] A subscription shall enable to scope the objects on a specified level below a specified base object.

[Req-4] A subscription shall enable to scope the objects starting at a specified base object down to and including a specified level below the base object.

[Req-5] A subscription shall enable to scope multiple object instances based on object instance identifiers.

[Req-6] A subscription shall enable to scope multiple object instances based on object classes.

[Req-7] A subscription shall enable to scope multiple object instances based on filter conditions.

The following potential additional requirements apply for subscriptions to CM notifications:

[Req-8] A subscription shall enable to scope attributes within scoped object instances.

[Req-9] A subscription shall enable to scope attribute fields within scoped object instances.

[Req-10] A subscription shall enable to scope attribute elements within scoped object instances.

[Req-11] A subscription shall enable to scope attributes, attribute fields and attribute elements based on filter conditions.

End of modifications