- 3GPP TSG-SA5 Meeting #	S5- 225038rev2

- June 2022, E-meeting

- Source:	Nokia

- Title:	pCR 28.908 Add use case on AIML training trustworthiness

- Document for:	Approval

- Agenda Item:	6.7.5.3

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.908-020 “Study on Artificial Intelligence/Machine Learning (AI/ ML) management”.

# 3	Rationale

During AI/ML training, testing and inference, the AI/ML trustworthiness management is needed. E.g.,  mandatory regulatory requirements for AI/ML trustworthiness may be put in place, for e.g., the EU has proposed an AI regulation act for AI/ML consisting of seven key requirements that the AI/ML systems should meet (based on the risk level of the use case) for them to be considered trustworthy. This pCR is to add the UC and potential requirements on AI/ML training trustworthiness.

# 4	Detailed proposal

Start of modification

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[3]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[4]	3GPP TS 28.105: " Artificial Intelligence / Machine Learning (AI/ML) management ".

Next modification

# 5	Use cases, potential requirements and possible solutions

## 5.A	AI/ML trustworthiness

### 5.A.2	Use cases

#### 5.A.2.3	AI/ML training trustworthiness

The AI/ML training may need to be performed according to the desired trustworthiness measure of the AI/ML model. For e.g.,

- The AI/ML model can be trained to generate explanations for the predictions.

- The AI/ML model can be trained to detect and mitigate biased outcomes.

- The AI/ML model can be trained to perform well on unseen or missing data.

- The AI/ML model can be trained together with adversarial input samples so that the trained model can detect adversaries.

Depending on the use case, one or more training trustworthiness techniques can be applied during training the AI/ML model. Therefore, the AI/ML training producer can be queried to provide information on the supported training trustworthiness capabilities enabling the AI/ML training consumer to request for a subset of supported training trustworthiness characteristics to be configured, measured, and reported.

### 5.A.3	Potential requirements

REQ-ML_TRAIN_TRUST-1 The MLTraining MnS producer should support a capability to enable an authorized consumer to request reporting on the supported training explainability capabilities of an AI/MLEntity.

REQ-ML_TRAIN_TRUST-2 The MLTraining MnS producer should have a capability to train a specific AI/MLEntity using training data with explainability characteristics as defined by the consumer.

REQ-ML_TRAIN_TRUST-3 The MLTraining MnS producer should support a capability to enable an authorized consumer to define the reporting characteristics related to the training explainability reports (e.g., global explanations) of an AI/MLEntity.

REQ-ML_TRAIN_TRUST-4 The MLTraining MnS producer should support a capability to enable an authorized consumer to request reporting on the supported training fairness capabilities of an AI/MLEntity.

REQ-ML_TRAIN_TRUST-5 The MLTraining MnS producer should have a capability to train a specific AI/MLEntity using training data with fairness characteristics as defined by the consumer.

REQ-ML_TRAIN_TRUST-6 The MLTraining MnS producer should support a capability to enable an authorized consumer to define the reporting characteristics related to the training fairness reports (e.g., measured disparate impact) of an AI/MLEntity.

REQ-ML_TRAIN_TRUST-7 The MLTraining MnS producer should support a capability to enable an authorized consumer to request reporting on the supported training robustness (technical) capabilities of an AI/MLEntity.

REQ-ML_TRAIN_TRUST-8 The MLTraining MnS producer should have a capability to train a specific AI/MLEntity using training data with robustness (technical) characteristics as defined by the consumer.

REQ-ML_TRAIN_TRUST-9 The MLTraining MnS producer should support a capability to enable an authorized consumer to define the reporting characteristics related to the training robustness (technical) reports (e.g., measured missingness ratio) of an AI/MLEntity.

REQ-ML_TRAIN_TRUST-10 The MLTraining MnS producer should support a capability to allow an authorized consumer to request reporting on the supported training robustness (adversarial) capabilities of an AI/MLEntity.

REQ-ML_TRAIN_TRUST-11 The MLTraining MnS producer should have a capability to train a specific AI/MLEntity using training data with robustness (adversarial) characteristics as defined by the consumer.

REQ-ML_TRAIN_TRUST-12 The MLTraining MnS producer should support a capability to enable an authorized consumer to define the reporting characteristics related to the training robustness (adversarial) of an AI/MLEntity.

### 5.A.4	Possible solutions

TBD

End of modifications