- 3GPP TSG-SA5 Meeting #	S5- 225041

- June 2022, E-meeting

- Source:	Nokia

- Title:	pCR 28.908 Add potential solution on AIML Inference History

- Document for:	Approval

- Agenda Item:	6.7.5.5

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1] 	3GPP TR 28.908-010 “Study on Artificial Intelligence/Machine Learning (AI/ ML) management”.

# 3	Rationale

This pCR is to add the UC and potential requirements on AI/ML Inference History.

# 4	Detailed proposal

Start of modification

# 5	Use cases, potential requirements and possible solutions

## 5.2	Event data for ML training

### 5.2.1	Description

In analytics solutions, Performance Measurements (PMs) and Fault Reports (FRs) from various network function are collected and analytics applied on the PMs and FRs to come up with statistical insights and predictions of events from the raw data. For most algorithms, the prediction accuracy depends upon the amount of relevant historical data, motivating the need to store ever more data, which correspondingly increases the storage and processing resource requirements. However, not all recorded data is useful as the derived events, e.g. captured through analytics processes, may have loss of information OR mis-information e.g., with respect to time of the event.

### 5.2.2	Use cases

#### 5.2.2.1	Pre-processed event data for ML training

For AI/ML algorithms, a large amount of data points does not necessarily add value, e.g., if most of it includes biased data which ends up getting discarded during the pre-processing stages. Instead, the AI/ML algorithms need to have information-rich events data that is condensed but with most of it useful for the required training. For example, one could train an interference optimization solution that learns the best way to combat interference by looking at  counters of handover failures correlated with signal quality. However, for most of the time in the radio network, there will be no interference events, but this cannot be determined if the events are not captured form the data. As such all the data must be kept and used for training. However, the data could also be mined for the interference event or signatures thereof. Then an equivalent interference management solution could be trained using the interference signatures or the signatures combined with only a small amount of raw data.

It is as such necessary to provide means to isolate and store the information rich events in the network, to ensure that minimizing storage and processing costs by discarding the unnecessary raw data does not compromise the ability to and still avails adequate historical information to adequately train AI/ML applications. In other words it is necessary for network functions to their management system to generate data on about the observed network events, e.g., based on the criteria set by the Operator, which events can then be stored to be used later to train AI/ML applications.

Network Function

AI/ML Trainig Function as Network events Data Consumer

Events processing

Events data

Management Function

Events processing

Network/system data

Network/system events data

Fig 2.2.1-1. Exposing and storing network events data

### 5.2.3	Potential requirements

REQ-EVENT-DATA-1	The 3GPP management system shall enable an authorized consumer to request from the network data producer for network events corresponding to the data produced by that network data producer.

REQ-EVENT-DATA-2 The 3GPP management system shall enable A network data producer to generate network events in place of or alongside the network data that they produce

REQ-EVENT-DATA-3 The 3GPP management system shall enable a  network events aggregator to take the events from different network entities and re-expose them in an aggregated way that eliminates duplications

### 5.2.4	Possible solutions

- Introduce a Network Events Processing Agent (named e.g. NetworkEventsProcessor or NEPA ) as an IOC contained in any ManagedFunction, ManagementFunction or subnetwork. This NEPA is responsible for generating the events data which can be availed to the AI/ML enabled function.

- Introduce an information Model for a Network Event as the datatype that can be exposed by an NEPA. The model may distinguish between Metric-Threshold Crossing events and Object-Status Change Events

  - Metric Threshold Crossing events are the events captured when a metric crosses a configured threshold. The definition of the Metric Threshold Crossing events may reuse the threshold monitoring MnS defined in TS28.550 as well as the related NRMs defined in TS28.622. Thereby, new parameters and metrics for which thresholds can be set may have to be defined

  - Object Status Change Events are the events captured when the status of a managed object instance changes be it due to internal processes within the MOI or due to external actors doing something on the MOI.

- Introduce a Status-Change-monitoring IOC contained on any ManagedElement, ManagedFunction, or ManagementFunction, to enable a consumer to define Object Status Change Events.

- Define/specify a set of network events for different managed objects. Example network events include

Table : Examples of potential Metric Threshold Crossing events

| Event- source Function | Event Name | Event description | Input/Metric | MOI | Threshold | Condition | Monitoring Period |
| --- | --- | --- | --- | --- | --- | --- | --- |
| BTSs; NBs; eNBs; gNBs; BSC; RNC | High Call Drop Rate event | Call Drop Rate more than a configurable threshold | Call Drop Rate | Cell | 2% | greater than | 15 minutes |
|  | Low Availability KPIs event | Availability KPIs dropping below a configurable threshold | Availability | Cell | 99% | less than | 30 minutes |
|  | Low Retainability KPIs event | Retainability KPIs dropping below a configurable threshold | Retainability | Cell | 98% | less than | 30 minutes |
|  | High Traffic event | Traffic greater than a configurable threshold | Cell load / PRB Utilization | Cell | 80% | greater than | 15 minutes |

Table : Examples of potential Object-Status Change events

| Event-source Function | Event Name | Event description | MOI | Affected unit /parameter | Change/ value |
| --- | --- | --- | --- | --- | --- |
| BTSs; NBs; eNBs; gNBs; BSC; RNC; NMS | HW Upgrade event | System Module HW version upgraded | BTS | System Module, Radio Module, … | HW version |
|  | SW Upgrade event | System Software Upgraded | BTS | System Module, Radio Module, … | SW version |
|  | Capability Enablement event | A specific Capability Enabled on the MOI | BTS | Spectrum Sharing | Spectrum Range for affected RATs |
|  | New Sector Addition Event | A new Sector getting added to a Site | Cell | Capacity and Coverage | Number of Sectors / Cells |
| Network Management | Parameter change event | CM Parameter changes applied for specific network element | Cell | Configuration Parameter | Parameter value |
|  | Home status event | MOI (e.g. site) Rehoming | Site | BSC/RNC, OSS | New BSC/ RNC/ OSS |
| SON, Analytics function | New Site event | New Site Integrated | Site/ gNB | C-SON Functions | Optimization Parameters in C-SON |
|  | - Predicted Congestion | - Trigger for Load Balancing detected | - Cell | - C-SON LBO | - Mobility Parameter changes |
|  | - Frequent Handover Failures | - Trigger for MRO detected | - Cell | - C-SON MRO | - Handover Parameter Changes |
|  | - PCI Conflict | - PCI conflict detected | - Cell | - C-SON PCI | - PCI Changes |
|  | - PRACH Conflict | - PRACH conflict detected | - Cell | - C-SON PRACH | - PRACH related parameter Changes |
|  | - NCR Change | - New First tier neighbor getting added | - Cell | - C-SON ANR | - NCR Changes |
|  | - Frequency Layer Change | - New Frequency Layer added onto a site | - BTS | - C-SON | - Frequency Layer Addition |

End of modifications