3GPP TSG-SA5 Meeting #145-e 	S5-225246rev1

- e-meeting, 15 - 24 August 2022

- Source:	Huawei

- Title:	Add potential solution of SMF configuration

- Document for:	Approval

Agenda Item:	6.8.4.3

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	        3GPP draft TR 28.833: "Study on management aspects of 5GLAN"

# 3	Rationale

This contribution is to add potential solution to support the key issue agreed for SMF configuration for 5G VN group communication.

# 4	Detailed proposal

1st Change

### 5.2.4	Solutions

5.2.4.1			Potential solution on Information model for DNN/S-NSSAI of SMF

As described in clause 5.29.3 of TS 23.501-h50, a UE gets access to 5G LAN-type services via a PDU Session established by a dedicated SMF. And, the network is configured that the SMF is selected for a certain 5G VN group, which the SMF will register on the NRM with DNN/S-NSSAI. So, a DNN and S-NSSAI is associated with a 5G VN group. This certain SMF is identified during PDU session establishment process with the DNN/S-NSSAI used for a given 5G VN group. The PDU session establishment procedure for a UE to access the 5G LAN-type service is described in TS 23.502.

The NRM IOC of SMFFunction is specified in clause 5.3.2 of TS 28.541. The attributes of managedNFProfile in SMF IOC can include smfinfo of the configured SMF to serve a 5G VN group communication. So as the DNN/S-NSSAI for the SMF serving a 5G LAN-type communication service can be provided based on current definition of NRM IOC for SMFFunction.

End of changes