3GPP TSG-SA5 Meeting #145e 	S5-225257

- e-meeting, 15 - 24th August 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.538 | CR | 0018 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ECSFunction IOC update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eECM |  |  |  |  |  | Date: |  |  | 2022-08-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TS 23.558 and TS23.548, Spatial Validity Conditions is added as ECS configuration information, but this information is missing in ECSFuntion IOC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Spatial Validity Conditions in ECFFunction IOC according to TS 23.558 and TS23.548 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement for Spatial Validity Conditions is not fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1, 6.3.5, A.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge link: https://forge.3gpp.org/rep/sa5/MnS/-/blob/28.538_Rel-18_CR018_ECSFunction_IOC_update/OpenAPI/TS28538_EdgeNrm.yaml |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

## 6.1	Information Model definitions for Edge NRM

### 6.1.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| TS 28.622 [4], IOC, Top | Top |
| TS 28.622 [4], IOC, SubNetwork | SubNetwork |
| TS 28.622 [4], IOC, ManagedFunction | ManagedFunction |
| TS 28.541 [3], IOC, PCFFunction | PCFFunction |
| TS 28.541 [3], IOC, NEFFunction | NEFFunction |
| TS 28.541 [3], IOC, UPFunction | UPFFunction |
| TS 28.541 [3], IOC, EP_N5 | EP_N5 |
| TS 28.541 [3], IOC, EP_N33 | EP_N33 |
| TS 28.541 [3], IOC, EP_N6 | EP_N6 |
| TS 28.541 [3], dataType, tAI | tAI |
| TS 28.658 [12], dataType, PLMNId | PLMNId |
| TS 28.541 [3], dataType, mCC | mCC |

Next Modified Section

### 6.3.5	ECSFunction

#### 6.3.5.1	Definition

This IOC represents the ECS functionality for supporting Edge Computing. For more information about the ECS, see 3GPP TS 23.558 [2].

#### 6.3.5.2	Attributes

The ECSFunction IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622 [4]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| ecsAddress | M | T | T | F | T |
| providerIdentifier | O | T | T | F | T |
| softwareImageInfo | M | T | T | F | T |
| trackingAreaIdList | O | T | T | F | T |
| mCC | O | T | T | F | T |
| geographicalLocation | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| edgeDataNetworkRef | M | T | T | F | T |
| eESFunctionRef | M | T | T | F | T |

Next Modified Section

## A.2	Solution Set (SS) definitions

### A.2.1	OpenAPI document "TS28538_EdgeNrm.yaml"

openapi: 3.0.1

info:

title: 3GPP Edge NRM

version: 17.1.0

description: >-

OAS 3.0.1 specification of the Edge NRM

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.538; Edge NRM

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.538/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

ServingLocation:

type: object

properties:

geographicalLocation:

$ref: '#/components/schemas/GeoLoc'

topologicalLocation:

$ref: '#/components/schemas/TopologicalServiceArea'

TopologicalServiceArea:

type: object

properties:

cellIdList:

type: array

item: integer

trackingAreaIdList:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/TaiList'

servingPLMN:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/PLMNId'

GeoLoc:

type: object

properties:

geographicalCoordinates:

$ref: '#/components/schemas/GeographicalCoordinates'

civicLocation:

type: string

GeographicalCoordinates:

type: object

properties:

latitude:

type: integer

longitude:

type: integer

EDNConnectionInfo:

type: object

properties:

dNN:

type: string

eDNServiceArea:

$ref: '#/components/schemas/ServingLocation'

AffinityAntiAffinity:

type: object

properties:

affinityEAS:

type: array

item: string

antiAffinityEAS:

type: string

VirtualResource:

type: object

properties:

virtualMemory:

type: integer

virtualDisk:

type: integer

virtualCPU:

type: string

SoftwareImageInfo:

type: object

properties:

minimumDisk:

type: integer

minimumRAM:

type: integer

discFormat:

type: string

operatingSystem:

type: string

swImageRef:

type: string

description: indicates the reference to the actual software image that is represented by URL (see clause 7.1.6.5 in ETSI NFV IFA-011 [7]).

#-------- Definition of concrete IOCs --------------------------------------------

MnS:

oneOf:

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

SubNetwork-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: ' TS28623_GenericNrm.yaml #/components/schemas/SubNetwork-Attr'

- type: object

properties:

Subnetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

ECSFunction:

$ref: '#/components/schemas/ECSFunction-Multiple'

EdgeDataNetwork:

$ref: '#/components/schemas/EdgeDataNetwork-Multiple'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/SubNetwork-ncO'

EdgeDataNetwork-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

ednIdentifier:

type: string

eDNConnectionInfo:

$ref: '#/components/schemas/EDNConnectionInfo'

- type: object

properties:

EASFunction:

$ref: '#/components/schemas/EASFunction-Multiple'

EESFunction:

$ref: '#/components/schemas/EESFunction-Multiple'

EASFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

eASIdentifier:

type: string

eESAddress:

type: array

item: string

eASRequirementsRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

eASAddress:

type: array

item: string

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

EESFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

eESIdentifier:

type: string

eESServingLocation:

type: array

item:

$ref: '#/components/schemas/ServingLocation'

eESAddress:

type: array

item: string

softwareImageInfo:

$ref: '#/components/schemas/SoftwareImageInfo'

serviceContinuitySupport:

type: boolean

eASFunctonRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

ECSFunction-Single:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

eCSAddress:

type: string

providerIdentifier:

type: string

edgeDataNetworkRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

eESFuncitonRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

softwareImageInfo:

$ref: '#/components/schemas/SoftwareImageInfo'

trackingAreaIdList:

$ref: 'TS28541_NrNrm.yaml#/components/schemas/TaiList'

geographicalLocation:

$ref: '#/components/schemas/GeoLoc'

mcc:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Mcc'

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/ManagedFunction-ncO'

EASRequirements:

allOf:

- $ref: 'TS28623_GenericNrm.yaml#/components/schemas/Top'

- type: object

properties:

requiredEASservingLocation:

$ref: '#/components/schemas/ServingLocation'

affinityAntiAffinity:

$ref: '#/components/schemas/AffinityAntiAffinity'

serviceContinuity:

type: boolean

virtualResource:

$ref: '#/components/schemas/VirtualResource'

softwareImageInfo:

$ref: '#/components/schemas/SoftwareImageInfo'

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

EASFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/EASFunction-Single'

ECSFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/ECSFunction-Single'

EESFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/EESFunction-Single'

EdgeDataNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/EdgeDataNetwork-Single'

#--------------------------------- Definition ------------------------------------

resources-edgeNrm:

oneOf:

- $ref: '#/components/schemas/MnS'

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/EASFunction-Single'

- $ref: '#/components/schemas/ECSFunction-Single'

- $ref: '#/components/schemas/EESFunction-Single'

- $ref: '#/components/schemas/EdgeDataNetwork-Single'

- $ref: '#/components/schemas/EASRequirements'

End Of Modified Section