3GPP TSG-SA5 Meeting #145-e 	S5-225396

- e-meeting, 15 - 24 August 2022

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Add key issue for Energy Saving compensation procedure

- Document for:	Approval

Agenda Item:	6.6.1.1

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below

# 2	References

[1]			3GPP TS 28.913: "Study on new aspects of EE for 5G networks phase 2"

[2]			3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[3]			3GPP TS 32.551: "Energy Saving Managament (ESM); Concepts and requirements".

# 3	Rationale

The energy saving procedures described in TS 28.310 [2] clause 6.2 does not specify the impact on the candidate cell(s) when energy saving is activated in the capacity booster cell in 5G. The concept of ES compensation and compensating for energy saving state are described in TS 32.551 [3] for the legacy technologies.

This contribution proposes to add a new key issue to study potential solutions for the concept of ES compensation for 5G.

# 4	Detailed proposal

The following changes are proposed for TR 28.913[1].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.554: " Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[3]	ETSI GS NFV-IFA 027 V4.2.2 (2021-07): "Network Functions Virtualisation (NFV) Release 4; Management and Orchestration; Performance Measurements Specification".

[4]	ETSI ES 202 336-12 V1.2.1 (2019-02): "Environmental Engineering (EE); Monitoring and control interface for infrastructure equipment (power, cooling and building environment systems used in telecommunication networks); Part 12: ICT equipment power, energy and environmental parameters monitoring information model".

[5]	ETSI GS NFV-EVE 004 V1.1.1 (2016-03): "Network Functions Virtualisation (NFV); Virtualisation Technologies; Report on the application of Different Virtualisation Technologies in the NFV Framework".

[6]	ETSI GR NFV-IFA 029 V3.3.1 (2019-11): "Network Functions Virtualisation (NFV) Release 3; Architecture; Report on the Enhancements of the NFV architecture towards "Cloud-native" and "PaaS"".

[7]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[8]	3GPP TS 38.401: "NG-RAN; Architecture description".

[9]	The Greenhouse Gas Protocol - https://ghgprotocol.org/sites/default/files/standards/ghg-protocol-revised.pdf

[X]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[Y]	3GPP TS 32.551: "Energy Saving Managament (ESM); Concepts and requirements".

2nd Change

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

compensatingForEnergySaving state: Refer to 32.551[Y] for the definition.

ES compensation: Refer to 32.551[Y] for the definition.

energySaving state: Refer 28.310 [X] for the definition.

3rd Change

## 4.y	Key Issue #<y>: Energy Saving compensation procedure

### 4.y.1	Description

#### 4.y.1.1	Introduction

The energy saving procedures described in TS 28.310 [Y] clause 6.2 do not specify the impact on the candidate cell(s) when energy saving is activated in the capacity booster cell in 5G. The concept of ES compensation and compensating for energy saving state are described in TS 32.551 [Y] for the legacy technologies. This key issue is to study potential solutions for the concepts of ES compensation for 5G.

#### 4.y.1.2	Potential requirements

REQ-ESCOL-FUN-y1 The Domain-centralized ES shall support a capability to initiate energy saving compensation activation to one or multiple cells or network elements.

REQ-ESCOL-FUN-y2 The Domain-centralized ES shall support a capability to initiate energy saving compensation deactivation to one or multiple cells or network elements.

REQ-ESCOL-FUN-y3 The distributed ES function shall support a capability to initiate energy saving compensation activation to one or multiple cells or network elements.

REQ-ESCOL-FUN-y4 The distributed ES function shall support a capability to initiate energy saving compensation deactivation to one or multiple cells or network elements.

### 4.y.2	Potential solutions

#### 4.y.2.i	Potential solution #1: Energy saving compensation activation and deactivation procedures

##### 4.y.2.i.1	Introduction

The MnS producer for Domain-centralized ES or the distributed ES function, that makes a decision for the NR capacity booster cell to enter or exit energySaving state, should be able to initiate energy saving compensation activation and/or deactivation on one or multiple cells or network elements.

##### 4.y.2.i.2	Description

For the energy saving use cases (refer to TS 28.310 [X], clause 5.1.3), when a NR capacity booster cell enters energySaving state, then the candidate cell(s) may transition to the

-	compensatingForEnergySaving.

Correspondingly, such use cases may provide these additional procedures:

-	Energy saving compensation activation: the procedure to increase the coverage area for the candidate cell(s).

-	Energy saving compensation deactivation: the procedure to decrease a previously increased coverage area.

End of change