3GPP TSG-SA5 Meeting #145-e 	S5-225509rev1

- e-meeting, 15 - 24 August 2022

- Source:	CATT

- Title:	pCR TR 28.908 Add use case on online AI/ML entity testing

- Document for:	Approval

- Agenda Item:	6.7.5.3

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]		SP-211443 New Study on AI/ ML management

[2]		3GPP TR 28.908-040 Management and orchestration; Study on Artificial Intelligence/Machine Learning (AI/ML) management

[3]	3GPP TS 28.105: "Management and orchestration; Artificial Intelligence / Machine Learning (AI/ML) management".

# 3	Rationale

This pCR is to add the UC and potential requirements on AI/ML model testing.

# 4	Detailed proposal

Start of modification

#### 5.4.2.1	Consumer-requested AI/ML entity testing

After receiving an AI/ML training report about a trained AI/ML entity from the AI/ML Training MnS producer, the consumer may request the testing MnS producer to test the AI/ML entity before applying it to the target inference function. In the AI/ML testing request, the consumer provides the testing data which have the same pattern as the input part of the training data.

Any AI/ML entity needs to be tested with specific inputs and features that are applicable to the use case and the applicable deployment environment.

The AI/ML testing MnS producer performs the AI/ML testing using the consumer’s provided testing data. The AI/ML testing is to conduct inference on the tested AI/ML entity using the testing data as the inference inputs and produce the inference output for each testing dataset example.

The AI/ML testing MnS producer may be the same as or different from the AI/ML Training MnS producer.

After completing the AI/ML testing, the AI/ML testing MnS producer provides the testing report indicating the success or failure of the AI/ML testing to the consumer. For a successful AI/ML testing, the testing report contains the testing results, i.e., the inference output for each testing dataset example.

The AI/ML testing MnS producer needs to have the capabilities and provide the services needed to enable the consumer to request testing and receive results on the testing of a specific AI/ML entity or of an application or function that contains an AI/ML entity.

To achieve the desired outcomes, any AI/ML entity needs to be tested with the appropriate testing data (e.g. batch data or continuous data streams), which can reflect the current status of the network where the AI/ML entity is expected to be deployed. Correspondingly, the AI/ML testing MnS producer needs to support the required management services to test the AI/ML entities.

Next modification

### 5.4.3	Potential requirements

REQ-AI/MLTEST-1 The AI/ML testing MnS producer should have a capability for an authorized consumer to request the testing of a specific AI/ML entity.

REQ-AI/MLTEST-2 The AI/ML testing MnS producer should have a capability to create a testing process instance per the testing request for an authorized consumer.

REQ-AI/MLTEST-3 The AI/ML testing MnS producer should have a capability to report to an authorized consumer the results of a specific instance of AI/ML testing process with the result of a successful AI/ML entity testing containing the inference output for each testing data example.

REQ-AI/MLTEST-4 The AI/ML testing MnS producer should have a capability for an authorized consumer (e.g. the operator) to configure or modify an instance of AI/ML testing process.

REQ-AI/MLTEST-5 The AI/ML testing MnS producer should have a capability to test a specific AI/ML entity using specific data specified by the consumer or using data at a location address specified by the consumer or using data with specific characteristics defined by the consumer or using continuous data streams by the consumer.

REQ-AI/MLTEST-6 The AI/ML testing MnS producer should have a capability to test a specific AI/ML entity for a specified expected runtime context as may be stated by the consumer.

REQ-AI/MLTEST-8 The AI/ML testing MnS producer should support a capability for an authorized consumer to define the reporting characteristics related to a specific instance of AI/ML testing request.

REQ-AI/MLTEST-9 The AI/ML testing MnS producer should support a capability for an authorized consumer to manage the AI/ML testing request, including suspending, resuming, canceling the request, or  adjusting the desired runtime context of the testing.

End of modifications