3GPP TSG-SA5 Meeting #145-e 	S5-225521

e-meeting, Aug 15 - 24, 2022

- Source:	Ericsson

- Title:	pCR Add Intent Reporting support to NRM

- Document for:	Approval

- Agenda Item:		6.7.3.2

# 1	Decision/action requested

The group is asked to discuss and approval.

# 2	References

[1]	3GPP TS 28.312: “Intent driven management services for mobile networks (Release 17)”
v17.0.1

[2]	3GPP TS 28.912: “Study on enhanced intent driven management services for mobile networks (Release 18)” v0.3.0

# 3	Rationale

As indicated in 28.912 [2], “Issue #4.5 Intent Report” the current ability to report intent fulfilment status is not sufficient, and further support to support Intent Reporting is required.

In particular the ability to specify reporting output parameters, per MnS consumer per Intent, are needed to satisfy the following:  “Different MnS consumer may have different requirements for intent report (e.g. Some MnS consumer may want to have corresponding performance value information while others don't want, Different MnS consumer may want to calculate or monitor the performance value in different period).”

The NRM needed to support the Intent Reporting reqs specified in [2], sec 4.5.1 is not yet defined.  A solution is briefly described in [2], sec 4.5.2.1 however it needs to be enhanced to better support the above description.  In particular the ability for MnS Consumer to specific reporting output parameters needs to be included.

# 4	Detailed proposal

This contribution proposes the addition of IntentReport IOC and associated datatypes to support Intent Reporting requirement defined in 28.912 [2], sec 4.5.1.

The proposed IntentReport IOC and associated datatypes are based on those already defined in Intent IOC.  The purpose of the monitor is to allow MnS Consumer to configure and receive “reports” on specific attributes of interest in the associated Intent.  Only attributes (e.g. expectation Ids, specific targets) configured in the report will contained, allowing MnS Consumer to create report(s) for different purposes for the same associated Intent.

The datatype naming proposed is xxxInfo, where each item in Intent that MnS Consumer wishes to report on, is configured in the associated IntentReport xxxInfo parameter.

For example, MnS Consumer can monitor the status of a specific ExpectionObject would configure the specific ExpectionObject (by its id) in the IntentReport.

Since IntentReport IOC is structured similar to Intent IOC, using datatypes (not IOCs) subscribing to individual attribute updates in the report does not provide individual parameter (e.g. per Expectation id) notifications.  A notification will be provided on the IntentReport IOC, based on the reporting control.

1st Change

## 6.2	Information model definition for Intent (MnS component typeB)

### 6.2.1	Generic Information model definition

#### 6.2.1.1	Class diagram

##### 6.2.1.1.1	Relationship

Figure 6.2.1.1.1-1: Relationship UML diagram for Intent

##### 6.2.1.1.2	Inheritance

Figure 6.2.1.1.2-1: Inheritance UML diagram for intent

#### 6.2.1.2	Class definition

##### 6.2.1.2.1	Intent <<IOC>>

6.2.1.2.1.1	Definition

This IOC represents the properties of an Intent driven management information between MnS consumer and MnS producer.

The Intent IOC contains one or multiple IntentExpectation(s) which includes MnS consumer's requirements, goals and contexts given to a 3GPP system.

The Intent IOC includes the attribute objectClass and objectInstance from the TOP IOC. The value of attribute objectClass is "Intent" and the value of attribute objectInstance is the DN of the instance of Intent IOC.

6.2.1.2.1.2	Attributes

The Intent IOC includes attributes inherited from TOP IOC (defined in 3GPP TS 28.622 [6]) and the following attributes.

Table 6.2.1.2.1.2-1

| - Attribute Name | - Support Qualifier | - isReadable | - isWritable | - isInvariant | - isNotifyable |
| --- | --- | --- | --- | --- | --- |
| intentExpectations | M | T | T | F | F |
| userLabel | M | T | T | F | F |
| intentContexts | O | T | T | F | F |
| intentFulfilmentinfo | M | T | F | F | T |

6.2.1.2.1.3	Attribute constraints

None.

##### 6.2.1.2.x	IntentReportCtrl <<IOC>>

6.2.1.2.x.1	Definition

This IOC represents a request to produce Intent Reports which provide intent driven management updates from MnS producer.

The IntentReportCtrl contains configuration used by MnS Producer to determine the content and frequency at which reports should be provided for an Intent.

MnS Consumer creates an instance of IntentReportCtrl instance with a reference to the Intent instance for which the Intent reports will be produced.  Multiple IntentReportCtrl instances can reference the same Intent instance, meaning different reports can be produced for the same Intent.

The MnS Consumer configures the report output via intentExpectation, intentContext, and intentFulfilment attributes.  These parameters are writable and can be configured to restrict (i.e. filter) the report results.

The content of each Intent report produced is based on its containing IntentReportCtrl configuration at the time it is produced.  Only content configured in the reporting control will be included in its intent reports.

The userLabel contains the same value as userLabel in the associated Intent.

The IntentReportCtrl configures the report output, and determines when new IntentReport instances are created based.

The reportPeriod value indicates the frequency at which report are created by MnS Producer.  A value of “0” indicates the MnS Producer will determine the periodicity.

The IntentReportCtrl IOC includes the attribute objectClass and objectInstance from the TOP IOC. The value of attribute objectClass is "IntentReport" and the value of attribute objectInstance is the DN of the instance of the IntentReport IOC.

6.2.1.2.x.2	Attributes

The Intent IOC includes attributes inherited from TOP IOC (defined in 3GPP TS 28.622 [6]) and the following attributes.

Table 6.2.1.2.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| intentReference | M | T | T | F | F |
| userLabel | M | T | T | F | F |
| reportingPeriod | M | T | T | F | F |
| intentExpectation | O | T | T | F | F |
| intentContext | O | T | T | F | F |
| intentFulfilmentInfo | O | T | T | F | F |

6.2.1.2.x.3	Attribute constraints

None.

##### 6.2.1.2.x	IntentReport <<IOC>>

6.2.1.2.x.1	Definition

This IOC represents an Intent Report to provide Intent status details from MnS producer.

The userLabel contains the same value as userLabel in the associated Intent.

The reportTimestamp contains the time at which the report was last updated.  No further updates to the contents of a report are made once set.

The xxxInfo parameters are not writable in the report output.  The content of the report is based on the configuration of its containing IntentReportCtrl.   Each xxxInfo parameter contains information about the status of the associated xxx parameter in the Intent.

6.2.1.2.x.2	Attributes

The Intent IOC includes attributes inherited from TOP IOC (defined in 3GPP TS 28.622 [6]) and the following attributes.

Table 6.2.1.2.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| intentReference | M | T | F | F | F |
| userLabel | M | T | F | F | F |
| reportTimestamp | M | T | F | F | F |
| intentFulfilmentInfo | O | T | F | F | F |
| intentExpectationInfo | O | T | F | F | F |
| intentContextInfo | O | T | F | F | F |

6.2.1.2.x.3	Attribute constraints

None.

#### 6.2.1.3	DataType definition

##### 6.2.1.3.1	IntentExpectation <<dataType>>

6.2.1.3.1.1	Definition

IntentExpectation <<dataType>>represents MnS consumer's requirements, goals and contexts given to a 3GPP system. The information of IntentExpectation is generated by MnS consumer and delivered to MnS producer.

6.2.1.3.1.2	Attributes

The IntentExpectation includes the following attributes.

Table 6.2.1.3.1.2-1

| - Attribute Name | - Support Qualifier | - isReadable | - isWritable | - isInvariant | - isNotifyable |
| --- | --- | --- | --- | --- | --- |
| expectationId | M | T | T | T | F |
| expectationVerb | O | T | T | T | F |
| expectationObject | O | T | T | F | F |
| expectationTargets | M | T | T | F | F |
| expectationContexts | O | T | T | F | F |
| expectationfulfilmentInfo | O | T | F | F | T |
| NOTE: The scenario/requirements-specific IntentExpectations are defined utilizing the constructs of this generic IntentExpectation model. |  |  |  |  |  |

6.2.1.3.1.3	Attribute constraints

None.

##### 6.2.1.3.3	ExpectationObject <<dataType>>

6.2.1.3.3.1	Definition

The ExpectationObject <<dataType>> represents the Object of the IntentExpectation that are required to be applied on.

6.2.1.3.3.2	Attributes

The ExpectationObject includes the following attributes.

Table 6.2.1.3.3.2-1

| - Attribute Name | - Support Qualifier | - isReadable | - isWritable | - isInvariant | - isNotifyable |
| --- | --- | --- | --- | --- | --- |
| objectType | CM | T | T | F | F |
| objectInstance | CM | T | T | F | F |
| objectContexts | O | T | T | F | F |

6.2.1.3.3.3	Attribute constraints

Table 6.2.1.3.3.3-1

| - Name | - Definition |
| --- | --- |
| objectType Support Qualifier | Condition: The intent expectation is not for a specific object instance or MnS consumer have no knowledge of the DN of this specific object instance. |
| objectInstance Support Qualifier | Condition: The intent expectation is for a specific object instance and MnS consumer have the knowledge of the DN of this specific object instance. |

##### 6.2.1.3.4	ExpectationTarget <<dataType>>

6.2.1.3.4.1	Definition

The ExpectationTarget  <<dataType>> represents the targets of the IntentExpectation that are required to be achieved.

6.2.1.3.4.2	Attributes

The ExpectationTarget includes the following attributes.

Table 6.2.1.3.4.2-1

| - Attribute Name | - Support Qualifier | - isReadable | - isWritable | - isInvariant | - isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetName | M | T | T | F | F |
| targetCondition | M | T | T | F | F |
| targetValueRange | M | T | T | F | F |
| targetContexts | O | T | T | F | F |
| targetfulfilmentInfo | O | T | F | F | T |

6.2.1.3.4.3	Attribute constraints

None.

##### 6.2.1.3.5	Context <<dataType>>

6.2.1.3.5.1	Definition

The Context <<dataType>> represents the properties of a context. A context describes the list of constraints and conditions that should evaluate to true when the targets are fulfilled but are themselves not to be enforced. The context may apply to the intent, the intent expectation, the intent targets or to the object.

6.2.1.3.5.2	Attributes

The Context includes the following attributes.

Table 6.2.1.3.5.2-1

| - Attribute Name | - Support Qualifier | - isReadable | - isWritable | - isInvariant | - isNotifyable |
| --- | --- | --- | --- | --- | --- |
| contextAttribute | M | T | T | F | F |
| contextCondition | M | T | T | F | F |
| contextValueRange | M | T | T | F | F |

6.2.1.3.5.3	Attribute constraints

None.

##### 6.2.1.3.6	FulfilmentInfo << dataType >>

6.2.1.3.6.1	Definition

This dataType represents the properties of a specific fulfilment information for an aspect of the intent (i.e. either an expectation, a target or the whole intent). The fulfilment information describes the MnS producer's assessment of the degree to which a specific aspect of the intent has been fulfilled. The MnS consumer may however assess the fulfilment differently e.g. the MnS consumer may evaluate the delivered outcome or network state to compute its fulfilment satisfaction.

The fulfilmentStatus field indicates whether the intent is fulfilled or not fulfilled. The possible values of the fulfilment include:

The degree of fulfilment of an intent with the NOTFULFILLED status may have multiple explanations and related states. These different progress states and conditions are recorded in the notFulfilledState field. The possible values of the notFulfilledState include:

-	NOTFULFILLED: This is the default status for any aspect of the intent and the fulfilmentStatus remains as "NOTFULFILLED" until the MnS producer is satisfied that the actions undertaken meet the requirements as stated by the MnS consumer.

-	FULFILLED: This is the status if the MnS producer considers that the intent, expectation or target has been fulfilled as desired by the MnS consumer that created the intent. The consumer may provide a fulfilment satisfaction report that either confirms the fulfilment or describes its evaluation the fulfilment.

For some scenarios (in particular for the "SUSPENDED" and the "FULFILMENTFAILED" notFulfilledStates), the notFulfilledState should be supported by extra information describing or related to the state. This extra information is recorded into the notFulfilledReasons field.

-	ACKNOWLEDGED: this is the default status and is the initial notFulfilledState right after the intent has been received.

-	COMPLIANT: this is the state after the feasibility check has been run for the intent and the intent accepted as being compliant for fulfilment.

-	DEGRADED: this is the state if an intent that was previously fulfilled but after a period of observation it is found not be meeting the initially stated requirements.

-	SUSPENDED: this is the state if the MnS producer decides to suspect the fulfilment of the intent, expectation or target for whatever reason. This notFulfilledState shall be supported by a reason such as the event(s) that were observed when fulfilment was attempted.

-	TERMINATED: This state is registered if the respective aspect of the intent (i.e. either an expectation, a target or the whole intent) shall not be considered for fulfilment e.g. when an authorized MnS consumer sends an indication terminating the specific aspect of the intent. For instance, if the MnS consumer sends an update of the intent in which a particular target is eliminated, then that target shall be marked as cancelled.

-	FULFILMENTFAILED: This is the state when the MnS producer decides that the intent, expectation or target cannot be fulfilled. This state shall be supported by a reason such as the event(s) that were observed when fulfilment was attempted.

6.2.1.3.6.2	Attributes

The FulfilmentInfo includes the following attributes.

Table 6.2.1.3.6.2-1

| - Attribute Name | - Support Qualifier | - isReadable | - isWritable | - isInvariant | - isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fulfilmentStatus | M | T | F | F | T |
| notFulfilledState | CM | T | F | F | T |
| notFulfilledReasons | CO | T | F | F | T |

6.2.1.3.6.3	Attribute constraints

Table 6.2.1.3.6.3-1

| - Name | - Definition |
| --- | --- |
| notFulfilledState Support Qualifier | Condition: when FulfilmentInfo is implemented for IntentFulfilmentInfo |
| notFulfilledReasons Support Qualifier | Condition: when FulfillmentInfo is implemented for IntentFulfilmentInfo |

##### 6.2.1.3.x	IntentContextInfo <<dataType>>

6.2.1.3.x.1	Definition

IntentContextInfo <<dataType>> reports the context evaluation details.

For each contextAttribute configured in the report its current value (contextAttribValue) and evaluation status (contextEvalStatus is provided.

6.2.1.3.x.2	Attributes

The IntentContextInfo includes the following attributes.

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| contextAttribute | M | T | F | F | F |
| contextAttribValue | M | T | F | F | F |
| contextEvalStatus | M | T | F | F | F |

6.2.1.x.1.3	Attribute constraints

None.

##### 6.2.1.3.x	IntentExpectationInfo <<dataType>>

6.2.1.3.x.1	Definition

IntentExpectationInfo <<dataType>> provides the fulfilment status for the specified expectation.

For each expectationId in the report its current value and fulfilment status will be reported.

6.2.1.3.x.2	Attributes

The IntentExpectationInfo includes the following attributes.

Table 6.2.1.x.3.2-1

| expectationId | M | T | T | T | F |
| --- | --- | --- | --- | --- | --- |
| expectationObject | M | T | F | F | T |
| expectationValue | M | T | F | F | T |
| expectationfulfilmentInfo | M | T | F | F | T |
| expectationTargetInfo | M | T | F | F | T |
| expectationObjectInfo | M | T | F | F | T |

6.2.1.3.3.3	Attribute constraints

None.

##### 6.2.1.3.x	ExpectationObjectInfo <<dataType>>

6.2.1.3.x.1	Definition

The ExpectationObjectInfo <<dataType>> reports on which objects the reports applies to.

6.2.1.3.x.2	Attributes

The ExpectationObjectInfo includes the following attributes.

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| objectType | M | T | F | F | F |
| objectInstance | M | T | F | F | F |
| objectContexts | M | T | F | F | F |

6.2.1.x.1.3	Attribute constraints

None.

##### 6.2.1.3.x	ExpectationTargetInfo <<dataType>>

6.2.1.3.x.1	Definition

The ExpectationTargetInfo <<dataType>> reports on the target achievement details.

6.2.1.3.x.2	Attributes

The ExpectationObjectInfo includes the following attributes.

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| targetName | M | T | F | F | F |
| targetContexts | M | T | F | F | F |
| targetfulfilmentInfo | M | T | F | F | F |

6.2.1.x.1.3	Attribute constraints

##### None.

##### 6.2.1.3.x	ExpectationContextInfo <<dataType>>

6.2.1.3.x.1	Definition

ExpectationContextInfo <<dataType>> reports the context information per Intent reporting period.

6.2.1.3.x.2	Attributes

The IntentContextInfo includes the following attributes.

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceStartTimeContext | O | T | F | F | F |
| serviceEndTimeTargetContext | O | T | F | F | F |
| uEMobilityLevelContext | O | T | F | F | F |
| resourceSharingLevelContext | O | T | F | F | F |

6.2.1.x.1.3	Attribute constraints

None.

#### 6.2.1.4	Attribute definition

Table 6.2.1.4-1

| - Attribute Name | - Documentation and Allowed Values | - Properties |
| --- | --- | --- |
| userLabel | A user-friendly (and user assignable) name of the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentExpectations | It describes the expectations including requirements, goals and contexts (including constraints and filter information) given to a 3GPP system. It states the list of specific outcomes desired to be realized for expectation object(s). allowedValues: Not Applicable | type: IntentExpectation multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentFulfilmentInfo | It describes status of fulfilment of an intent and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationFulfilmentInfo | It describes status of fulfilment of an intentExpectation and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| targetFulfilmentInfo | It describes status of fulfilment of an expectationTarget and the related reasons for that status. allowedValues: Not Applicable | type: FulfilmentInfo multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentFulfillStatus | It describes the current status of the intent fulfilment result, which is configured by MnS producer and can be read by MnS consumer. allowedValues: "FULFILLED", "NOT_FULFILLED" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| notFulfilledState | It describes the current progress of or the reason for not achieving fulfilment for the intent, intentExpectation or expectationTarget. It is configured/written by MnS producer and can be read by MnS consumer. allowedValues: "ACKNOWLEDGED", "COMPLIANT", "DEGRADED", "SUSPENDED", "TERMINATED" "FULFILMENTFAILED" | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| notFulfilledReason | It describes the reasons/observations related to the specific noted notFulfilledState allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentContexts | It describes the list of IntentContext(s) which represents the constraints and conditions that should apply for the entire intent even if there may be specific contexts defined for specific parts of the intent. allowedValues: triple of (attribute, condition, value range) | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationId | A user-friendly (and user assignable) name of the intentExpectation. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationVerb | It describes the characteristic of the intentExpectation and is the property that describes the types of intentExpectations. Examples of verbs and their related types of expectation are Deliver: DeliveryIntentExpectation, e.g. Deliver a RAN network, Service, Slice, function Ensure: AssuranceintentExpectation, e.g. Ensure the performance targets values allowedValues: Deliver, Ensure | type: String multiplicity: 1 isOrdered:N/A isUnique: N/A defaultValue: None isNullable: False |
| expectationObject | It describes the expectation objects of the IntentExpectation that are required to be applied on. allowedValues: Not Applicable | type: ExpectationObject multiplicity: 1 isOrdered:N/A isUnique: N/A defaultValue: None isNullable: False |
| objectType | It describes the type of expectation object of the IntentExpectation that are required to be applied on. It can be class name of the managed object. allowedValues: see scenario specific Intent Expectation | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectInstance | It describes a specific object instance (e.g. instance of managed object) to which the intentExpectation should apply. allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectContexts | It describes the list of ObjectContext(s) which represents the constraints and conditions to be used as filter information to identify the object(s) to which a given intentExpectation should apply. Note there may be other constraints and conditions defined either for the entire intent, for the specific intentExpectation or for the expectationTarget of the considered intentExpectation. The concrete ObjectContext depends on the ExpectationObject, which is defined in clause 6.2.2. All the concrete ObjectContexts follow the common structure of ObjectContext | type: Context multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectionTargets | It describes the list of ExpectationTarget(s) which represent specific outcomes on the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s) that are desired to be realized for a given intentExpectation. The concrete ExpectationTarget depends on the ExpectationObject, which is defined in clause 6.2.2. All the concrete ExpectationTargets follow the common structure of ExpectationTarget | type: ExpectationTarget multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationContexts | It describes the list of context(s) which represents the constraints and conditions that should apply for a specific intentExpectation. Note there may be other constraints and conditions defined for the entire intent or for specific parts of the intentExpectation. allowedValues: depends on Expectation Object in the IntentExpectation | type: Context multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| targetName | It describes the name of the Expectation of the expectation target which represents specific outcomes on the metrics that characterize the performance of the object(s) or some abstract index that expresses the behavior of the object(s) that are desired to be realized for a given intentExpectation.allowedValues: depends on ExpectationObject in the IntentExpectation | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| targetCondition | It expresses the limits within which the targetName is allowed/supposed to be allowedValues: is equal to; is less than; is greater than: - "is within the range"; - "is outside the range" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "is equal to" isNullable: False |
| targetValueRange | It describes the range of values that applicable to the targetName and the targetcondition. allowedValues: depends on the targetName | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| targetContexts | It describes the list of constraints and conditions that should apply for a specific expectationTarget. Note there may be other constraints and conditions defined for the entire intent or the intentExpectation. allowedValues: Not Applicable | type: Context multiplicity: 1..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| contextAttribute | It describes a specific attribute of or related to the object or to characteristics thereof (e.g. its control parameter, gauge, counter, KPI, weighted metric, etc) to which the expectation should apply or an attribute related to the operating conditions of the object (such as weather conditions, load conditions, etc). | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| contextCondition | It expresses the limits within which the ContextAttribute is allowed/supposed to be allowedValues: is equal to; is less than; is greater than; "is within the range";"is outside the range" | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: "is equal to" isNullable: False |
| contextValueRange | It describes the range of values that applicable to the ContextAttribute and the ContextCondition. AllowedValue: depends on the contextAttribute | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: Null isNullable: True |
| intentReference | It contains a specific object instance (e.g. instance of managed object) to an Intent. allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentExpectationInfo | Reports the expectation information. | type: IntentExpectationInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentContextInfo | Reports the context evaluation information. | type: IntentContextInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| contextEvalStatus | Indicates the evaulation status of the Context conditions. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportTimestamp | The time at which the report was produced. | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| reportingPeriod | The internal (in seconds) at which reports are to be generated. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| intentExpectation | Configures the expectation parameters for intent reporting. | type: IntentExpectation multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| intentContext | Configures the context parameters for intent reporting. | type: IntentContext multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationTargetInfo | Reports the expectation target information. | type: ExpectationTargetInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| expectationObjectInfo | Reports the expectation target information. | type: ExpectationObjectInfo multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

End of changes

Each expectation, and target has a fulfilment info - This info can be extended with the achievedValue attribute to include the specific achieved value - the value is writable by the producer. The consumer acn then subscribe to notifications from MOI changes on that value

Alternative: the achievedValue attributeis aded tot he targets themsleves and each time there is a changethe consumerif notifies if they subscribed tot hat attribute change.

Second option which could be supported.

How does the consumer configure the attributes they want reported or not? We can add to each attribute a boolean to indicate if MOI changes for that atribute should be notified.

The IntentExpectation includes the following attributes.

Table 6.2.1.3.1.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| expectationId | M | T | T | T | F |
| expectationVerb | O | T | T | T | F |
| expectationObject | O | T | T | F | F |
| expectationTargets | M | T | T | F | F |
| expectationContexts | O | T | T | F | F |
| expectationfulfilmentInfo | O | T | F | F | T |
| notifyAchievement | O | T | F | F | T |
| NOTE: The scenario/requirements-specific IntentExpectations are defined utilizing the constructs of this generic IntentExpectation model. |  |  |  |  |  |

The ExpectationTarget includes the following attributes.

Table 6.2.1.3.4.2-1

| Attribute Name |  | Support Qualifier |  | isReadable |  | isWritable |  | isInvariant |  | isNotifyable |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| targetName |  | M |  | T |  | T |  | F |  | F |
| targetCondition |  | M |  | T |  | T |  | F |  | F |
| targetValueRange |  | M |  | T |  | T |  | F |  | F |
| targetContexts |  | O |  | T |  | T |  | F |  | F |
| targetfulfilmentInfo |  | O |  | T |  | F |  | F |  | T |
| achievedValue |  | O |  | T |  | F |  | F |  | T |
| notifyAchievement | O |  | T |  | T |  | F |  | T |  |

The FulfilmentInfo includes the following attributes.

Table 6.2.1.3.6.2-1

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| fulfilmentStatus | M | T | F | F | T |
| notFulfilledState | CM | T | F | F | T |
| notFulfilledReasons | CO | T | F | F | T |

6.2.1.3.6.3	Attribute constraints

Table 6.2.1.3.6.3-1

| Name | Definition |
| --- | --- |
| notFulfilledState Support Qualifier | Condition: when FulfilmentInfo is implemented for IntentFulfilmentInfo |
| notFulfilledReasons Support Qualifier | Condition: when FulfillmentInfo is implemented for IntentFulfilmentInfo |