3GPP TSG-SA5 Meeting #147 	S5-232589

Athens, Greece, 27th February - 3rd March 2023

# Source:	Ericsson

# Title:	Solving termination action issue

# Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

Include the proposed changes in TR 28.826.

# 2	References

[1]		3GPP TR 28.826: " Study on Nchf charging services phase 2 improvements and optimizations"

# 3	Rationale

Solving editor’s note on termination action.

# 4	Detailed proposal

First change

#### 5.1.5.4	Solution #1.4: Stop and discard the used units during re-authorization

A possible solution for key issue #1l covering requirements REQ-CH_INFO-07, control of granted quota during re-authorization.

The NF Service Consumer allows the service to continue whilst the re-authorization is progress, using the quota between the threshold and the granted quota. Either until the quota has been consumed or receiving a charging data response from CHF.

If the remaining part is used up before receiving the Charging Data Response:

-	NF Service Consumer stops the service delivery and discards the recorded used units.

If the Charging Data Response with new quota is received before the quota is consumed:

-	NF Service consumer continues the service delivery, deducting the used units from the new granted quota.

If the Charging Data Response with termination action, described in clause 5.4.3 of TS 32.290 [12], is received before the quota is consumed:

-	the NF Service consumer stops service delivery and discards the recorded used unit.

Second change

#### 5.1.5.5	Solution #1.5: Stop and report the used units during re-authorization

A possible solution for key issue #1l covering requirements REQ-CH_INFO-07, control of granted quota during re-authorization.

Clarify the description in clause 5.4.2 of TS 32.290 [12]. The NF Service Consumer allows the service to continue whilst the re-authorization is progress, until the remaining part had been used up.

The NF Service Consumer allows the service to continue whilst the re-authorization is progress, using the quota between the threshold and the granted quota. Either until the quota has been consumed or receiving the charging data response from CHF.

If the remaining part is used up before receiving a Charging Data Response:

-	NF Service Consumer stops the service delivery and reports the used unit in a Charging Data Request [Termination].

If the Charging Data Response with new quota is received before the quota is consumed:

-	NF Service consumer continues the service delivery, deducting the used units from the new granted quota.

If the Charging Data Response with termination action, described in clause 5.4.3 of TS 32.290 [12], is received before the quota is consumed:

-	the NF Service consumer stops service delivery and reports the used unit in a Charging Data Request [Termination].

Third change

#### 5.1.5.7	Solution #1.7: Continue and report the used units during re-authorization

A possible solution for key issue #1l covering requirements REQ-CH_INFO-07, control of granted quota during re-authorization.

The NF Service Consumer allows the service to continue whilst the re-authorization is progress, using the quota between the threshold and the granted quota. Either until the quota has been consumed or receiving a Charging Data Response from CHF.

If the remaining part is used up before receiving a Charging Data Response:

-	NF Service Consumer stops the service delivery and waits for the Charging Data Response. When the Charging Data Response is received it will follow the same actions as if it was received before the quota was consumed, see next two if-statements.

If the Charging Data Response with new quota is received before the quota is consumed:

-	NF Service Consumer continues the service delivery, deducting the used units from the new granted quota.

If the Charging Data Response with termination action, described in clause 5.4.3 of TS 32.290 [12], is received before the quota is consumed:

-	the NF Service Consumer stops service delivery and reports the recorded used unit in a Charging Data Request [Termination]

End of changes