3GPP TSG-SA5 Meeting #147 	S5-232098

Athens, Greece, 27th February - 3rd March 2023

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Add a new key issue for Advertising NRM properties by the MnS producer

# Document for:	Approval

Agenda Item:	6.8.2.3

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below

# 2	References

[1]	3GPP TS 28.831: " Management and orchestration; Study on basic Service-Based Management Architecture (SBMA) enabler enhancements"

# 3	Rationale

SA5 Provisioning MnS defines NRMs with IOCs with attributes along with attribute properties and Provisioning APIs for CRUD operations. Different MnS producers implementing the NRM might have different levels of support for the IOCs defined in SA5. A mechanism for the MnS consumer to be aware of the NRM properties of the MnS producer is necessary. This contribution proposes a new key issue to describe the issue with the current situation and identify the NRM properties that the MnS producer shall advertise to the MnS consumer.

This contribution proposes to add a new key issue to study potential solutions for the described problem, for the following objective as described in FS_eSBMAe SID.

- Investigate if new capabilities should be added to the Provisioning MnS, for example the concept of creating and removing attributes of managed object instances, or filter profiles.

In addition, the mechanism for the MnS producer to advertise the supported capabilities of the CRUD operations that are defined as part of the Provisioning MnS needs to be analyzed. However, this aspect is not covered by this proposed key issue.

Revision history:

- S5-225397 is a revision of S5-224280

- S5-226101 is a revision of S5-225397

- S5-232098 is a revision of S5-226101

# 4	Detailed proposal

The following changes are proposed for TR 28.831[1].

1st Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	XML Path Language (XPath), Version 1.0, W3C Recommendation 16 November 1999 (Status updated October 2016), (https://www.w3.org/TR/1999/REC-xpath-19991116/)

[3]	XML Path Language (XPath) 2.0, W3C Recommendation 14 December 2010 (Link errors corrected 3 January 2011; Status updated October 2016), (https://www.w3.org/TR/xpath20/)

[4]	XML Path Language (XPath) 3.1, W3C Recommendation 21 March 2017 (https://www.w3.org/TR/xpath-31/)

[5]	IETF RFC 6901: "JavaScript Object Notation (JSON) Pointer".

[6]	IETF Internet-Draft: "JSONPath: Query expressions for JSON"; draft-ietf-jsonpath-base-05; April 2022 (https://datatracker.ietf.org/doc/html/draft-ietf-jsonpath-base-05).

[7]	XML Information Set (Second Edition), W3C Recommendation 4 February 2004 (https://www.w3.org/TR/xml-infoset/)

[8]	XQuery and XPath Data Model 3.1, W3C Recommendation 21 March 2017 (https://www.w3.org/TR/xpath-datamodel-31/)

[9]	IETF RFC 7950: "The YANG 1.1 Data Modeling Language"

[10]	XForms 2.0 (https://www.w3.org/community/xformsusers/wiki/XForms_2.0)

[11]	3GPP TS 32.158: "Management and orchestration; Design rules for REpresentational State Transfer (REST) Solution Sets (SS)"

[12]	IETF RFC 787: "Problem Details for HTTP APIs."

[a]                       3GPP TS 28.537: “Management and orchestration; Management capabilities”.

[b]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[c]	3GPP TS 28.541: “Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3”.

[d]                        3GPP TS 28.533: “Management and orchestration; Architecture framework”.

[e]                        RFC8525: YANG Library

2nd Change

## 4.x	Key Issue #x: Advertising NRM properties by the MnS producer

### 4.x.1	Issue description

SA5 Provisioning MnS defines NRMs with IOCs with attributes along with attribute properties. Different MnS producers implementing the NRM might have different levels of support for the IOCs and attributes defined in SA5. This key issue is to study the current situation and propose solution to provide a mechanism for MnS producer to advertise its NRM properties to the MnS consumers.

The following aspects of the MnS producer NRM properties are analysed as part of this key issue:

-  A MnS producer might not support an IOC since the underlying functionality is not supported. For example, a MnS producer that does not have the capability to support performance metrics collection and reporting would not support PerfMetricJob IOC (defined in clause 4.3.31 of TS 28.622[b]). In another example a MnS producer supporting a non-split NG-RAN deployment will not support the IOCs relevant only for a 3-split NG-RAN deployment. The MnS producer supporting a non-split NG-RAN deployment would not support the E1 and F1 endpoint interfaces represented by IOCs EP_F1C, EP_F1U and EP_E1 (refer to clauses 4.3.1.1, 4.3.1.1 and 4.3.1.1 of TS 28.541 [c].

- For the IOCs defined without an upper boundary for the cardinality relationship (defined with a “*”), the MnS producer would not support an infinite number of instances of an IOC to be created. The MnS producer would have an upper limit defined on the multiplicity of the IOCs that it supports. For example, the maximum number of instances of object PerfMetricJob that can be created would be limited to a finite number.

- For the IOCs with name-containment relationship with ProxyClass representing different IOCs, the MnS producer may support all or only a subset of name-containment for the IOCs that can exist at different levels in the containment tree. A mechanism for the MnS consumer to be aware of which name-containment relationships are supported by the MnS producer is to be analysed. For example, the PerfMetricJob IOC can be name-contained by SubNetwork, ManagedElement, or ManagedFunction, but the MnS producer may support the name containment under SubNetwork only, and not support name-containment under ManagedElement, or ManagedFunction.

- For a MnS producer that supports vendor specific data using VsDataContainer (defined in clause 4.3.9 of TS 28.622[b]), a mechanism for the MnS consumer to retrieve the information on the vendor specific data (i.e., the information contained in the vsDataContainer) supported is to be analysed.

- A MnS producer might not support all attributes (i.e., the attributes with the support qualifier defined as optional or conditional mandatory or conditional optional) of an IOC.

- For the list attributes defined without an upper boundary for multiplicity (defined with a “*”), the MnS producer would not support an infinite number of elements for attribute. The MnS producer would have an upper limit defined on the multiplicity for the attribute that it supports.

- The MnS producer might support all or a subset or none of the defined notifications. The MnS producer might support the notification only for a subset of IOCs and attributes. This is applicable for alarm notifications and the configuration notifications (notifyMOICreation and notifyMOIDeletion notification that are applicable at IOCs and notifyMOIAttributeValueChanges, notifyMOIChanges notifications that are applicable at attribute level). A mechanism for the MnS consumer be aware of the supported notifications and the IOCs for which the notifications are supported is to be analysed.

This clause analyses the current situation and proposes a solution where the NRM properties related schema may be advertised by the MnS producer.

### 4.x.2	Current situation

Discovery of Management Services use cases and procedures are defined in clause 5 of TS 28.537[a]. It has been defined that MnS producers need to register themselves with their management capabilities in the 3GPP management system. The data describing a MnS producer, and their capabilities is called MnS information or MnS profile.

The related NRMs MnSRegistry and MnsInfo is defined in clause 4.3.42 and 4.3.41 of TS 28.622[b] respectively. MnSInfo hold the information related to the MnS label (attribute mnsLabel), type (attribute mnsType), version (attribute mnsVersion), address (attribute mnsAddress) and scope (attribute mnsScope).

The MnS producer profile is defined in clause 4.2.4 of TS 28.533[d]. This is described as a set of meta data that describes the MnS Producer. The profile holds information about the supported MnS components and their version numbers. This may also include information about support of optional features.

These existing mechanisms however do not define the mechanism for the MnS consumer to be aware of the MnS producer NRM properties described in clause 4.x.1, and hence a mechanism to advertise this needs to be studied and specified.

Further, MnS producer using YANG-Netconf solution set may use the YANG Library (ietf-yang-library defined in reference [e]) to advertise supported IOCs, attributes and constrains to the MnS consumers.

However, MnS producer based on OpenAPI solution set does not advertise the supported IOCs, attributes and constrains to the MnS consumers. The MnS consumers may expect that the complete NRM defined in SA5 is supported by the MnS producer. There is no mechanism currently defined where the MnS consumer can get this information. Hence, a mechanism to advertise such NRM properties in OpenAPI needs to be studied and specified.

### 4.x.3	Analysis

### 4.x.4	Potential requirements

### 4.x.5	Potential solution

### 4.x.6	CR proposal

### 4.x.7	Conclusion

End of change