3GPP TSG-SA5 Meeting #147 S5-232159 # Athens, Greece, 27th Feb 2023 – 3rd Mar 2023 CR-Form-v12.2. CHANGE REQUEST. 28.552; CR; 0398; rev; -; Current version:; 18.1.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests..

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New measurement “DL PDCP buffered throughput per UE per DRB”. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to A.33 of the 3GPP TS 28.552 to monitor DL PDCP buffered throughput per UE and bearer is essential, to ensure end user satisfaction and well-functioning and well configured cells. If an end user often experiences low quality during use of a service, the end-user might change wireless subscription provider, i.e. loss of income for the network operator. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New measurement “DL PDCP buffered throughput per UE per DRB” is proposed based on S5-226081 Discussion paper “Identification of the Bursts on PDCP SDU Level in CU UP” which was endorsed in 3GPP SA5 #146 meeting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Monitoring of the user satisfaction and well-functioning and well configured cells is not possible. The monitoring of the flow split in NSA option 3x and in DC is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

##### 5.1.1.3.X	Average DL UE buffered Throughput per DRB

a)	This measurement provides the average down link buffered UE throughput per DRB on NRCellCU.  The DRBs are mapped with the same 5QI for NR SA or mapped with the same QCI for EN-DC. This measurement is intended for throughput per UE and bearer independent of traffic patterns and packet size.  Initial buffering time in CU and on F1, meant as time interval the first PDCP SDU of the new burst is received in CU until this first part is received in DU, is excluded. The monitoring is supported also in DC scenario and in NSA option3a and 3x.

b)	DER(N=1)

c)	This measurement is obtained by the following formula for a measurement period:

ThroughputVolumeThroughputTime [kbits/s]

where each ThroughputVolume and ThroughputTime is intended to represent one DL burst as explained in the Fig. 5.1.1.3.X-1, Fig. 5.1.1.3.X-3, Fig. 5.1.1.3.X-5 and Table 5.1.1.3.X-2, Table 5.1.1.3.X-4, Table 5.1.1.3.X-6 for DRB (SA, NSA option 3a), split DRB (DC), split DRB (NSA option 3x), respectively. Separate counters are maintained for each mapped 5QI (or QCI for option 3).

Time

PDCP SDU 3

PDCP PDU 3

Volume

PDCP SDU 2

PDCP PDU 2

PDCP SDU 1

PDCP PDU 1

Time in DU buffer for PDCP PDU1

Time in DU buffer for the given PDCP

PDU is calculated as:

(PDCP PDU x Volume)/(

Achievable DRB

throughput)

T1'

T2'

T0'

Time in DU buffer for PDCP PDU2

Time in DU buffer for PDCP PDU3

PDCP SDU 4

PDCP PDU 4

PDCP SDU4 arrived to CU and there are not any other PDCP SDUs in CU UP waiting for transmission to DU nor the ones mapped to PDCP PDUs and sent to DU are in the DU buffer of the bearer thus it is start of new burst.

DU

CU UP

PDCP PDU2 is sent to DU when PDCP PDU1 still kept in the DU buffer therefore time period these two PDCP SDUs will spend in DU buffer is cumulated, i.e. obtained as sum of volume of the PDCP PDU1 and PDCP PDU2 divided with Achievable DRB throughput. The same happens for PDCP PDU 2 and 3.

.

.

Figure 5.1.1.3.X-1 Average DL buffered UE throughput per DRB (SA, NSA option 3a)

| T0’ | First PDCP SDU of the new burst arrived to CU and there are not any other PDCP SDUs in CU UP waiting for transmission to DU nor the ones mapped to PDCP PDUs and sent to DU are in the DU buffer. |
| --- | --- |
| T1’ | First PDCP PDU has been received in DU after T0’ (can be obtained as point in time when PDCP PDU sent from CU to DU plus F1 delay). |
| T2’ | The buffer in DU gets empty after T0’. |
| ThroughputTime | T2’ – T1’ [ms] Note: Contribution of the given PDCP SDUi to ThroughputTime, i.e. the time period the PDCP PDUi related to the PDCP SDUi will spend in the buffer of DU enitity, is obtained as volume of the PDCP PDUi divided with Achievable DRB throughput of the UE.In case in the point of time the PDCP PDUi is sent to DU while the previous one still kept in the DU buffer the time period these two PDCP SDUs will spend in DU buffer is cumulated, i.e. obtained as sum of volume of the PDCP PDUi and PDCP PDUi-1 divided with Achievable DRB throughput. The Achievable DRB throughput is obtained as the “Desired buffer size for data radio bearer” as part of last DDDS feedback [TS 38.425] divided with the DDDS reporting period time interval. In case the desired buffer size is 0 and an PDCP PDU sent to DU it is considered it will be spent the whole time interval in the buffer of DU until desired buffer size >0 is reported in the next DDDS feedback. Note; The precision of the measured ThroughputTime may be impacted with the precision of the measured F1 delay in case of not time synchronized CU and DU when it is obtained as F1 RTT/2. |
| ThroughptVolume | The PDCP SDU volume in bits successfully transmitted (acknowledged by DDDS) in DL to UE per bearer and one burst (consisting of PDCP SDU 1, 2 and 3 in example in Fig. 5.1.1.3.X-1). |

Table 5.1.1.3.X-2 DRB (SA, NSA option 3a)

Time

Volume

PDCP SDU 1

PDCP PDU 1

PDCP SDU 2

PDCP PDU 2

PDCP SDU 3

PDCP PDU 3

PDCP SDU 4

PDCP PDU 4

Time in DU buffer in the first

leg for PDCP PDU 1

Time in DU buffer in the second

leg for PDCP PDU

4

Time in DU buffer for the given PDCP

PDU x and leg is calculated as:

(PDCP PDU x Volume)/(

Achievable DRB

throughput)

T1

T5

T2

T7

T8

T0

Time

CU UP

DU1

DU2

PDCP SDU 5

PDCP PDU 5

PDCP SDU 3 and 5 arrived to CU and there are not any other PDCP SDUs in CU UP waiting for transmission to DU nor the ones mapped to PDCP PDUs and sent to DU are in the DU buffer of the bearer in any leg thus it is start of new burst.

Time in DU buffer in

the first leg for

PDCP PDU 2

Time in DU buffer in

the first leg for

PDCP PDU 3

T3

T4

T6

An example of two bursts consisting from PDCP SDU 1, 2 and 3,4.

Figure 5.1.1.3.X-3  Average DL buffered UE throughput per split DRB (DC)

| T0 | First PDCP SDU of the new burst arrived to CU and there are not any other PDCP SDUs in CU UP waiting for transmission to DU nor the ones mapped to PDCP PDUs and sent to DU are in the DU buffer for any of the legs relevant to the bearer. |
| --- | --- |
| T1 | First PDCP PDU has been received in DU1 of the first leg after T0 (can be obtained as point in time when PDCP PDU sent from CU to DU1 plus F1 delay). |
| T2 | First PDCP PDU has been received in DU2 of the second leg after T0 (can be obtained as point in time when PDCP PDU sent from CU to DU2 plus F1 delay). |
| T3 | The buffer in DU1 of the first leg gets empty after T0. |
| T4 | The buffer in DU2 of the second leg gets empty after T0 |
| ThroughputTime | T4 – T1 [ms] Note: Contribution of the given PDCP SDUi to ThroughputTime, i.e. the time period the PDCP PDUi related to the PDCP SDUi will spend in the buffer of DU related leg is obtained as volume of the PDCP PDUi divided with Achievable DRB throughput. In case in the point of time the PDCP PDUi is sent to DU while the previous one still kept in the DU buffer of the same leg the time period these two PDCP SDUs will spend in DU buffer is cumulated, i.e. obtained as sum of volume of the PDCP PDUi and PDCP PDUi-1 divided with Achievable DRB throughput. The Achievable DRB throughput is obtained as the “Desired buffer size for data radio bearer” as part of last DDDS feedback [TS 38.425] divided with the DDDS reporting period time interval. In case the desired buffer size is 0 and an PDCP PDU sent to DU it is considered it will be spent the whole time interval in the buffer of DU until desired buffer size >0 is reported in the next DDDS feedback. Note; The precision of the measured ThroughputTime may be impacted with the precision of the measured F1 delay in case of not time synchronized CU and DU when it is obtained as F1 RTT/2. |
| ThroughputVolume | The PDCP SDU volume in bits successfully transmitted (acknowledged by DDDS) in DL to UE per bearer and one burst (consisting of PDCP SDU 1, 2, 3 and 4 in example in Fig. 5.1.1.3.X-3). |

Table 5.1.1.3.X-4 Split DRB (DC)

Time

Volume

PDCP SDU 1

PDCP PDU 1

PDCP SDU 2

PDCP PDU 2

PDCP SDU 3

PDCP PDU 3

PDCP SDU 4

PDCP PDU 4

Time in DU buffer in the first

leg for PDCP PDU

1

Time in DU buffer in the first leg for

PDCP PDU

3

Time in MeNB buffer in the second leg for

PDCP PDU

2

Time in MeNB buffer in the second leg for PDCP PDU 4

Time in DU buffer for the given PDCP

PDU x and leg is calculated as:

(PDCP PDU x Volume)/(

Achievable DRB

throughput)

T1

T3

T2

T4

T5

T6

T0

Time

CU UP

DU

MeNB

PDCP SDU 5

PDCP PDU 5

PDCP SDU5 arrived to CU and there are not any other PDCP SDUs in CU UP waiting for transmission to DU/MeNB nor the ones mapped to PDCP PDUs and sent to DU/MeNB are in the DU/MeNB buffer of the bearer in any leg thus it is start of new burst.

Figure 5.1.1.3.X-5  Average DL buffered UE throughput per split DRB (NSA option 3x)

| T0 | First PDCP SDU of the new burst arrived to CU and there are not any other PDCP SDUs in CU UP waiting for transmission to DU/MeNB nor the ones mapped to PDCP PDUs and sent to DU/MeNB are in the DU/MeNB buffer for any of the legs relevant to the bearer. |
| --- | --- |
| T1 | First PDCP PDU has been received in DU of the first leg after T0 (can be obtained as point in time when PDCP PDU sent from CU to DU plus F1 delay). |
| T2 | First PDCP PDU has been received in MeNB of the second leg after T0 (can be obtained as point in time when PDCP PDU sent from CU to MeNB plus X2 delay). |
| T5 | The buffer in DU of the first leg gets empty after T0. |
| T6 | The buffer in MeNB of the second leg gets empty after T0 |
| ThroughputTime | T6 – T1 [ms] Note: Contribution of the given PDCP SDUi to ThroughputTime, i.e. the time period the PDCP PDUi related to the PDCP SDUi will spend in the buffer of DU related leg is obtained as volume of the PDCP PDUi divided with Achievable DRB throughput. In case in the point of time the PDCP PDUi is sent to DU while the previous one still kept in the DU buffer of the same leg the time period these two PDCP SDUs will spend in DU buffer is cumulated, i.e. obtained as sum of volume of the PDCP PDUi and PDCP PDUi-1 divided with Achievable DRB throughput. The Achievable DRB throughput is obtained as the “Desired buffer size for data radio bearer” as part of last DDDS feedback [TS 38.425] divided with the DDDS reporting period time interval. In case the desired buffer size is 0 and an PDCP PDU sent to DU it is considered it will be spent the whole time interval in the buffer of DU until desired buffer size >0 is reported in the next DDDS feedback. In case of split DRB NSA option 3x to calculate the Achievable DRB throughput for LTE leg the “Desired buffer size for E-RAB” as part of last DDDS feedback [TS 36.425] reported from MeNB to CU UP via X2 interface is considered. Note; The precision of the measured ThroughputTime may be impacted with the precision of the measured F1/X2 delay in case of not time synchronized CU and DU when it is obtained as F1/X2 RTT/2. |
| ThroughputVolume | The PDCP SDU volume in bits successfully transmitted (acknowledged by DDDS) in DL to UE per bearer and one burst (consisting of PDCP SDU 1, 2, 3 and 4 in example in Fig. 5.1.1.3.X-5). |

Table 5.1.1.3.X-6 Split DRB (NSA option 3x)

d)	Each measurement is a real value representing the throughput in kbit per second. The number of measurements is equal to one. If the optional QoS level subcounter is performed, the number of measurements is equal to the number of mapped 5Q and QCIs for option 3.

e)	The measurement name has the form 
DRB.PDCP.UEThpDl, or optionally DRB.PDCP.UEThpDl.QOS, where QOS identifies the target quality of service class.

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality).

End of Modified Section