3GPP TSG-SA5 Meeting #147 	S5-232292

Athens, Greece, 27th February - 3rd March 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0036 | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Improve definition of network slice traffic prediction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph2 |  |  |  |  |  | Date: |  |  | 2032-02-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Analytics output for network slice traffic prediction refers to network slice instance, but domain-specific (CN/RAN) MDAS producers are not aware of the network slice, instead they are aware of the network slice subnet instance. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that analytics output for network slice traffic prediction is valid for both network slice and network slice subnet. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Domain-specific MDA producers cannot provide the specified output. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.2.3.3, 8.5.10.1, 8.5.10.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

##### 8.4.2.3.3	Analytics output

The specific information elements of the analytics output for network slice traffic prediction analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.2.3.3-1.

Table 8.4.2.3.3-1: Analytics output for network slice traffic prediction analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| trafficProjections | This specifies the traffic projections for a network slice or a network slice subnet. | M | type: TrafficProjections multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |

2nd change

### 8.5.10	TrafficProjections <<dataType>>

#### 8.5.10.1	Definition

This data type specifies the traffic projection for a network slice or a network slice subnet.

#### 8.5.10.2	Information elements

Table 8.5.10.2-1

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| projectionTime | The time duration for which the projections are made. | M | type: ProjectionDuration multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| uPFProjections | This specifies the traffic projection of a UPF in the network slice or network slice subnet. It shall be present only if the analysis target contains CN part. | CM | type: UPFProj multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBProjections | This specifies the traffic projection of a gNB in the network slice or network slice subnet. It shall be present only if the analysis target contains AN part. | CM | type: gNBProj multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| sMFProjections | This specifies the projected number of PDU session of a SMF in the network slice or network slice subnet. It shall be present only if the analysis target contains CN part. | CM | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aMFProjections | This specifies the projected number of registered subscriber of an AMF in the network slice or network slice subnet. It shall be present only if the analysis target contains CN part. | CM | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of changes