3GPP TSG-SA5 Meeting #147	S5-232355

# 27 February - 03 March 2023, Athens, GR

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | - | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [Input to Draft CR] Add MDA capability for resource utilization analysis |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | The 3GPP system is a resource limited system, no matter whether the NF is working on virtualized resources or physical resources. Resource shortage would affect the QoS and potentially impact users’ quality of experience (QoE), e.g., by lowering the users’ data throughput, prolonging the users’ data delay, raising the rejections for the establishment of new connections (e.g., RRC connection), sessions (e.g., PDU session) and resources (e.g., QoS flows, DRBs, etc.) and increasing the drops of the existing connections, sessions, and resources. Resource excess would cause wastage that leads to additional CapEx and OpEx. Therefore, it is imperative to ensure optimum and efficient resource utilization for the NFs. The resource utilization of an NF is heavily dependent on traffic patterns, which could vary in different areas (e.g., business area, entertainment area, and residential area) in different time periods. It is desirable that the spare resource of the low-usage areas can be allocated to the busy areas. It is expected that MDA can perform an analysis of the resource utilization for NFs to indicate the resource usage patterns in the past and predict the resource usage trend for some time periods in the future. It is also very useful that MDA provides recommendations to orchestrate the resources among NFs between the low usage and high usage areas for some time periods. The recommended actions could be for example to schedule the "scale in" and "scale out" of VNFs to optimize the allocation of the virtualized resources. The resource utilization analysis is also studied in TR 28.809. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added UC, requirements, and solutions for resource utilization analysis. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Resource utilization analysis cannot be supported by MDA. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.x (new), 8.4.x (new), 8.5.x (new), 8.5.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

### 7.2.x	Resource related analytics

#### 7.2.x.1	NF resource utilization analysis

##### 7.2.x.1.1	Description

This MDA capability is for analysis of resource utilization of 3GPP NFs.

##### 7.2.x.1.2	Use case

The 3GPP system is a resource limited system, no matter whether the NF is working on virtualized resources or physical resources.

Resource shortage would affect the QoS and potentially impact users’ quality of experience (QoE), e.g., by lowering the users’ data throughput, prolonging the users’ data delay, raising the rejections for the establishment of new connections (e.g., RRC connection), sessions (e.g., PDU session) and resources (e.g., QoS flows, DRBs, etc.) and increasing the drops of the existing connections, sessions, and resources.

Resource excess would cause wastage that leads to additional CapEx and OpEx.

Therefore, it is imperative to ensure optimum and efficient resource utilization for the NFs.

The resource utilization of an NF is heavily dependent on traffic patterns, which could vary in different coverage areas (e.g., business area, entertainment area, and residential area) and in different time periods (weekdays and time of the day). It is desirable that the spare resource of the low-usage areas can be allocated to the busy areas.

It is expected that MDA can perform an analysis of the resource utilization for NFs to indicate the resource usage patterns in the past and predict the resource usage trend for some time periods in the future.

It is also very useful that MDA provides recommendations that can be utlised to efficiently orchestrate the resources among NFs between the low usage and high usage areas for some time periods. The recommended actions could be for example to schedule the "scale in" and "scale out" of VNFs to optimize the allocation of the virtualized resources.

##### 7.2.x.1.3	Requirements

Table 7.2.1.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-RES_UTI_ANA-01 | MDA capability for resource utilization analysis shall be able to identify the NFs with low or under-utilised resource usage in a specific coverage area or a sub-network during some time periods in the past. | NF resource utilization analysis |
| REQ-RES_UTI_ANA-02 | MDA capability for resource utilization analysis shall be able to identify the NFs with high resource usage in a specific coverage area or a sub-network during some time periods in the past. | NF resource utilization analysis |
| REQ-RES_UTI_ANA-03 | MDA capability for resource utilization analysis shall be able to provide the prediction of resource usage of NFs in a specific area or a sub-network for some time periods in the future. | NF resource utilization analysis |
| REQ-RES_UTI_ANA-04 | MDA capability for resource utilization analysis shall be able to provide recommended actions to orchestrate the resource allocation for NFs. | NF resource utilization analysis |

Next modified section

### 8.4.x	Resource related analytics

#### 8.4.x.1	NF resource utilization analysis

##### 8.4.x.1.1	MDA type

The MDA type for NF resource utilization analysis is: ResourceAnalytics.ResourceUtilizationAnalysisNF.

##### 8.4.x.1.2	Enabling data

The enabling data for ResourceAnalytics.ResourceUtilizationAnalysisNF MDA type are provided in table 8.4.x.1.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.x.1.2-1: Enabling data for NF resource utilization analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | VR (including Virtual CPU, Virtual Memory, and Virtual Disk) usage of NF | VR usage of NF (clause 5.7.1 of TS 28.552 [4]) |
|  | Connection Point data volumes of NF | Connection data volumes of NF (clause 5.7.2 of TS 28.552 [4]) |
|  | N3 interface data volume | N3 interface related measurements (clause 5.4.1 of TS 28.552 [4]) |
|  | N4 interface session establishments | N4 session establishments (clause 5.4.3.1 of TS 28.552 [4]) |
|  | N6 interface link usage | N6 related measurements (clause 5.4.2 of TS 28.552 [4]) |
|  | N9 interface data volume | GTP Data Packets and volume on N9 interface (clause 5.4.4.2 of TS 28.552 [4]) |
|  | Radio resource utilization | Radio resource utilization (clause 5.1.1.2 of TS 28.552 [4]) |
|  | RRC connection number | RRC connection number (clause 5.1.1.4 of TS 28.552 [4]) |
|  | Mean number of PDU sessions in NR cell | Mean number of PDU sessions being allocated (clause 5.1.1.5.4 of TS 28.552 [4]) |
|  | Mean number of DRBs in NR cell | Mean number of DRBs being allocated ( clause 5.1.1.10.9 of TS 28.552 [4]) |
|  | QoS flow release in NR cell | QoS flow release (clause 5.1.1.13.1 of TS 28.552 [4]) |
|  | Number of Active UEs | Number of Active UEs (clause 5.1.1.23 of TS 28.552 [4]) |
|  | PDCP Data Volume | PDCP Data Volume (clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [4]) |
|  | Number of PDU sessions | Number of PDU sessions (mean) (clause 5.3.1.1 of TS 28.552 [4]) |
|  | Number of QoS flows | Mean number of QoS flows (clause 5.3.2.1.7 of TS 28.552 [4]) |
| Geographical data | The geographical information (longitude, latitude, altitude) of the deployed RAN (NG-RAN and E-UTRAN). | The geographical information (longitude, latitude, altitude) information (see the peeParametersList attribute of the ManagedFunction IOC in TS 28.622 [19]). |
| Configuration data | The NRMs of the analyzed NFs | The NRMs defined in TS 28.622 [19] and TS 28.541 [15]. |
| Network Data Analytics | Analysis data from the control plane produced by NWDAF including NF load, observed service experience, user plane performance, and slice load level analytics. | Analytics data from NWDAF in TS 23.288 [10] including e.g. NF load analytics (clause 6.5), observed service experience related network data analytics (clause 6.4), analytics for user plane performance (i.e. average/maximum traffic rate, average/maximum packet delay, average packet loss rate in clause 6.14), and Slice load level related network data analytics (clause 6.3). |

##### 8.4.x.1.3	Analytics output

The specific information elements of the analytics output for NF resource utilization analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.x.1.3-1.

Table 8.4.x.1.3-1: Analytics output for NF resource utilization analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| lowResourceUsageNFs | The NFs with low resource usage (see Note 1) during some time periods in the past. | M | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| highResourceUsageNFs | The NFs with high resource usage (see Note 1) during some time periods in the past. |  | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| predictedResourceUsageForNFs | The predicted resource usage for NFs during some time periods in the future. | M | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| recommendedActions | The recommended actions to orchestrate the resource allocation for NFs. The recommended action may be (but not limited to): - scale in a list of NFs; - scale out a list of NFs. | M | type: RecommendedAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NOTE 1: it is up to the MDA MnS producer to decide the thresholds for low and high usage. |  |  |  |

Next modified section

### 8.5.x	ResourceUsageNF <<dataType>>

#### 8.5.x.1	Definition

This data type specifies the type of resource usage for an NF.

#### 8.5.x.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| nFId | It provides the DN of the NF (which can be a 5GC or an NG-RAN NF). | M | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| startTime | It indicates the start time of the time period. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| endTime | It indicates the end time of the time period. | M | type: DateTime multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| overallResourceUsage | It provides the overall average usage (in percentage) of all of the resources. | M | type: Real multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| specificResourceUsage | It provides the average usage of the specific type(s) of resources. | M | type: ResourceUsage multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

#### 8.5.x.3	Constraints

None.

Next modified section

### 8.5.y	ResourceUsage <<dataType>>

#### 8.5.y.1	Definition

This data type specifies the type of resource usage.

#### 8.5.y.2	Information elements

| Name | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| resourceType | It indicates the type of resource. The allowed value is “VirtualCpu”, “VirtualMemory”, “VirtualDisk”, “DLPRBTotal”, “ULPRBTotal”, or a vendor-specific value. | M | type: string multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| meanUsage | It provides the mean usage or predicted mean usage (in percentage) of the resource (indicated by the “resourceType” information element). | M | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

#### 8.5.y.3	Constraints

None.

End of modified sections