3GPP TSG-SA5 Meeting #147	S5-232442

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0171 | rev | - | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 28.531 OpenAPI SS for 28.531 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung R&D Institute UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNETSLICE_PRO |  |  |  |  |  | Date: |  |  |  |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | OpenAPI solution set is missing in 28.531 Claue 9 have some error |  |  |  |  |  |  |  |  |
| Summary of change: |  | OpenAPI solution set for 28.531 Fixing errors in clause 9 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete Specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | 9, |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

# 9	Void

Next Modified Section

# x	RESTful HTTP-based solution set of provisioning

## x.1	Network slice provisioning management service

### x.1.1	Mapping of operations

#### x.1.1.1	Introduction

Table x.1.1-1: Mapping of IS operations to SS equivalents

| IS operation | HTTP Method | Resource URI | Qualifier |
| --- | --- | --- | --- |
| allocateNsi | POST | {MnSRoot}/NSProvMnS/{MnSVersion}/ServiceProfile | M |
| deallocateNsi | DELETE | {MnSRoot}/NSProvMnS/{MnSVersion}/ServiceProfile={ServiceProfileId} | M |

#### x.1.1.2	Operation allocateNsi

This operation is to allocate a network slice instance provided by the service provider, the network slice instance may be new or existing.

Table x.1.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| attributeListIn | request body | attributeListIn | AttributeNameValuePairSet | M |

Table x.1.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| attributeListOut | response body | attributeListOut | AttributeNameValuePairSet | M |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponse | O |
| networkSliceDN | response body | networkSliceDN | string | M |

The message flow for allocation is as follows:

1.	The MnS consumer sends a HTTP POST request to the MnS producer.

- The target URI is equal to the concatenation of URI of the parent resource of resource to be created, and the resource (in this case ServiceProfile) to be created.

- The message body shall carry the complete representation of the resource to be created. The resource identifier shall be absent or carry null semantics.

2.	The MnS producer sends a HTTP POST response to the MnS consumer.

- On success, "201 Created" shall be returned. The Location header shall carry the URI of the new resource (in this case ServiceProfile) and the message body shall contain the complete complete representation of the ServiceProfile and networkSliceDN identifying the NetworkSlice MOI created.

- On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

#### x.1.1.3	Operation deallocateNsi

This operation deallocate a service profile in an NSI. The provider may terminate the requested NSI or modify the requested NSI without termination to satisfy the request.

Table x.1.1.3-1: Mapping of IS operation input parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| networkSliceDN | Request body | networkSliceDN | string | M |
| serviceProfileId | Request body | serviceProfileId | string | M |

Table x.1.1.3-2: Mapping of IS operation output parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponse | O |

The message flow for deallocation is as follows:

1.	The MnS consumer sends a HTTP DELETE request to the MnS producer.

- The target URI is equal to the concatenation of URI of the parent resource and the resource (in this case ServiceProfile) to be deleted.

- The message body shall contain the networkSliceDN identifying the NetworkSlice MOI.

2.	The MnS producer sends a HTTP DELETE response to the MnS consumer.

-  On success, "204 No content" shall be returned.

- On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

### x.1.2	Resources

#### x.1.2.1	Resource definitions

##### x.1.2.1.1	Resource “…/ServiceProfile

x.1.2.1.1.1	Description

This resource represents collects of network slice related requirement (i.e. ServiceProfiles).

x.1.2.1.1.2	URI

Resource URI: {MnSRoot}/NSProvMnS/{MnSVersion}/ServiceProfile

The resource URI variables are defined in the following table.

Table x.1.2.1.1.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.2 of TS 32.158 [15] |
| NSProvMnS | Network Slice Provisionig management service |
| MnSVersion | See clause 4.4.2 of TS 32.158 [15] |
| ServiceProfile | Name of the resource to be created |

x.1.2.1.1.3	HTTP methods

x.1.2.1.1.3.1	POST

The POST method create a serviceProfile, the provider may create a NSI or using existing NSI to satisfy the serviceProfile.

This method shall support the URI query parameters specified in the following table.

Table x.1.2.1.1.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures, and the response data structures and response codes specified in the following tables.

Table x.2.1.1.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| AttributeNameValuePairSet | List of attributes name value pair for the target resource. This parameter specifies the network slice related requirements or network related requirements defined in ServiceProfile in Clause 6.3.3 in TS 28.541 [6]. | M |

Table x.2.1.1.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| SliceAllocationInfoType | 20x | In case of success the representation of the created NetworkSlice MOI is returned. | M |
| ErrorResponse | 4xx/5xx | Returned in case of an error | O |

##### x.1.2.1.2	Resource “…/ServiceProfile={ServiceProfileId}

x.1.2.1.2.1	Description

This resource represents instance of ServiceProfile resource.

x.1.2.1.2.2	URI

Resource URI: {MnSRoot}/NSProvMnS/{MnSVersion}/ServiceProfile={ServiceProfileID}

The resource URI variables are defined in the following table.

Table x.1.2.1.1.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.2 of TS 32.158 [15] |
| NSProvMnS | Network Slice Provisionig management service |
| MnSVersion | See clause 4.4.2 of TS 32.158 [15] |
| ServiceProfile | Name of the resource to be created |
| ServiceProfileId | Identifier of the targeted resource |

x.1.2.1.2.3	HTTP methods

x.1.2.1.2.3.1	DELETE

The DELETE method deletes a ServiceProfile.

This method shall support the request data structures, and the response data structures and response codes specified in the following tables.

Table x.2.2.2.2.3.1-1: Data structures supported by the DELETE Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table x.2.2.2.2.3.1-2: Data structures supported by the DELETE Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| ErrorResponse | 4xx/5xx | Returned in case of an error | O |

### x.1.3	Data Type Definitions

#### x.1.3.1      General

This clause defines the data types used by the Network Slice Provisioning MnS. Table 9.1.3.1-1 specifies the data types defined in the present document and Table table 9.1.3.1-2 the data types imported

Table x.1.3.1-1: Data types defined in this specification

| Data type | Reference | Description |
| --- | --- | --- |
| SliceAllocationInfoType | 9.1.3.3 | Provides information related to slice allocation |

Table x.1.3.1-2: Data types imported

| Data type | Reference | Description |
| --- | --- | --- |
| AttributeNameValuePairSet | TS 28.623 [x] | Set of attribute name/value pairs |
| ErrorResponse | TS 28.623 [x] | Used in the response body of multiple HTTP methods in case of error |

x.1.3.3	Type SliceAllocationInfoType

Table x.1.3.2-1: SliceAllocationInfoType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| attributeListOut | AttributeNameValuePairSet | This list of name/value pairs contains the attributes of the ServiceProfile[6] which has been allocated and the actual value assigned to each. | M |
| networkSliceDN | string | The DN of NetworkSlice MOI uniquely identifying the network slice instance. | M |

## x.2	Network slice subnet provisioning management service

### x.2.1	Mapping of operations

#### x.2.1.1	Introduction

Table x.2.1.1-1: Mapping of IS operations to SS equivalents

| IS operation | HTTP Method | Resource URI | Qualifier |
| --- | --- | --- | --- |
| allocateNssi | POST | {MnSRoot}/NSSProvMnS/{MnSVersion}/SliceSubnetMgmt/SliceProfile | M |
| deallocateNssi | DELETE | {MnSRoot}/NSSProvMnS/{MnSVersion}/SliceSubnetMgmt/SliceProfile={SliceProfileid} | M |

#### x.2.1.2	Operation allocateNssi

This operation is to allocate a network slice instance provided by the service provider, the network slice subnet instance may be new or existing.

Table x.2.1.2-1: Mapping of IS operation input parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| attributeListIn | request body | attributeListIn | AttributeNameValuePairSet | M |

Table x.2.1.2-2: Mapping of IS operation output parameters to SS equivalents (HTTP POST)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| attributeListOut | response body | attributeListOut | AttributeNameValuePairSet | M |
| status | response status codes | n/a | n/a | M |
|  | Response body | error | ErrorResponse | O |
| networkSliceSubnetDN | response body | networkSliceSubnetDN | string | M |

The message flow for allocation is as follows:

1.	The MnS consumer sends a HTTP POST request to the MnS producer.

- The target URI is equal to the concatenation of URI of the parent resource of resource to be created, and the resource (in this case SliceProfile) to be created.

- The message body shall carry the complete representation of the resource to be created. The resource identifier shall be absent or carry null semantics.

2.	The MnS producer sends a HTTP POST response to the MnS consumer.

- On success, "201 Created" shall be returned. The Location header shall carry the URI of the new resource (in this case SliceProfile) and the message body shall contain the complete complete representation of the SliceProfile and networkSliceSubnetDN identifying the NetworkSliceSubnet MOI created.

- On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

#### x.2.1.3	Operation deallocateNssi

This operation deallocate a slice profile in an NSSI. The provider may terminate the requested NSSI or modify the requested NSSI without termination to satisfy the request.

Table x.2.1.3-1: Mapping of IS operation input parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| networkSliceSubnetDN | request body | networkSliceSubnetDN | string | M |
| sliceProfileId | request body | sliceProfileId | string | M |

Table x.2.1.3-2: Mapping of IS operation output parameters to SS equivalents (HTTP DELETE)

| IS operation parameter name | SS parameter location | SS parameter name | SS parameter type | Qualifier |
| --- | --- | --- | --- | --- |
| status | response status codes | n/a | n/a | M |
|  | response body | error | ErrorResponse | O |

The message flow for deallocation is as follows:

1.	The MnS consumer sends a HTTP DELETE request to the MnS producer.

- The target URI is equal to the concatenation of URI of the parent resource and the resource (in this case SliceProfile) to be deleted.

- The message body shall contain the networkSliceSubnetDN identifying the NetworkSliceSubnet MOI.

2.	The MnS producer sends a HTTP DELETE response to the MnS consumer.

- On success, "204 No content" shall be returned.

- On failure, an appropriate error code shall be returned. The response message body may provide additional error information.

### x.2.2	Resources

#### x.2.2.1	Resource definitions

##### x.2.2.1.1	Resource “.../SliceProfile

x.2.2.1.1.1	Description

This resource represents collects of network slice subnet related requirements (i.e. SliceProfile).

x.2.2.1.1.2	URI

Resource URI: {MnSRoot}/NSSProvMnS/{MnSVersion}/SliceProfile

The resource URI variables are defined in the following table.

Table x.2.2.1.1.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.2 of TS 32.158 [15] |
| NSSProvMnS | Network Slice Subnet Provisionig management service |
| MnSVersion | See clause 4.4.2 of TS 32.158 [15] |
| SliceProfile | Name of the resource to be created |

x.2.2.1.1.3	HTTP methods

x.2.2.1.1.3.1	POST

The POST method create a SliceProfile, the provider may create a new NSSI or using existing NSSI to support the SliceProfile.

This method shall support the URI query parameters specified in the following table.

Table x.2.2.1.1.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | S |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures, and the response data structures and response codes specified in the following tables.

Table x.2.1.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| AttributeNameValuePairSet | List of attributes name value pair for the target resource. This parameter specifies the network slice subnet related requirements defined in SliceProfile in Clause 6.3.4 in TS 28.541 [6]. | M |

Table x.2.1.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| SliceSubnetAllocationInfoType | 20x | In case of success the representation of the created NetworkSliceSubnet MOI is returned. | M |
| ErrorResponse | 4xx/5xx | Returned in case of an error | O |

##### x.1.2.1.2	Resource “…/SliceProfile={SliceProfileId}

x.1.2.1.2.1	Description

This resource represents instance of SliceProfile resource.

x.1.2.1.2.2	URI

Resource URI: {MnSRoot}/NSSProvMnS/{MnSVersion}/SliceProfile={SliceProfileID}

The resource URI variables are defined in the following table.

Table x.1.2.1.1.2-1: URI variables

| Name | Definition |
| --- | --- |
| MnSRoot | See clause 4.4.2 of TS 32.158 [15] |
| NSProvMnS | Network Slice Provisionig management service |
| MnSVersion | See clause 4.4.2 of TS 32.158 [15] |
| SliceProfile | Name of the resource to be created |
| SliceProfileId | Identifier of the targeted resource |

x.1.2.1.2.3	HTTP methods

x.2.2.1.1.3.2	DELETE

The DELETE method deletes a SliceProfile.

This method shall support the request data structures, and the response data structures and response codes specified in the following tables.

Table x.2.2.2.2.3.1-1: Data structures supported by the DELETE Request Body on this resource

| Data type | Description | S |
| --- | --- | --- |
| n/a | n/a | n/a |

Table x.2.2.2.2.3.1-2: Data structures supported by the DELETE Response Body on this resource

| Data type | Response codes | Description | S |
| --- | --- | --- | --- |
| ErrorResponse | 4xx/5xx | Returned in case of an error | O |

### x.1.3	Data Type Definitions

#### x.1.3.1      General

This clause defines the data types used by the Network Slice Subnet Provisioning MnS. Table x.1.3.1-1 specifies the data types defined in the present document and Table table x.1.3.1-2 the data types imported

Table x.1.3.1-1: Data types defined in this specification

| Data type | Reference | Description |
| --- | --- | --- |
| SliceSubnetAllocationInfoType | x.1.3.3 | Provides information related to slice subnet allocation |

Table x.1.3.1-2: Data types imported

| Data type | Reference | Description |
| --- | --- | --- |
| AttributeNameValuePairSet | TS 28.623 [x] | Set of attribute name/value pairs |
| ErrorResponse | TS 28.623 [x] | Used in the response body of multiple HTTP methods in case of error |

x.1.3.3	Type SliceSubnetAllocationInfoType

Table x.1.3.2-1: SliceAllocationInfoType

| Attribute name | Data type | Description | SQ |
| --- | --- | --- | --- |
| attributeListOut | AttributeNameValuePairSet | This list of name/value pairs contains the attributes of the SliceProfile[6] which has been allocated and the actual value assigned to each. | M |
| networkSliceSubnetDN | string | The DN of NetworkSliceSubnet MOI uniquely identifying the network slice instance. | M |

2nd Modified Section

Annex A (normative):
OpenAPI specification

# A.0	Introduction

This clause describes the capabilities of the service in the structure of the OpenAPI Specification Version 3.0.1 [A9]. The OpenAPI definitions are provided in YAML or JSON format.

# A.1	Network Slice Provisioning management service

## A.1.1	OpenAPI document "TS28531_NSProvMnS.yaml"

openapi: 3.0.1

info:

title: Network Slice Provisioning MnS

version: 17.1.0

description: >-

OAS 3.0.1 definition of the Provisioning MnS

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.531; Network Slice management services

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.531/

servers:

- url: '{MnSRoot}/NSProvMnS/{MnSVersion}/ServiceProfile'

variables:

MnSRoot:

description: See clause 4.4.2 of TS 32.158

default: http://example.com/3GPPManagement

MnSVersion:

description: Version number of the OpenAPI definition

default: XXX

paths:

'/ServiceProfile={ServiceProfileId}':

delete:

summary: Deletes one or multiple resources

description: >-

With HTTP DELETE resources are deleted. The resources to be deleted are

identified with the target URI.

parameters:

- name: ServiceProfileId

in: path

required: true

schema:

$ref: '#/components/schemas/ServiceProfileId'

responses:

'200':

description: >-

Success case ("200 OK").

This status code shall be returned, when query parameters are present in

the request and one or multiple resources are deleted.

The URIs of the deleted resources are returned in the response message body

'204':

description: >-

Success case ("204 No Content").

This status code shall be returned, when no query parameters are present in

the request and only one resource is deleted.

The message body is empty.

content:

application/json:

schema:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

'/ServiceProfile':

post:

summary: Creates a complete single resource

description: >-

With HTTP POST a complete single resource is created. The identifier of the new

resource is generated by the MnS producer.

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

responses:

'201':

description: >-

Success case ("201 Created").

This status code shall be returned when the resource is created.

The representation of the created resource is returned in the response

message body.

content:

application/json:

schema:

$ref: '#/components/schemas/SliceAllocationInfoType'

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

components:

schemas:

SliceAllocationInfoType:

type: object

properties:

attributeListOut:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

networkSliceDN:

type: string

ServiceProfileId:

type: string

# A.2	Network Slice Subnet Provisioning management service

## A.1.1	OpenAPI document "TS28531_NSSProvMnS.yaml"

openapi: 3.0.1

info:

title: Network Slice Subnet Provisioning MnS

version: 17.1.0

description: >-

OAS 3.0.1 definition of the Provisioning MnS

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 28.531; Network Slice Subnet management services

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.531/

servers:

- url: '{MnSRoot}/NSSProvMnS/{MnSVersion}/ServiceProfile'

variables:

MnSRoot:

description: See clause 4.4.2 of TS 32.158

default: http://example.com/3GPPManagement

MnSVersion:

description: Version number of the OpenAPI definition

default: XXX

paths:

'/SliceProfile={SliceProfileId}':

delete:

summary: Deletes one or multiple resources

description: >-

With HTTP DELETE resources are deleted. The resources to be deleted are

identified with the target URI.

parameters:

- name: SliceProfileId

in: path

required: true

schema:

$ref: '#/components/schemas/SliceProfileId'

responses:

'200':

description: >-

Success case ("200 OK").

This status code shall be returned, when query parameters are present in

the request and one or multiple resources are deleted.

The URIs of the deleted resources are returned in the response message body

'204':

description: >-

Success case ("204 No Content").

This status code shall be returned, when no query parameters are present in

the request and only one resource is deleted.

The message body is empty.

content:

application/json:

schema:

type: array

items:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Uri'

default:

description: Error case.

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/ErrorResponse'

'/SliceProfile':

post:

summary: Creates a complete single resource

description: >-

With HTTP POST a complete single resource is created. The identifier of the new

resource is generated by the MnS producer.

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

responses:

'201':

description: >-

Success case ("201 Created").

This status code shall be returned when the resource is created.

The representation of the created resource is returned in the response

message body.

content:

application/json:

schema:

$ref: '#/components/schemas/SliceSubnetAllocationInfoType'

default:

description: Error case.

content:

application/json:

schema:

$ref: 'comDefs.yaml#/components/schemas/ErrorResponse'

components:

schemas:

SliceSubnetAllocationInfoType:

type: object

properties:

attributeListOut:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/AttributeNameValuePairSet'

networkSliceSubnetDN:

type: string

SliceProfileId:

type: string